3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210049

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1491 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC idle mode test case 6.3.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the context of test case 6.3.1.7, the function f_NR5GC_SorRegistration () is called with parameter p_RegType passed as (Initial_Secure, Emergency). Since this is a template which does not reduce to a single value, a run-timer error occurs when the valueof operation is applied when calling f_NR5GC_RRC_Idle_Steps5_13() |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove use of valueof in f_NR5GC_SorRegistration() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not execute correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_SorRegistration |
| --- | --- |
| Reason for change | In the context of test case 6.3.1.7, the parameter p_RegType is passed as (Initial_Secure, Emergency). Since this is a template which does not reduce to a single value, a run-timer error occurs when the valueof operation is applied when calling f_NR5GC_RRC_Idle_Steps5_13() |
| Summary of change | Remove use of valueof operation |
| TTCN module | NR5GC\6_3_1\SOR_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle, but implemented differently. |

Before Change:

function f_NR5GC_SorRegistration(NR_CellId_Type p_NR_CellId, octetstring p_Data, template (value) SOR_Header p_Header, boolean p_MacError:= false, template (present) TypeOfRegistration_Type p_RegType := Initial_NoSecurity, template (present) SecurityHeaderType p_ExpectedSecurityStatus := tsc_SHT_NoSecurityProtection) runs on NR5GC_PTC return O16_Type//@sic R5s201455 sic@ { ... ... f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (p_NR_CellId)), TESTMode_OFF, valueof(p_RegType));//@sic R5s201455 sic@ ... ... }

After Change:

function f_NR5GC_SorRegistration(NR_CellId_Type p_NR_CellId, octetstring p_Data, template (value) SOR_Header p_Header, boolean p_MacError:= false, template (present) TypeOfRegistration_Type p_RegType := Initial_NoSecurity, template (present) SecurityHeaderType p_ExpectedSecurityStatus := tsc_SHT_NoSecurityProtection) runs on NR5GC_PTC return O16_Type//@sic R5s201455 sic@ { ... ... f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (p_NR_CellId)), TESTMode_OFF, p_RegType);//@sic R5s201455 sic@ ... ... }

MCC160 implementation

function f_NR5GC_SorRegistration(NR_CellId_Type p_NR_CellId, octetstring p_Data, template (value) SOR_Header p_Header, boolean p_MacError:= false, template (present) TypeOfRegistration_Type p_RegType := Initial_NoSecurity, template (present) SecurityHeaderType p_ExpectedSecurityStatus := tsc_SHT_NoSecurityProtection) runs on NR5GC_PTC return O16_Type//@sic R5s201455 sic@ { var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var O16_Type v_Mac_AMF; var O16_Type v_Mac_UE; var B128_Type v_Mac_A17; var B128_Type v_Mac_A18; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var O2_Type v_Counter :='0001'O; var TypeOfRegistration_Type v_TypeOfRegistration; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; //Steps 2 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13 v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?, p_ExpectedSecurityStatus);//@sic R5s201455 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, p_RegType)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } select (v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.registrationType) { case (tsc_NG_RegistrationEmergency) { v_TypeOfRegistration := Emergency; } case (tsc_NG_RegistrationInitial) { v_TypeOfRegistration := Initial_NoSecurity; } case else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration type incorrect"); } } f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (p_NR_CellId)), TESTMode_OFF, v_TypeOfRegistration); //@sic R5s201455 R5s210049 sic@ //calculate v_Mac_AMF, which is used in REGISTRATION ACCEPT message v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(p_Header)), v_NAS_SecurityParams.KAUSF, v_Counter, p_Data, v_NAS_SecurityParams.KDF); v_Mac_AMF := bit2oct(v_Mac_A17); if(p_MacError) { v_Mac_AMF := not4b(v_Mac_AMF);//unmatched SOR-MAC-IAUSF }; //calculate v_Mac_UE, which is used in REGISTRATION COMPLETE message after successful check v_Mac_A18 :=f_NG_Authentication_A18(v_NAS_SecurityParams.KAUSF, v_Counter, v_NAS_SecurityParams.KDF); v_Mac_UE :=bit2oct(v_Mac_A18); //The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing expected steering of roaming information v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ f_NR5GC_GetMobileIdGUTI (p_NR_CellId), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (p_NR_CellId)), {bit2oct(f_NR_CellInfo_GetTAC(p_NR_CellId))}), -,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-, cs_NG_SORTransparentContainer_IeiIel(p_Header, v_Mac_AMF, v_Counter, p_Data)); // Registration Accept SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, // must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); return v_Mac_UE; }