3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210079

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1513 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC CA test cases 8.1.4.1.7.1 and 8.1.4.1.7.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The definition of v_CellPowerList_AtT0 in not in line with 38.523-1 Table 8.1.4.1.7.1.3.2-1. - Cell3 (source SCell) should have the same C_RNTI as cell1 (source PCell) - In the step 4, PCell is changed via reconfigurationWithSync and a new SCell is added. However, the previously configured SCell (which becomes the new PCell) is not released. As per 38.331, SCells shall be explicitly released. - The variable v_SCGConfig_SCell is not initialsied when there is no value for p_TargetSCellId. This results in uninitialized value passed into f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common() causing a run-time error in this function. - The current implementation of f_NR_TimingOtherCell() actually can support timing calculation for cells with different numerology. But the returned timing is using the numerologiy of target cell not source cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update setting of v_CellPowerList_AtT0 to make cell 23 (target PCell) off and cell 2 (targetSCell) non-suitable - Change tsc_C_RNTI_Value3 to tsc_C_RNTI_Value1. - Add v_SCellToReleaseList with tsc_SCellIndex_1 in rrcReconfiguration. Note : An associated prose CR on 38.523-1 will be raised at RAN#90 - Remove passing v_SCGConfig_SCell to ‘f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(…) - Removed the error code, and use v_SlotTimingInfo based on target cell to calculate v_SubFrameTiming. Removing call to f_CellTimingInfo_TimingOtherCell() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.7.1.NR5GC, 8.1.4.1.7.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_4_1_7_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | The definition of v_CellPowerList_AtT0 in not in line with 38.523-1 Table 8.1.4.1.7.1.3.2-1. |
| Summary of change | Update setting of v_CellPowerList_AtT0 to make cell 23 (target PCell) off and cell 2 (targetSCell) non-suitable |
| TTCN module | RRC_NR_CA_IntraNR_Handover_NR5GC |
| MCC160 Comment | Accepted. |

Before Change

function f_TC_8_1_4_1_7_x_NR5GC_TestBody(NR_CellId_Type p_SourcePCell, NR_CellId_Type p_SourceSCell, NR_CellId_Type p_TargetPCell, NR_CellId_Type p_TargetSCell) runs on NR5GC_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={cs_NR_CellPower(p_SourcePCell, -88, -88), //TDD power levels are FFS cs_NR_CellPower(p_SourceSCell, -88, -88), //TDD power levels are FFS cs_NR_CellPower(p_TargetPCell, -88, -88), //TDD power levels are FFS cs_NR_CellPower(p_TargetSCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 :={cs_NR_CellPower(p_SourcePCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(p_SourceSCell, -88, -88), //TDD power levels are FFS cs_NR_CellPower(p_TargetPCell, -88, -88), //TDD power levels are FFS cs_NR_CellPower(p_TargetSCell, -88, -88) //TDD power levels are FFS }; ...

After Change

function f_TC_8_1_4_1_7_x_NR5GC_TestBody(NR_CellId_Type p_SourcePCell, NR_CellId_Type p_SourceSCell, NR_CellId_Type p_TargetPCell, NR_CellId_Type p_TargetSCell) runs on NR5GC_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={cs_NR_CellPower(p_SourcePCell, -88, -88), //TDD power levels are FFS cs_NR_CellPower(p_SourceSCell, -88, -88), //TDD power levels are FFS cs_NR_CellPower(p_TargetPCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //TDD power levels are FFS cs_NR_CellPower(p_TargetSCell, -88, -88) }; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 :={cs_NR_CellPower(p_SourcePCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(p_SourceSCell, -88, -88), //TDD power levels are FFS cs_NR_CellPower(p_TargetPCell, -88, -88), //TDD power levels are FFS cs_NR_CellPower(p_TargetSCell, -88, -88) //TDD power levels are FFS }; ...

## Change 2

| Function name | f_NR5GC_508RRC_IntraNR_CA_HO_InterCell() |
| --- | --- |
| Reason for change | - Cell3 (source SCell) should have the same C_RNTI as cell1 (source PCell) - In the step 4, PCell is changed via reconfigurationWithSync and a new SCell is added. However, the previously configured SCell (which becomes the new PCell) is not released. As per 38.331, SCells shall be explicitly released. - The variable v_SCGConfig_SCell is not initialsied when there is no value for p_TargetSCellId. This results in uninitialized value passed into f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common() causing a run-time error in this function. |
| Summary of change | 1.Change tsc_C_RNTI_Value3 to tsc_C_RNTI_Value1. 2.Add v_SCellToReleaseList with tsc_SCellIndex_1 in rrcReconfiguration. Note : An associated prose CR on 38.523-1 will be raised at RAN#90 3.Remove passing v_SCGConfig_SCell to ‘f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(…) |
| TTCN module | NR5GC_Handover |
| MCC160 Comment | - Not needed, the new C-RNTI value is also set in the Scell in f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common, step 8. - Accepted in principle, conditional to prose CR approval. See actual implementation below. - Not needed. In this function, the variable is initialised to omit. See also comments to Change 3. |

Before Change

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(NR_CellId_Type p_SourcePCellId, NR_CellId_Type p_SourceSCellId, NR_CellId_Type p_TargetPCellId, template(omit) NR_CellId_Type p_TargetSCellId, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (omit) MasterKeyUpdate p_MasterKeyUpdate := omit ) runs on NR5GC_PTC { var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetPCellId); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var NextHopChainingCount v_NCC; var template (omit) CellGroupConfig.sCellToAddModList v_SCellToAddModList := omit; if (isvalue(p_MasterKeyUpdate)){ v_NCC := valueof(p_MasterKeyUpdate.nextHopChainingCount); } else { v_NCC := tsc_NR_38508_NextHopChainingCount; } if (isvalue(p_TargetSCellId)) { //Set sCellToAddModList with p_TargetSCellId info v_SCGConfig_SCell := f_NR_GetSCGConfig_SCellAdd(valueof(p_TargetSCellId),tsc_SCellIndex_1);//@sic R5s201249 sic@ v_SCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; } else { // Release the p_SourceSCellId v_SCellToReleaseList := {tsc_SCellIndex_1}; } v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_TargetPCellId, p_RNTI_Value, p_RBConfig_KeyChange, -, -, -, p_MasterKeyUpdate, -, -, v_SCellToAddModList, v_SCellToReleaseList ); //Steps 1 to 18.1 f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(p_SourcePCellId, p_SourceSCellId, p_TargetPCellId, p_TargetSCellId, v_SCGConfig_SCell, p_RNTI_Value, p_RBConfig_KeyChange, v_RRCReconfiguration, v_RachProcedureConfig, v_DRBInfoList, -, -, -, v_NCC, - ); }

After Change

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(NR_CellId_Type p_SourcePCellId, NR_CellId_Type p_SourceSCellId, NR_CellId_Type p_TargetPCellId, template(omit) NR_CellId_Type p_TargetSCellId, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value1,//@sic R5s201249 sic@ boolean p_RBConfig_KeyChange := true, template (omit) MasterKeyUpdate p_MasterKeyUpdate := omit ) runs on NR5GC_PTC { var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetPCellId); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var NextHopChainingCount v_NCC; var template (omit) CellGroupConfig.sCellToAddModList v_SCellToAddModList := omit; if (isvalue(p_MasterKeyUpdate)){ v_NCC := valueof(p_MasterKeyUpdate.nextHopChainingCount); } else { v_NCC := tsc_NR_38508_NextHopChainingCount; } if (isvalue(p_TargetSCellId)) { //Set sCellToAddModList with p_TargetSCellId info v_SCGConfig_SCell := f_NR_GetSCGConfig_SCellAdd(valueof(p_TargetSCellId),tsc_SCellIndex_1);//@sic R5s201249 sic@ v_SCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCellToReleaseList := {tsc_SCellIndex_1}; } else { // Release the p_SourceSCellId v_SCellToReleaseList := {tsc_SCellIndex_1}; } v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_TargetPCellId, p_RNTI_Value, p_RBConfig_KeyChange, -, -, -, p_MasterKeyUpdate, -, -, v_SCellToAddModList, v_SCellToReleaseList ); //Steps 1 to 18.1 f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(p_SourcePCellId, p_SourceSCellId, p_TargetPCellId, p_TargetSCellId, //REMOVED v_SCGConfig_SCell,//@sic R5s201249 sic@ p_RNTI_Value, p_RBConfig_KeyChange, v_RRCReconfiguration, v_RachProcedureConfig, v_DRBInfoList, -, -, -, v_NCC, - ); }

MCC160 proposed implementation for change 2.2

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(NR_CellId_Type                       p_SourcePCellId,

NR_CellId_Type                       p_SourceSCellId,

NR_CellId_Type                       p_TargetPCellId,

template(omit) NR_CellId_Type        p_TargetSCellId,

RNTI_Value                           p_RNTI_Value :=  tsc_C_RNTI_Value3,

boolean                              p_RBConfig_KeyChange := true,

template (omit) MasterKeyUpdate      p_MasterKeyUpdate := omit

) runs on NR5GC_PTC

{

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetPCellId);

var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA

var template (value) DL_DCCH_Message v_RRCReconfiguration;

var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList();

var NextHopChainingCount v_NCC;

var template (omit) CellGroupConfig.sCellToAddModList v_SCellToAddModList := omit;

var template (omit) CellGroupConfig.sCellToReleaseList v_SCellToReleaseList := omit;

var template (omit) CellGroupConfig v_SCGConfig_SCell := omit;//@sic R5s201249 sic@

if (isvalue(p_MasterKeyUpdate)){

v_NCC := valueof(p_MasterKeyUpdate.nextHopChainingCount);

} else {

v_NCC := tsc_NR_38508_NextHopChainingCount;

}

if (isvalue(p_TargetSCellId)) {

//Set sCellToAddModList with p_TargetSCellId info

v_SCGConfig_SCell := f_NR_GetSCGConfig_SCellAdd(valueof(p_TargetSCellId),tsc_SCellIndex_1);//@sic R5s201249 sic@

v_SCellToAddModList := v_SCGConfig_SCell.sCellToAddModList;

if (p_SourceSCellId == p_TargetPCellId) { //PCell becomes SCell

v_SCellToReleaseList := {tsc_SCellIndex_1}; //@sic R5s210079 sic@

}

} else {

// Release the p_SourceSCellId

v_SCellToReleaseList := {tsc_SCellIndex_1};

}

## Change 3

| Function name | f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(…) |
| --- | --- |
| Reason for change | The variable v_SCGConfig_SCell in function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell() is not initialsied when there is no value for p_TargetSCellId. This results in uninitialized value passed into f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common() causing a run-time error in this function. |
| Summary of change | Remove parameter p_SCGConfig_SCell and add initialisation of SCGConfig for SCell |
| TTCN module | NR5GC_Handover |
| MCC160 Comment | In the code, the value of v_SCGConfig_Scell is only used when p_TargetSCellId has a value. This has been taken care of in the coding, nevertheless, the operator “isvalue” should have been used instead of “ispresent”. See proposed implementation below. |

Before Change

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(NR_CellId_Type p_SourcePCellId, NR_CellId_Type p_SourceSCellId, NR_CellId_Type p_TargetPCellId, template(omit) NR_CellId_Type p_TargetSCellId, template(omit) CellGroupConfig p_SCGConfig_SCell, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (value) DL_DCCH_Message p_RRCReconfiguration, template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig, template (value) NR_DRBInfoList_Type p_DrbInfoList, template (value) UL_GrantConfig_Type p_UL_GrantConfig_SourceCell := cs_UL_GrantConfig_OnSR, template (value) UL_GrantConfig_Type p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete, NextHopChainingCount p_NCC := tsc_NR_38508_NextHopChainingCount, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit ) runs on NR_BASE_PTC { //@sic R5s201249: added p_SCGConfig_SCell sic@ var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(p_DrbInfoList); var IntegerList_Type v_DrbIdList; var template(value) TimingInfo_Type v_TimingInfo; //@sic R5s201387 sic@ v_Timing := f_NR_GetNextSendOccasion_DL(p_SourcePCellId, 700); // 700 ms // Time for schedule of DL RRC message v_Timing1 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 10); // time for Target SCell-> PCell configuration SS configuration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 20); // time for Target PCell SS configuration // 1. Source PCell: Stop periodic TA. // NOTE 1: Unless explicitly specified UL grant configuration keeps configured as per default at the source cell. f_NR_ULGrantConfiguration_Common(p_SourcePCellId, cs_TimingInfo_NR(v_Timing), -, cs_NR_UplinkTimeAlignment_Stop, p_UL_GrantConfig_SourceCell); // 2. Target PCell: Configure target PCell for no RACH response transmission. //10. Target PCell: Configure UL grant configuration ("OnSR", periodic TA is not started). f_NR_ULGrantConfiguration_Common(p_TargetPCellId, v_TimingInfo, -, -, p_UL_GrantConfig_TargetCell); //11. Target PCell: Configure Target PCell as PCell. //12. Target SCell: Configure Target SCell as SCell with new PCell association. //NOTE 5: The FollowOnFlag is set to true in the ASP reconfiguring PCell. if (ispresent (p_TargetSCellId)) { f_NR_ConfigPCell_AddRelSCells(p_TargetPCellId, {valueof(p_TargetSCellId)}, tsc_PCellIndex_0, v_TimingInfo, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-206369 sic@ //Calculate T2 in the TargetSCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), valueof(p_TargetSCellId)); f_NR_ConfigSCell(valueof(p_TargetSCellId), p_SCGConfig_SCell, v_TimingInfo); } …

After Change

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(NR_CellId_Type p_SourcePCellId, NR_CellId_Type p_SourceSCellId, NR_CellId_Type p_TargetPCellId, template(omit) NR_CellId_Type p_TargetSCellId, //REMOVED template(omit) CellGroupConfig p_SCGConfig_SCell,//@sic R5s201249 sic@ RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (value) DL_DCCH_Message p_RRCReconfiguration, template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig, template (value) NR_DRBInfoList_Type p_DrbInfoList, template (value) UL_GrantConfig_Type p_UL_GrantConfig_SourceCell := cs_UL_GrantConfig_OnSR, template (value) UL_GrantConfig_Type p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete, NextHopChainingCount p_NCC := tsc_NR_38508_NextHopChainingCount, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit ) runs on NR_BASE_PTC { //@sic R5s201249: added p_SCGConfig_SCell sic@ var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(p_DrbInfoList); var IntegerList_Type v_DrbIdList; var template(value) TimingInfo_Type v_TimingInfo; //@sic R5s201387 sic@ var template(value) CellGroupConfig v_SCGConfig;//@sic R5s201249 sic@ v_Timing := f_NR_GetNextSendOccasion_DL(p_SourcePCellId, 700); // 700 ms // Time for schedule of DL RRC message v_Timing1 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 10); // time for Target SCell-> PCell configuration SS configuration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 20); // time for Target PCell SS configuration // 1. Source PCell: Stop periodic TA. // NOTE 1: Unless explicitly specified UL grant configuration keeps configured as per default at the source cell. f_NR_ULGrantConfiguration_Common(p_SourcePCellId, cs_TimingInfo_NR(v_Timing), -, cs_NR_UplinkTimeAlignment_Stop, p_UL_GrantConfig_SourceCell); // 2. Target PCell: Configure target PCell for no RACH response transmission. //10. Target PCell: Configure UL grant configuration ("OnSR", periodic TA is not started). f_NR_ULGrantConfiguration_Common(p_TargetPCellId, v_TimingInfo, -, -, p_UL_GrantConfig_TargetCell); //11. Target PCell: Configure Target PCell as PCell. //12. Target SCell: Configure Target SCell as SCell with new PCell association. //NOTE 5: The FollowOnFlag is set to true in the ASP reconfiguring PCell. if (ispresent (p_TargetSCellId)) { f_NR_ConfigPCell_AddRelSCells(p_TargetPCellId, {valueof(p_TargetSCellId)}, tsc_PCellIndex_0, v_TimingInfo, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-206369 sic@ //Calculate T2 in the TargetSCell v_SCGConfig := f_NR_GetSCGConfig_SCellAdd(p_TargetSCellId, tsc_SCellIndex_1, false, omit);//@sic R5s201249 sic@ v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), valueof(p_TargetSCellId)); f_NR_ConfigSCell(valueof(p_TargetSCellId), v_SCGConfig, v_TimingInfo);//@sic R5s201249 sic@ } …

MCC160 proposed implementation

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(NR_CellId_Type                               p_SourcePCellId,

NR_CellId_Type                               p_SourceSCellId,

NR_CellId_Type                               p_TargetPCellId,

template(omit) NR_CellId_Type                p_TargetSCellId,

template(omit) CellGroupConfig               p_SCGConfig_SCell,

RNTI_Value                                   p_RNTI_Value :=  tsc_C_RNTI_Value3,

boolean                                      p_RBConfig_KeyChange := true,

template (value) DL_DCCH_Message             p_RRCReconfiguration,

template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig,

template (value) NR_DRBInfoList_Type         p_DrbInfoList,

template (value) UL_GrantConfig_Type         p_UL_GrantConfig_SourceCell := cs_UL_GrantConfig_OnSR,

template (value) UL_GrantConfig_Type         p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR,

template (present) UL_DCCH_Message           p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete,

NextHopChainingCount                         p_NCC := tsc_NR_38508_NextHopChainingCount,

template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit

) runs on NR_BASE_PTC

{ //@sic R5s201249: added p_SCGConfig_SCell sic@

...

//At time T2, steps 7-12:

//7. Target PCell: Inform the SS about the HO and about the source PCell id.

f_NR_SS_PdcpHandoverCtrl(p_TargetPCellId, cas_NR_PdcpHandoverInit_REQ(p_SourcePCellId, p_TargetPCellId, cs_TimingInfo_NR(v_Timing2)));

//8. Target PCell: Configure RACH procedure either dedicated or C-RNTI based.

// NOTE 4: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI

f_NR_CellInfo_SetRNTI(p_TargetPCellId, p_RNTI_Value);

//Calculate T2 in the TargetPCell

v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), p_TargetPCellId);

if (isvalue(p_TargetSCellId)) {

//@sic R5s201249 sic@

f_NR_CellInfo_SetRNTI(valueof(p_TargetSCellId), p_RNTI_Value);

f_NR_SS_C_RNTI_Config(valueof(p_TargetSCellId), p_RNTI_Value);

}

f_NR_SS_C_RNTI_Config(p_TargetPCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag);  //@sic R5-206369 sic@

f_NR_SS_RachProcedureConfigHO(p_TargetPCellId, p_RachProcedureConfig, v_TimingInfo, p_RACH_ConfigDedicated);

//9. Target PCell: Activate security.

if ((p_NCC > tsc_NR_38508_NextHopChainingCount) or p_RBConfig_KeyChange) { // @sic R5s190795 R5s200159 sic@

f_NR_SS_AS_ActivateSecurity_Current(p_TargetPCellId, p_NCC, v_TimingInfo);

} else {

f_NR_SS_AS_ActivateSecurity_NoRefresh(p_TargetPCellId, v_TimingInfo);  //@sic R5s200102 sic@

}

//10. Target PCell: Configure UL grant configuration ("OnSR", periodic TA is not started).

f_NR_ULGrantConfiguration_Common(p_TargetPCellId, v_TimingInfo, -, -, p_UL_GrantConfig_TargetCell);

//11. Target PCell: Configure Target PCell as PCell.

//12. Target SCell: Configure Target SCell as SCell with new PCell association.

//NOTE 5: The FollowOnFlag is set to true in the ASP reconfiguring PCell.

if (isvalue (p_TargetSCellId)) {

f_NR_ConfigPCell_AddRelSCells(p_TargetPCellId, {valueof(p_TargetSCellId)}, tsc_PCellIndex_0, v_TimingInfo, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag);  //@sic R5-206369 sic@

//Calculate T2 in the TargetSCell

v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), valueof(p_TargetSCellId));

f_NR_ConfigSCell(valueof(p_TargetSCellId), p_SCGConfig_SCell, v_TimingInfo);

}

// After time T2 (without activation time):

// 13. Target PCell: Receive RRCReconfigurationComplete.

SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetPCellId, p_RRCConnReconfigComplete));

//14. Target Pell: Start periodic TA.

f_NR_ULGrantConfiguration_Common(p_TargetPCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Start, -);

//15. Target Pell: Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS REPORT PDU).

f_NR_SS_PdcpHandoverCtrl(p_TargetPCellId, cas_NR_PdcpHandoverComplete_REQ(p_TargetPCellId));

//16. Target PCell: Re-configure RACH procedure as for initial access.

f_NR_SS_RachProcedureConfig_Def(p_TargetPCellId);

//17. Source PCell: If source PCell is not target SCell

//17.1 Reset SRBs and release DRBs.

//17.2 Configure from PCell to normal cell.

if (isvalue (p_TargetSCellId)) {

if (p_SourcePCellId != valueof(p_TargetSCellId)) {

v_DrbIdList := f_NR_GetActiveDRBs(p_SourcePCellId);

f_NR_ReconfigurePCellToNormalCell( p_SourcePCellId, tsc_PCellIndex_0, -, v_DrbIdList );

}

}

//18. Source SCell: If source SCell is neither target PCell nor target SCell:

//18.1 Re-configure source SCell to normal cell and remove source SCell from the SCell list in the source PCell.

if (not(match(p_SourceSCellId, p_TargetPCellId)) and not match(p_SourceSCellId, p_TargetSCellId)) {

f_NR_ReconfigSCellToNormalCell(p_SourceSCellId, v_RadioBearerList, -);

f_NR_Rel_SCell_Common( p_SourcePCellId,  p_SourceSCellId, tsc_PCellIndex_0, cs_TimingInfo_Now);//Release SCell from PCell @sic R5s201249 sic@

}

}

## Change 4

| Function name | f_NR_TimingOtherCell() |
| --- | --- |
| Reason for change | The current implementation actually can support timing calculation for cells with different numerology. But the returned timing is using the numerologiy of target cell not source cell. |
| Summary of change | Removed the error code. Use v_SlotTimingInfo based on target cell to calculate v_SubFrameTiming. Removing call to f_CellTimingInfo_TimingOtherCell() |
| TTCN module | NR_ConfigurationSteps |
| MCC160 Comment | Accepted. See proposed implementation below. |

Before Change

function f_NR_TimingOtherCell(NR_CellId_Type p_NR_CellId1, template (value) TimingInfo_Type p_TimingInfo1, NR_CellId_Type p_NR_CellId2) runs on NR_BASE_PTC return template (value) TimingInfo_Type { var CellTimingInfo_Type v_CellTimingInfo1 := f_NR_CellInfo_GetCellTimingInfo(p_NR_CellId1); var CellTimingInfo_Type v_CellTimingInfo2 := f_NR_CellInfo_GetCellTimingInfo(p_NR_CellId2); var SubcarrierSpacing v_SCS_Cell1 := f_NR_CellInfo_GetSCS(p_NR_CellId1); var SubcarrierSpacing v_SCS_Cell2 := f_NR_CellInfo_GetSCS(p_NR_CellId2); if (v_SCS_Cell1 != v_SCS_Cell2) { FatalError(__FILE__, __LINE__, "timing calculation not supported (yet) for cells with different numerology"); } return f_CellTimingInfo_TimingOtherCell(v_CellTimingInfo1, p_TimingInfo1, v_CellTimingInfo2); }

After Change

function f_NR_TimingOtherCell(NR_CellId_Type p_NR_CellId1, template (value) TimingInfo_Type p_TimingInfo1, NR_CellId_Type p_NR_CellId2) runs on NR_BASE_PTC return template (value) TimingInfo_Type { var CellTimingInfo_Type v_CellTimingInfo1 := f_NR_CellInfo_GetCellTimingInfo(p_NR_CellId1); var CellTimingInfo_Type v_CellTimingInfo2 := f_NR_CellInfo_GetCellTimingInfo(p_NR_CellId2); var SubcarrierSpacing v_SCS_Cell1 := f_NR_CellInfo_GetSCS(p_NR_CellId1); var SubcarrierSpacing v_SCS_Cell2 := f_NR_CellInfo_GetSCS(p_NR_CellId2); var template(value) SlotTimingInfo_Type v_SlotTimingInfo;//KS R5s21xxx var integer v_SystemTimeUnits; var SubFrameTiming_Type v_SubFrameTiming; select (v_SCS_Cell2){ case (kHz15) {v_SlotTimingInfo := cs_SlotTimingInfo_Numerology0;} case (kHz30) {v_SlotTimingInfo := cs_SlotTimingInfo_Numerology1(0);} case (kHz60) {v_SlotTimingInfo := cs_SlotTimingInfo_Numerology2(0);} case (kHz120){v_SlotTimingInfo := cs_SlotTimingInfo_Numerology3(0);} case else {FatalError(__FILE__, __LINE__, "Invalid SubCarrierSpacing");} } if (f_TimingInfo_IsNow(p_TimingInfo1)) { //@sic R5s180633 sic@ return cs_TimingInfo_Now; } v_SubFrameTiming := valueof(p_TimingInfo1.SubFrame); v_SystemTimeUnits := f_SystemTimeUnits_ConvertFromSubFrameTiming(v_SubFrameTiming) + f_SystemTimeUnits_ConvertFromCellTimingInfo(v_CellTimingInfo1) - f_SystemTimeUnits_ConvertFromCellTimingInfo(v_CellTimingInfo2); v_SubFrameTiming := f_SystemTimeUnits_ConvertToSubFrameTiming(v_SystemTimeUnits, v_SlotTimingInfo); return cs_TimingInfo_NR(v_SubFrameTiming); //Removed return f_CellTimingInfo_TimingOtherCell(v_CellTimingInfo1, p_TimingInfo1, v_CellTimingInfo2); }

MCC160 proposed implementation

function f_NR_TimingOtherCell(NR_CellId_Type p_NR_CellId1,

template (value) TimingInfo_Type p_TimingInfo1,

NR_CellId_Type p_NR_CellId2) runs on NR_BASE_PTC return template (value) TimingInfo_Type

{

var CellTimingInfo_Type v_CellTimingInfo1 := f_NR_CellInfo_GetCellTimingInfo(p_NR_CellId1);

var CellTimingInfo_Type v_CellTimingInfo2 := f_NR_CellInfo_GetCellTimingInfo(p_NR_CellId2);

var SubcarrierSpacing v_SCS_Cell1 := f_NR_CellInfo_GetSCS(p_NR_CellId1);

var SubcarrierSpacing v_SCS_Cell2 := f_NR_CellInfo_GetSCS(p_NR_CellId2);

var template(omit) SlotTimingInfo_Type v_SlotTimingInfo := omit;//@sic R5s210079 sic@

if (v_SCS_Cell1 != v_SCS_Cell2) { //@sic R5s210079 sic@

select (v_SCS_Cell2){

case (kHz15) {v_SlotTimingInfo := cs_SlotTimingInfo_Numerology0;}

case (kHz30) {v_SlotTimingInfo := cs_SlotTimingInfo_Numerology1(0);}

case (kHz60) {v_SlotTimingInfo := cs_SlotTimingInfo_Numerology2(0);}

case (kHz120){v_SlotTimingInfo := cs_SlotTimingInfo_Numerology3(0);}

case else {FatalError(__FILE__, __LINE__, "Invalid SubCarrierSpacing");}

}

}

return f_CellTimingInfo_TimingOtherCell(v_CellTimingInfo1, p_TimingInfo1, v_CellTimingInfo2, v_SlotTimingInfo);

}

function f_CellTimingInfo_TimingOtherCell(CellTimingInfo_Type p_CellTimingInfo1,

template (value) TimingInfo_Type p_TimingInfo1,

CellTimingInfo_Type p_CellTimingInfo2,

template(omit) SlotTimingInfo_Type p_TargetSlotTimingInfo := omit

) return template (value) TimingInfo_Type

{ /* NOTE: As illustrated in 36.523-3 figure 7.4.3.1-3 the timing offset between a cell is a delay compared to a reference cell0 with no timing offset

=> to calculate the 'normalised' time of cell0 the timing offset of a cell needs to be added to the timing retrieved for this cell

@sic R5s180533: generic implementation using f_SystemTimeUnits_ConvertFromSubFrameTiming, f_SystemTimeUnits_ConvertFromCellTimingInfo and f_SystemTimeUnits_ConvertToSubFrameTiming sic@ */

//@sic R5s210079: added p_TargetSlotTimingInfo sic@

var integer v_SystemTimeUnits;

var SubFrameTiming_Type v_SubFrameTiming;

var SlotTimingInfo_Type v_SlotTimingInfo;

if (f_TimingInfo_IsNow(p_TimingInfo1)) { //@sic R5s180633 sic@

return cs_TimingInfo_Now;

}

v_SubFrameTiming := valueof(p_TimingInfo1.SubFrame);

v_SystemTimeUnits := f_SystemTimeUnits_ConvertFromSubFrameTiming(v_SubFrameTiming)     // SystemTimeUnits for cell 1

+ f_SystemTimeUnits_ConvertFromCellTimingInfo(p_CellTimingInfo1)                     // add SFN offset and TCell of cell 1 (=> normalised time) @sic R5s120289: offset needs to be added but not substracted sic@

- f_SystemTimeUnits_ConvertFromCellTimingInfo(p_CellTimingInfo2);                    // subtract SFN offset and TCell of cell 2                 @sic R5s120289: offset needs to be substracted but not added sic@

if (isvalue(p_TargetSlotTimingInfo)) {

v_SlotTimingInfo := valueof(p_TargetSlotTimingInfo);

} else {

v_SlotTimingInfo := v_SubFrameTiming.Slot

}

v_SubFrameTiming := f_SystemTimeUnits_ConvertToSubFrameTiming(v_SystemTimeUnits, v_SlotTimingInfo);

return cs_TimingInfo_NR(v_SubFrameTiming);

}