3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210085

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1519 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR5GC idle mode test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following deregistration of the UE, all stored information about DRBs and PDU sessions should be cleared since these no longer exist in the UE. For some test cases this is not being done via a call to f_NR5GC_MobileInfo_Init(), this can give execution issues later in the test case (e.g. AT commands for creating new PDU sessions are not issued) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add calls to f_NR5GC_MobileInfo_Init() for some test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1.NR5GC, 6.1.1.2.NR5GC, 6.1.1.5.NR5GC, 6.3.1.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	5

2.3 Change 3	8

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

  - Change 1

| Function name | e.g. f_TC_6_1_1_1_NR5GC_TestBody() |
| --- | --- |
| Reason for change | Following deregistration of the UE, all stored information about DRBs and PDU sessions should be cleared since these no longer exist in the UE. For some test cases this is not being done via a call to f_NR5GC_MobileInfo_Init(), this can give execution issues later in the test case (e.g. AT commands for creating new PDU sessions are not issued) |
| Summary of change | e.g. for 6.1.1.1 : Add call to f_NR5GC_MobileInfo_Init() before steps 22 and 30. Note : The same change is also required for the following test cases: - 6.1.1.2 (before steps 22, 29 and 36) - 6.1.1.5 (before steps 1, 9 and 20) - 6.3.1.8 (before step 7) |
| TTCN module | Idle_PLMNSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_6_1_1_1_NR5GC_TestBody() runs on NR5GC_PTC { ... ... // @siclog "Step 4 - 21" siclog@ // Steps 3-20 of the registration procedure described in TS 38.508-1 table 4.5.2.2-2 are performed on NR Cell 12. f_NR5GC_RRC_Idle_Steps3_20(nr_Cell12, TESTMode_OFF); ... ... // @siclog "Step 22A-22B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 23b1-23b2 Void" siclog@ // @siclog "Step 23b3-23b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 22B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 1 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 23a1-23a17a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 22B, THEN Steps 4 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1 f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } ... ... // @siclog "Step 29A-29B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell13); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell13, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 29b1-29b2 Void" siclog@ // @siclog "Step 29b3-29b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 29B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 13 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell13, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 29a1-29a17a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 29B, THEN Steps 4 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 13 f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell13, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // @siclog "Step 30" siclog@ // SS adjusts cell levels according to row T3 of table 6.1.1.1.3.2-1 for FR1 and table 6.1.1.1.3.2-2 for FR2. f_NR_SetCellPowerList(v_CellPowerList_AtT3); t_HpPlmn_lower.start(f_NR_SetTimerToleranceMin(nr_Cell13, nonProtocolTimer, 120.0)); // @sic R5-205320 sic@ t_HpPlmn_upper.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0)); // @siclog "Step 31" siclog@ // Does the UE send a RRCSetupRequest on NR Cell 2 after 360 seconds, but before 660 seconds after step 30? f_NR_ULGrantConfiguration_Start(nr_Cell2,-,-,-,cs_NR_UplinkTimeAlignment_Stop); alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { // verdict fail RRCSetupRequest received on PLMNN3 /NR Cell 2 within t_HpPlmn_lower time f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 31"); } [] t_HpPlmn_lower.timeout { alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_HpPlmn_upper.stop; // verdict pass RRCSetupRequest received on PLMN3 /NR Cell 13 after t_HpPlmn_lower time but before t_HpPlmn_upper f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 31"); } [] t_HpPlmn_upper.timeout { // verdict fail No RRCSetupRequest received f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 31"); } } } } // @siclog "Step 31A-31B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell2); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell2, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 32b1-32b2 Void" siclog@ // @siclog "Step 32b3-32b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 31B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 14 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell2, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 32a1-32a17a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 31B, THEN Steps 4 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 14 f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell2, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 33 - 36" siclog@ // Void }

After Change:

function f_TC_6_1_1_1_NR5GC_TestBody() runs on NR5GC_PTC { ... ... // @siclog "Step 4 - 21" siclog@ // Steps 3-20 of the registration procedure described in TS 38.508-1 table 4.5.2.2-2 are performed on NR Cell 12. f_NR5GC_RRC_Idle_Steps3_20(nr_Cell12, TESTMode_OFF); f_NR5GC_MobileInfo_Init(); ... ... // @siclog "Step 22A-22B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 23b1-23b2 Void" siclog@ // @siclog "Step 23b3-23b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 22B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 1 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 23a1-23a17a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 22B, THEN Steps 4 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1 f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobileInfo_Init(); ... ... // @siclog "Step 29A-29B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell13); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell13, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 29b1-29b2 Void" siclog@ // @siclog "Step 29b3-29b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 29B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 13 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell13, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 29a1-29a17a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 29B, THEN Steps 4 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 13 f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell13, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobileInfo_Init(); // @siclog "Step 30" siclog@ // SS adjusts cell levels according to row T3 of table 6.1.1.1.3.2-1 for FR1 and table 6.1.1.1.3.2-2 for FR2. f_NR_SetCellPowerList(v_CellPowerList_AtT3); t_HpPlmn_lower.start(f_NR_SetTimerToleranceMin(nr_Cell13, nonProtocolTimer, 120.0)); // @sic R5-205320 sic@ t_HpPlmn_upper.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0)); // @siclog "Step 31" siclog@ // Does the UE send a RRCSetupRequest on NR Cell 2 after 360 seconds, but before 660 seconds after step 30? f_NR_ULGrantConfiguration_Start(nr_Cell2,-,-,-,cs_NR_UplinkTimeAlignment_Stop); alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { // verdict fail RRCSetupRequest received on PLMNN3 /NR Cell 2 within t_HpPlmn_lower time f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 31"); } [] t_HpPlmn_lower.timeout { alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_HpPlmn_upper.stop; // verdict pass RRCSetupRequest received on PLMN3 /NR Cell 13 after t_HpPlmn_lower time but before t_HpPlmn_upper f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 31"); } [] t_HpPlmn_upper.timeout { // verdict fail No RRCSetupRequest received f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.1 Step 31"); } } } } // @siclog "Step 31A-31B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell2); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell2, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 32b1-32b2 Void" siclog@ // @siclog "Step 32b3-32b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 31B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 14 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell2, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 32a1-32a17a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 31B, THEN Steps 4 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 14 f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell2, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 33 - 36" siclog@ // Void }