3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210093

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1527 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NR RLC test case 7.1.2.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - At step 1A TTCN should transmit the W+1th PDU, not the entire list - In the loop at step1 the timing v_TimingDL be calculated after the RLC SDU is sent (not before) - The timing of sending w+1 RLC PDU is handled by v_ (adding 20*p_InnerLoop to current time). SlotOffset is not required. - Redundant assigments of v_DataPerSlotArray, TimingDL and v_DataPerSlotList |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Transmit v_DataPerSlotList[0] at step 1A - Move caclualtion ofv_TimingDL - Change the SlotOffset of the w+1st DRB 0. - Remove some redundant assignments. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_7_1_2_3_5and5a(.) |
| --- | --- |
| Reason for change | - At step 1A TTCN should transmit the W+1th PDU, not the entire list - In the loop at step1 the timing v_TimingDL be calculated after the RLC SDU is sent (not before) - The timing of sending w+1 RLC PDU is handled by v_ (adding 20*p_InnerLoop to current time). SlotOffset is not required. - Redundant assigments of v_DataPerSlotArray, TimingDL and v_DataPerSlotList |
| Summary of change | - Transmit v_DataPerSlotList[0] at step 1A - Move caclualtion ofv_TimingDL - Change the SlotOffset of the w+1st DRB 0. - Remove some redundant assignments. |
| TTCN module | RLC_TC_Common_NR |
| MCC160 Comment | - Accepted - Accepted - Accepted - Accepted |

Before Change

function fl_TC_7_1_2_3_5and5a(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC { ………… // generate 1 more RLC SDU for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); v_NR_RLC_AMD_PDU_TX := f_NR_RLC_Get_AMD_FullSDU_TX(p_RLC_Rec, v_Poll, v_NR_RLC_DataList[0]); v_DataPerSlotList[0] := cs_NR_DRB_DataPerSlotList_DL_SingleAmdPdu(v_SlotsNumber * 20 * i, -, v_NR_RLC_AMD_PDU_TX); v_DataPerSlotArray[p_OuterLoop] := v_DataPerSlotList; // save W+1st RLC SDU @sic R5s201490 sic@ // EXCEPTION: The SS is configured for step 1 500 ms in advance. The transmissions are performed every second radio frame. // Step 2 is started 100 ms after the first DL AMD PDU has been transmitted in step 1. v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 500); v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100); // @siclog "Step 2" siclog@ // In the following steps the SS transmits 1 UL grant in every second radio frame to enable the UE to return each received AMD PDU in one looped back AMD PDU. // Step 2 executed before step 1 to ensure the UL grant is configured in time f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); // EXCEPTION: Step 1 shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration size). // @siclog "Step 1" siclog@ // The SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size. // Each RLC Data PDU contains one RLC SDU. The SS transmits an AMD PDU containing a SDU to the UE. for (j := 0; j < p_OuterLoop; j:= j + 1) { v_DataPerSlotList := v_DataPerSlotArray [j]; v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); DRB.send (cas_NR_DRB_COMMON_REQ_DataPerSlot(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_DataPerSlotArray [j])); } // @siclog "Step 1A" siclog@ // The SS transmits one RLC PDU in the slot following the transmissions of step 1. v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); v_DataPerSlotList := v_DataPerSlotArray [p_OuterLoop]; // get W+1st RLC SDU @sic R5s201490 sic@ DRB.send (cas_NR_DRB_COMMON_REQ_DataPerSlot(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_DataPerSlotList)); …………………….. }

After Change

function fl_TC_7_1_2_3_5and5a(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC { ………………………………… // generate 1 more RLC SDU for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); v_NR_RLC_AMD_PDU_TX := f_NR_RLC_Get_AMD_FullSDU_TX(p_RLC_Rec, v_Poll, v_NR_RLC_DataList[0]); v_DataPerSlotList[0] := cs_NR_DRB_DataPerSlotList_DL_SingleAmdPdu(0, -, v_NR_RLC_AMD_PDU_TX); //REMOVED v_DataPerSlotArray[p_OuterLoop] := v_DataPerSlotList; // save W+1st RLC SDU @sic R5s201490 sic@ // EXCEPTION: The SS is configured for step 1 500 ms in advance. The transmissions are performed every second radio frame. // Step 2 is started 100 ms after the first DL AMD PDU has been transmitted in step 1. v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 500); v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100); // @siclog "Step 2" siclog@ // In the following steps the SS transmits 1 UL grant in every second radio frame to enable the UE to return each received AMD PDU in one looped back AMD PDU. // Step 2 executed before step 1 to ensure the UL grant is configured in time f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); // EXCEPTION: Step 1 shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration size). // @siclog "Step 1" siclog@ // The SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size. // Each RLC Data PDU contains one RLC SDU. The SS transmits an AMD PDU containing a SDU to the UE. for (j := 0; j < p_OuterLoop; j:= j + 1) { //REMOVED v_DataPerSlotList := v_DataPerSlotArray [j]; //MOVED v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); DRB.send (cas_NR_DRB_COMMON_REQ_DataPerSlot(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_DataPerSlotArray [j])); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); } // @siclog "Step 1A" siclog@ // The SS transmits one RLC PDU in the slot following the transmissions of step 1. //REMOVED v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); //REMOVED v_DataPerSlotList := v_DataPerSlotArray [p_OuterLoop]; // get W+1st RLC SDU @sic R5s201490 sic@ DRB.send (cas_NR_DRB_COMMON_REQ_DataPerSlot(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), {v_DataPerSlotList[0]})); …………………………………… }