3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210103

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1531 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_EUTRA38_TAUReqFromN1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current implementation of the function f_EUTRA38_TAUReqFromN1 variable v_ExistingPDNs is declared but not initialized. This can cause run-time errors later when f_EUTRA38_TAUReqFromN1f_EUTRA38_MobileInfo_SetConfiguredPDNInfoList(v_ExistingPDNs); is called. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of initialising var ConfiguredPDNInfoList_Type v_ExistingPDNs; Initialised var ConfiguredPDNInfoList_Type v_ExistingPDNs :={}; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Runtime errors may cause testcase failure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	function f_EUTRA38_TAUReqFromN1	3

# Corrections required

## function f_EUTRA38_TAUReqFromN1

| Function name | f_EUTRA38_TAUReqFromN1 |
| --- | --- |
| Reason for change | In the current implementation of the function f_EUTRA38_TAUReqFromN1 variable v_ExistingPDNs is declared but not initialized. This can cause run-time errors later when f_EUTRA38_TAUReqFromN1f_EUTRA38_MobileInfo_SetConfiguredPDNInfoList(v_ExistingPDNs); is called. This needs to be addressed |
| Summary of change | Instead of initialising var ConfiguredPDNInfoList_Type v_ExistingPDNs; Initialised var ConfiguredPDNInfoList_Type v_ExistingPDNs :={}; |
| TTCN module | NR5GC_IRAT/Common/ EUTRA38_NR5GCProcedures.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_EUTRA38_TAUReqFromN1(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, template MobileIdentity p_MobileId := omit, N1ToS1_SecurityContext_Type p_CxtType := MAPPED_CXT, template EUTRA_CellId_Type p_CellIdPreviouslyCampedOn := eutra_Cell_NonSpecific, template (present) EstablishmentCause p_EstablishmentCause := mo_Signalling, boolean p_FirstN1_S1Transition := true) runs on EUTRA_Multi5GS_PTC return NAS_MSG_Indication_Type { var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL); var NAS_MSG_Indication_Type v_NasInd; var EUTRA_SecurityParams_Type v_SecurityParams; var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var template TrackingAreaId v_TAI:= omit; var EUTRA_CellId_Type v_OldCellId; var GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_Tac; var template UERadioCapInfoUpdateNeeded v_UeRadioCap:= cr_UeRadioCap; var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(NR); var ConfiguredPDNInfoList_Type v_ExistingPDNs; var integer v_NoOfDRBs := f_EUTRA38_MobileInfo_GetNoOfDRBs(); var EUTRA38_DRBInfoList_Type v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); // @sic R5s201363 sic@ var template (value) EUTRA38_DRBInfoList_Type v_NewDRBInfoList := {}; var integer v_NewDRBId; <<SKIPPED CODE>>

After Change:

function f_EUTRA38_TAUReqFromN1(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, template MobileIdentity p_MobileId := omit, N1ToS1_SecurityContext_Type p_CxtType := MAPPED_CXT, template EUTRA_CellId_Type p_CellIdPreviouslyCampedOn := eutra_Cell_NonSpecific, template (present) EstablishmentCause p_EstablishmentCause := mo_Signalling, boolean p_FirstN1_S1Transition := true) runs on EUTRA_Multi5GS_PTC return NAS_MSG_Indication_Type { var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL); var NAS_MSG_Indication_Type v_NasInd; var EUTRA_SecurityParams_Type v_SecurityParams; var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var template TrackingAreaId v_TAI:= omit; var EUTRA_CellId_Type v_OldCellId; var GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_Tac; var template UERadioCapInfoUpdateNeeded v_UeRadioCap:= cr_UeRadioCap; var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(NR); var ConfiguredPDNInfoList_Type v_ExistingPDNs :={};//WA#IRAT var integer v_NoOfDRBs := f_EUTRA38_MobileInfo_GetNoOfDRBs(); var EUTRA38_DRBInfoList_Type v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); // @sic R5s201363 sic@ var template (value) EUTRA38_DRBInfoList_Type v_NewDRBInfoList := {}; var integer v_NewDRBId; <<SKIPPED CODE>>