3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210104

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1532 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_GetHighestBandwidthFrequency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the function f_NR_GetHighestBandwidthFrequency for the band n41 is used bandwidth 100MH with SCS 15KHz whichis not correct. Based on 38.101 (Table 5.3.2-1: Maximum transmission bandwidth configuration NRB) The bandwidth 100MHz is not allowed for SCS 15kHz |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_NR_GetHighestBandwidthFrequency n41 change SCS from 15 KHz to 30KHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.1.x.ENDC, 7.1.1.4.2.x.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_NR_GetHighestBandwidthFrequency |
| --- | --- |
| Reason for change | In the function f_NR_GetHighestBandwidthFrequency for the band n41 is used bandwidth 100MH with SCS 15KHz whichis not correct. Based on 38.101 (Table 5.3.2-1: Maximum transmission bandwidth configuration NRB) The bandwidth 100MHz is not allowed for SCS 15kHz |
| Summary of change | For n41 change SCS from 15 KHz to 30KHz |
| TTCN module | NR_FrequencyHighestBandwidth.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_GetHighestBandwidthFrequency ( FreqBandIndicatorNR p_FreqBandIndicatorNR ) return template(value) NR_Frequencyf1tof4_Type { var template(value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy; var P_Max v_P_Max := f_NR_InitP_Max(f_NR_IsBandFR1(p_FreqBandIndicatorNR)); select ( p_FreqBandIndicatorNR ) { … case( 41 ) { //@sic R5-201022 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 500190 , 41 , 499374 , 0 , kHz30 , 273 ), 100 , kHz15 , 8 , 1 , 2 , 0 , 1 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 273 , v_P_Max), 100 ); }

After Change:

function f_NR_GetHighestBandwidthFrequency ( FreqBandIndicatorNR p_FreqBandIndicatorNR ) return template(value) NR_Frequencyf1tof4_Type { var template(value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy; var P_Max v_P_Max := f_NR_InitP_Max(f_NR_IsBandFR1(p_FreqBandIndicatorNR)); select ( p_FreqBandIndicatorNR ) { … case( 41 ) { //@sic R5-201022 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 500190 , 41 , 499374 , 0 , kHz30 , 273 ), 100 , kHz30 , 8 , 1 , 2 , 0 , 1 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 273 , v_P_Max), 100 ); }