3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210147

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1557 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 7.1.1.2.1 and 7.1.1.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RACH Msg4 UL grant size for Steps 19a1-19a2 of test case 7.1.1.2.1.NR5GC and Steps 20-21 of test case 7.1.1.3.1.NR5GC should be 144 bits. However, current TTCN uses cs_NR_RachProcedureConfig_CBRA_CrntiBased template which uses the minimum grant size. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use cs_NR_RachProcedureConfig_CRNTI_HO instead of cs_NR_RachProcedureConfig_CBRA_CrntiBased. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.1.NR5GC, 7.1.1.3.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_7_1_1_2_1_NR5GC |
| --- | --- |
| Reason for change | RACH Msg4 UL grant size for Steps 19a1-19a2 should be 144 bits. However, current TTCN uses cs_NR_RachProcedureConfig_CBRA_CrntiBased template which uses the minimum grant size. |
| Summary of change | Use cs_NR_RachProcedureConfig_CRNTI_HO instead of cs_NR_RachProcedureConfig_CBRA_CrntiBased. |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | Accepted |

Before change:

function f_TC_7_1_1_2_1_NR5GC() runs on NR5GC_PTC

{ // Correct Handling of DL MAC PDU / Assignment / HARQ process

var DRB_Identity v_DRBId;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList;

//@sic R5-192289 sic@

var template (value) CellGroupConfig v_CellGroupConfig;

var template(value) SpCellConfig v_SpCellConfig ;

var NR_DuplexMode_Type v_NR_DuplexMode ;

var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon ;

var SubcarrierSpacing v_SubcarrierSpacing;

var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext;

var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List;  //@sic R5s190360 sic@

var SubFrameTiming_Type v_TimingInfo; //@sic R5s200296 R5s200554 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters;

var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure;

//Init Cell parameters

f_NR5GC_Init(NR_1);

//@sic R5s190360 sic@

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

// Get NR capabilities sent during EUTRA preamble

f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON);

//Set null cipher Algorithm

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Retrieve DRB Id of the first PDU session

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId)};

//Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeA_ON,

cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, v_DRBId),

-,  //p_IpPduDelayTime

-,

-,  //MAC_CellGroupConfig

-,

v_SS_Drb_ConfigList, //@sic R5-203533 sic@

n16

);

f_NR_TestBody_Set(true);

f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop); //@sic R5s190886 sic@

f_TC_7_1_1_2_1_NR_TestBody1(v_DRBId);

//@sic R5s200554 sic@

v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1);

v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters);

// configure CBRA CRNTI Based RACH procedure

f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure);

//@sic R5-192289 sic@

v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1);

if (ischosen(v_NR_DuplexMode.TDD)) {

…

After change:

function f_TC_7_1_1_2_1_NR5GC() runs on NR5GC_PTC

{ // Correct Handling of DL MAC PDU / Assignment / HARQ process

var DRB_Identity v_DRBId;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList;

//@sic R5-192289 sic@

var template (value) CellGroupConfig v_CellGroupConfig;

var template(value) SpCellConfig v_SpCellConfig ;

var NR_DuplexMode_Type v_NR_DuplexMode ;

var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon ;

var SubcarrierSpacing v_SubcarrierSpacing;

var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext;

var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List;  //@sic R5s190360 sic@

var SubFrameTiming_Type v_TimingInfo; //@sic R5s200296 R5s200554 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters;

var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure;

//Init Cell parameters

f_NR5GC_Init(NR_1);

//@sic R5s190360 sic@

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

// Get NR capabilities sent during EUTRA preamble

f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON);

//Set null cipher Algorithm

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Retrieve DRB Id of the first PDU session

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId)};

//Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeA_ON,

cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, v_DRBId),

-,  //p_IpPduDelayTime

-,

-,  //MAC_CellGroupConfig

-,

v_SS_Drb_ConfigList, //@sic R5-203533 sic@

n16

);

f_NR_TestBody_Set(true);

f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop); //@sic R5s190886 sic@

f_TC_7_1_1_2_1_NR_TestBody1(v_DRBId);

//@sic R5s200554 sic@

v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1);

v_NR_RachProcedure := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters);

// configure CBRA CRNTI Based RACH procedure

f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure);

//@sic R5-192289 sic@

v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1);

if (ischosen(v_NR_DuplexMode.TDD)) {

…

## 2.2 Change 2

| Function name | f_TC_7_1_1_3_1_NR5GC |
| --- | --- |
| Reason for change | RACH Msg4 UL grant size for Steps 20-21 should be 144 bits. However, current TTCN uses cs_NR_RachProcedureConfig_CBRA_CrntiBased template which uses the minimum grant size. |
| Summary of change | Use cs_NR_RachProcedureConfig_CRNTI_HO instead of cs_NR_RachProcedureConfig_CBRA_CrntiBased. |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | Accepted |

Before change:

function f_TC_7_1_1_3_1_NR5GC() runs on NR5GC_PTC

{ // Correct Handling of UL MAC PDU / Assignment / HARQ process

var DRB_Identity v_DRBId ;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList;

//@sic R5-192289 sic@

var template (value) CellGroupConfig v_CellGroupConfig;

var template(value) SpCellConfig v_SpCellConfig ;

var NR_DuplexMode_Type v_NR_DuplexMode ;

var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon ;

var SubcarrierSpacing v_SubcarrierSpacing;

var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext;

var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List;  //@sic R5s190362 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters; //@sic R5s200555 sic@

var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure;

var SubFrameTiming_Type v_TimingInfo;

//Init Cell parameters

f_NR5GC_Init(NR_1);

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

// Get NR capabilities sent during EUTRA preamble

f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON);

//Set null cipher Algorithm

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Retrieve DRB Id of the first PDU session

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)};

//Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeA_ON,

cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

-,  //p_IpPduDelayTime

-,

-,  //MAC_CellGroupConfig

-,

v_SS_Drb_ConfigList

);

v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList);

f_NR_TestBody_Set(true);

f_TC_7_1_1_3_1_NR_TestBody1(v_DRBId);

//@sic R5s200555 sic@

v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1);

v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters);

// configure CBRA CRNTI Based RACH procedure

f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure);

//@sic R5-192289  sic@

v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1);

if (ischosen(v_NR_DuplexMode.TDD)) {

…

After change:

function f_TC_7_1_1_3_1_NR5GC() runs on NR5GC_PTC

{ // Correct Handling of UL MAC PDU / Assignment / HARQ process

var DRB_Identity v_DRBId ;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList;

//@sic R5-192289 sic@

var template (value) CellGroupConfig v_CellGroupConfig;

var template(value) SpCellConfig v_SpCellConfig ;

var NR_DuplexMode_Type v_NR_DuplexMode ;

var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon ;

var SubcarrierSpacing v_SubcarrierSpacing;

var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext;

var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List;  //@sic R5s190362 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters; //@sic R5s200555 sic@

var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure;

var SubFrameTiming_Type v_TimingInfo;

//Init Cell parameters

f_NR5GC_Init(NR_1);

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

// Get NR capabilities sent during EUTRA preamble

f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON);

//Set null cipher Algorithm

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Retrieve DRB Id of the first PDU session

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)};

//Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeA_ON,

cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

-,  //p_IpPduDelayTime

-,

-,  //MAC_CellGroupConfig

-,

v_SS_Drb_ConfigList

);

v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList);

f_NR_TestBody_Set(true);

f_TC_7_1_1_3_1_NR_TestBody1(v_DRBId);

//@sic R5s200555 sic@

v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1);

v_NR_RachProcedure := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters);

// configure CBRA CRNTI Based RACH procedure

f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure);

//@sic R5-192289  sic@

v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1);

if (ischosen(v_NR_DuplexMode.TDD)) {

…