3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210161

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1567 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction function f_EUTRA38_MultiPDN_GetAPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If ESMInformationFlag is True in PDNConnectivityRequest message then UE doesn’t include the AccessPointName value with Initial Attach Request message. Current implementation of function f_EUTRA38_MultiPDN_GetAPN cause a run-time error when UEs don’t include AccessPointName. For such situation, TTCN should use value from PICS pc_APN_ID_Internet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update TTCN to use AccessPointName value from PICS pc_APN_ID_Internet if UE does not provide the APN name (i.e. in case of Null APN) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NR5GC IRAT testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_EUTRA38_MultiPDN_GetAPN |
| --- | --- |
| Reason for change | If ESMInformationFlag is True in PDNConnectivityRequest message then UE doesn’t include the AccessPointName value with Initial Attach Request message. Current implementation of function f_EUTRA38_MultiPDN_GetAPN cause a run-time error when UEs don’t include AccessPointName. For such situation, TTCN should use value from PICS pc_APN_ID_Internet. |
| Summary of change | Update TTCN to use AccessPointName value from PICS pc_APN_ID_Internet if UE does not provide the APN name (i.e. in case of Null APN) |
| TTCN module | EUTRA38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment | Accepted in principle but implemented to echo latest changes to sister function f_NG_GetDNN |

Before Change:

function f_EUTRA38_MultiPDN_GetAPN(EUTRA_CellId_Type p_CellId, template (omit) AccessPointName p_APN) runs on EUTRA_PTC return template (value) AccessPointName { var octetstring v_APN_NetworkId := ''O; var PDU_PDN_DNN_Type v_PDNType := f_EUTRA38_MultiPDN_GetPDNType(p_APN); select (v_PDNType) { case(Internet_DNN) { // This must have been equal to the value sent, so just return this value v_APN_NetworkId := valueof(p_APN.nameValue); } case(IMS_DNN) { v_APN_NetworkId := f_EUTRA_GetAPNForIMS(p_CellId); } case else { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "APN PICS do not match received APN"); } } return cs_AccessPointName(v_APN_NetworkId); }

After Change:

function f_EUTRA38_MultiPDN_GetAPN(EUTRA_CellId_Type p_CellId, template (omit) AccessPointName p_APN) runs on EUTRA_PTC return template (value) AccessPointName { var octetstring v_APN_NetworkId := ''O; var PDU_PDN_DNN_Type v_PDNType := f_EUTRA38_MultiPDN_GetPDNType(p_APN); select (v_PDNType) { case(Internet_DNN) { // This must have been equal to the value sent, so just return this value if(isvalue(p_APN.nameValue)){ v_APN_NetworkId := valueof(p_APN.nameValue); }else{ //Anritsu CR to avoid Run time error in case of Null APN v_APN_NetworkId := f_DomainName_Encode(pc_APN_ID_Internet); } } case(IMS_DNN) { v_APN_NetworkId := f_EUTRA_GetAPNForIMS(p_CellId); } case else { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "APN PICS do not match received APN"); } } return cs_AccessPointName(v_APN_NetworkId); }

MCC160 Implementation

function f_EUTRA38_MultiPDN_GetAPN(EUTRA_CellId_Type p_CellId,

template (omit) AccessPointName p_APN) runs on EUTRA_PTC return template (value) AccessPointName

{

var octetstring v_APN_NetworkId;

var PDU_PDN_DNN_Type v_PDNType := f_EUTRA38_MultiPDN_GetPDNType(p_APN);

if (isvalue (p_APN)) {

v_APN_NetworkId := valueof(p_APN.nameValue);

} else {

select (v_PDNType) {

case(Internet_DNN) {

// This must have been equal to the value sent, so just return this value

v_APN_NetworkId := f_DomainName_Encode(pc_APN_ID_Internet);

}

case(IMS_DNN) {

v_APN_NetworkId := f_EUTRA_GetAPNForIMS(p_CellId);

}

}

}

}

return cs_AccessPointName(v_APN_NetworkId);

}