3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210189

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1578 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 6.1.2.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - USIM configuration makes the UE send a Service Request with ‘highPriority access’ service type. Change is required to pass Service Request check during preamble. - As described n change 1, add parameter for ServiceType - As described in change 2, add parameter for ServiceType - As described in change 3, Add parameter for ServiceType - Add parameter for ServiceType #ProseCR(R5-211173) raised in RAN5#90-e Meeting - As described in change 4, Add parameter for ServiceType |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Pass ‘0101’B as ServiceType for this testcase - Added parameter for Service Type with default value ‘0000’B - Added parameter for Service Type with default value ‘0000’B - Added parameter for Service Type with default value ‘0000’B - Add parameter for Service Type with default value ‘0000’B - Add parameter for Service Type with default value ‘0000’B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.14.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	6

2.3	Change 3	8

2.4	Change 4	10

2.5	Change 5	12

2.6	Change 6	17

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_6_1_2_14_NR5GC() |
| --- | --- |
| Reason for change | USIM configuration makes the UE send a Service Request with ‘highPriority access’ service type. Change is required to pass Service Request check during preamble. |
| Summary of change | Pass ‘0101’B as ServiceType for this testcase |
| TTCN module | Idle_CellReSelection_NR5GC |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_6_1_2_14_NR5GC() runs on NR5GC_PTC {// Cell reselection using cell status and cell reservations / Access Identity 11 or 15 - cellReservedForOperatorUse f_NR5GC_Init(NR_4); // System information combination NR-4 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR Cells //NR Cell 1 and NR Cell 3 are HPLMN f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_Def); f_NR_CellInfo_SetPLMN(nr_Cell3, tsc_NR_HPLMN_Def); f_NR_CellInfo_SetTAC (nr_Cell3, int2bit(2,24)); // NR Cell 1 and NR Cell 3 in different tracking areas f_NR_CellInfo_SetCellReservedForOperatorUse(nr_Cell3, -, reserved ); // Table 6.1.2.14.3.3-1: SIB1 for NR Cell 3 //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -80, -80); // Max Power level for FR2 is FFS // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); // The UE is equipped with a USIM containing default values and USIM Configuration 17 f_UT_USIM_Insert (UT, "38.508-1, USIM containing default values and USIM Configuration 17"); // @sic R5-205192 sic@ // Preamble: The UE is in state 1N-A on NR Cell 1 (serving cell) according to TS 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); f_TC_6_1_2_14_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell3, STATE_CONNECTED_3A_T3540); // @sic R5s201031 sic@ } //end of f_TC_6_1_2_14_NR5GC

After Change:

function f_TC_6_1_2_14_NR5GC() runs on NR5GC_PTC {// Cell reselection using cell status and cell reservations / Access Identity 11 or 15 - cellReservedForOperatorUse f_NR5GC_Init(NR_4); // System information combination NR-4 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR Cells //NR Cell 1 and NR Cell 3 are HPLMN f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_Def); f_NR_CellInfo_SetPLMN(nr_Cell3, tsc_NR_HPLMN_Def); f_NR_CellInfo_SetTAC (nr_Cell3, int2bit(2,24)); // NR Cell 1 and NR Cell 3 in different tracking areas f_NR_CellInfo_SetCellReservedForOperatorUse(nr_Cell3, -, reserved ); // Table 6.1.2.14.3.3-1: SIB1 for NR Cell 3 //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -80, -80); // Max Power level for FR2 is FFS // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); // The UE is equipped with a USIM containing default values and USIM Configuration 17 f_UT_USIM_Insert (UT, "38.508-1, USIM containing default values and USIM Configuration 17"); // @sic R5-205192 sic@ // Preamble: The UE is in state 1N-A on NR Cell 1 (serving cell) according to TS 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, -, -, -, -, '0101'B); f_NR_TestBody_Set(true); f_TC_6_1_2_14_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell3, STATE_CONNECTED_3A_T3540); // @sic R5s201031 sic@ } //end of f_TC_6_1_2_14_NR5GC

## Change 2

| Function name | f_NR5GC_Preamble() |
| --- | --- |
| Reason for change | As described n change 1, add parameter for ServiceType |
| Summary of change | Added parameter for Service Type with default value ‘0000’B |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_Preamble(NR_CellId_Type p_NR_CellId, NG_STATE_Type p_State, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, boolean p_ActivateSecurityInactiveState := true, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O ) runs on NR5GC_PTC { //@sic R5s190505: added p_ActivateSecurityInactiveState sic@ //@sic R5-199036: added p_IpPduDelayTime sic@ var RRCConnectionReleaseRequired_Type v_ReleaseConnection := p_ReleaseConnection; // @sic R5-197077 sic@ if (p_State == STATE_INACTIVE_2A) { v_ReleaseConnection := noRrcConnectionRelease; } // turn on NR cell f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); if (p_State != STATE_OFF_0A ) { // Turn on, Register and establish 1 PDU session // Switch on UE f_NR5GC_SwitchOnAndResetIMS(); // Go to RRC_IDLE f_NR5GC_RRC_IdleState1N_Def (p_NR_CellId, p_TestMode, v_ReleaseConnection, Initial_NoSecurity, p_IpPduDelayTime); } select (p_State) { case (STATE_OFF_0B) { // Now turn off again f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_IDLE_1A); } case (STATE_INACTIVE_2A) { f_NR_RRC_InactiveDef(p_NR_CellId, -, p_ActivateSecurityInactiveState); } case (STATE_CONNECTED_3A) { // Go to RRC_CONNECTED f_NR5GC_RRC_ConnectedState3N_Def (p_NR_CellId, p_TestMode); } } }

After Change:

function f_NR5GC_Preamble(NR_CellId_Type p_NR_CellId, NG_STATE_Type p_State, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, boolean p_ActivateSecurityInactiveState := true, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B ) runs on NR5GC_PTC { //@sic R5s190505: added p_ActivateSecurityInactiveState sic@ //@sic R5-199036: added p_IpPduDelayTime sic@ var RRCConnectionReleaseRequired_Type v_ReleaseConnection := p_ReleaseConnection; // @sic R5-197077 sic@ if (p_State == STATE_INACTIVE_2A) { v_ReleaseConnection := noRrcConnectionRelease; } // turn on NR cell f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); if (p_State != STATE_OFF_0A ) { // Turn on, Register and establish 1 PDU session // Switch on UE f_NR5GC_SwitchOnAndResetIMS(); // Go to RRC_IDLE f_NR5GC_RRC_IdleState1N_Def (p_NR_CellId, p_TestMode, v_ReleaseConnection, Initial_NoSecurity, p_IpPduDelayTime, p_ServiceType); } select (p_State) { case (STATE_OFF_0B) { // Now turn off again f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_IDLE_1A); } case (STATE_INACTIVE_2A) { f_NR_RRC_InactiveDef(p_NR_CellId, -, p_ActivateSecurityInactiveState); } case (STATE_CONNECTED_3A) { // Go to RRC_CONNECTED f_NR5GC_RRC_ConnectedState3N_Def (p_NR_CellId, p_TestMode); } } }

## Change 3

| Function name | f_NR5GC_RRC_IdleState1N_Def() |
| --- | --- |
| Reason for change | As described in change 2, add parameter for ServiceType |
| Summary of change | Added parameter for Service Type with default value ‘0000’B |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_RRC_IdleState1N_Def(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, TypeOfRegistration_Type p_RegType := Initial_Secure, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O) runs on NR5GC_PTC { //@sic R5-199036: added p_IpPduDelayTime sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; // Steps 2 - 4 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, p_RegType)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Steps 5 - 20 f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, p_TestMode, p_ReleaseConnection, p_RegType, -, p_IpPduDelayTime); }

After Change:

function f_NR5GC_RRC_IdleState1N_Def(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, TypeOfRegistration_Type p_RegType := Initial_Secure, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { //@sic R5-199036: added p_IpPduDelayTime sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; // Steps 2 - 4 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, p_RegType)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Steps 5 - 20 f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, p_TestMode, p_ReleaseConnection, p_RegType, -, p_IpPduDelayTime, -, p_ServiceType); }

## Change 4

| Function name | f_NR5GC_RRC_Idle_Steps5_20() |
| --- | --- |
| Reason for change | As described in change 3, Add parameter for ServiceType |
| Summary of change | Added parameter for Service Type with default value ‘0000’B |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_RRC_Idle_Steps5_20(GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, TypeOfRegistration_Type p_RegType := Initial_Secure, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC { var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); // Steps 5-15 f_NR5GC_RRC_Idle_Steps5_15(p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, v_PLMN, p_TestMode, p_RegType, -, -, p_IwkN26); // @sic R5-202564 sic@ //Steps 16 - 20 f_NR5GC_RRC_Idle_Steps16_20 (p_NR_CellId, -, p_TestMode, p_ReleaseConnection, p_UE_TestLoopModeA_NR_LB_Setup, p_IpPduDelayTime); // @sic R5s200751 sic@ }

After Change:

function f_NR5GC_RRC_Idle_Steps5_20(GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, TypeOfRegistration_Type p_RegType := Initial_Secure, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); // Steps 5-15 f_NR5GC_RRC_Idle_Steps5_15(p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, v_PLMN, p_TestMode, p_RegType, -, -, p_IwkN26); // @sic R5-202564 sic@ //Steps 16 - 20 f_NR5GC_RRC_Idle_Steps16_20 (p_NR_CellId, -, p_TestMode, p_ReleaseConnection, p_UE_TestLoopModeA_NR_LB_Setup, p_IpPduDelayTime,// @sic R5s200751 sic@ -, p_ServiceType); }

## Change 5

| Function name | f_NR5GC_RRC_Idle_Extension() |
| --- | --- |
| Reason for change | Add parameter for ServiceType #ProseCR(R5-211173) raised in RAN5#90-e Meeting |
| Summary of change | Add parameter for Service Type with default value ‘0000’B |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@ timer t_Wait := 10.0; // Step 0 @sic R5-204399 sic@ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; } [] t_Wait.timeout { // Step 1 if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@ /* Only trigger if there's no sessions established automatically to ensure there's (at least) one session established for the test body */ f_UT_RequestPDUSession(UT, 1, tsc_RandomAPN); } else { //Anritsu TTCN CR R5s210084 return; //if PDU connection exists, skip step 2-9a1 } } } // Steps 2 - 4 v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0000'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // Steps 5-6 // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req // FFS - Prose DOES NOT specify PDU Session Status to be reflected back from Req message // Step 7 SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); // @sic R5s201387 Baseline Moving sic@ // Step 8 f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsNewConnection, tsc_NR_RbId_SRB1); // @sic R5-206291 sic@ //Step 9a1 if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // Step 8Aa1 - 8Aa2 select (p_TestMode) { case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } }

After Change:

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@ timer t_Wait := 10.0; // Step 0 @sic R5-204399 sic@ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; } [] t_Wait.timeout { // Step 1 if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@ /* Only trigger if there's no sessions established automatically to ensure there's (at least) one session established for the test body */ f_UT_RequestPDUSession(UT, 1, tsc_RandomAPN); } else { //Anritsu TTCN CR R5s210084 return; //if PDU connection exists, skip step 2-9a1 } } } // Steps 2 - 4 v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, p_ServiceType)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // Steps 5-6 // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req // FFS - Prose DOES NOT specify PDU Session Status to be reflected back from Req message // Step 7 SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); // @sic R5s201387 Baseline Moving sic@ // Step 8 f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsNewConnection, tsc_NR_RbId_SRB1); // @sic R5-206291 sic@ //Step 9a1 if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // Step 8Aa1 - 8Aa2 select (p_TestMode) { case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } }

## Change 6

| Function name | f_NR5GC_RRC_Idle_Steps16_20() |
| --- | --- |
| Reason for change | As described in change 4, Add parameter for ServiceType |
| Summary of change | Added parameter for Service Type with default value ‘0000’B |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_RRC_Idle_Steps16_20(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := noRrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, boolean p_IdleExtensionProcedure := true) runs on NR5GC_PTC //Anritsu TTCN CR R5s210048 { //Steps 16 - 19a1 // No SRB2 so must be on SRB1 if (p_ExpectedNumberOfNewPDUSessions < 0 ) { // @sic R5-194797, R5-199075, R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection // @sic R5s201358 sic@ < 0"); } f_NR5GC_PDUSessionEstablishment (p_NR_CellId, p_ExpectedNumberOfNewPDUSessions, tsc_NR_RbId_SRB1); //Step 20a1 if (p_ReleaseConnection == rrcConnectionRelease or pc_noOf_PDUsNewConnection >0) {// @sic R5-199075, R5s200489 sic@ // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // @sic R5s200489 sic@ // Step 19Aa1 - 19Aa2 select (p_TestMode) { // @sic R5-194797 sic@ case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } if (p_IdleExtensionProcedure) { //Anritsu TTCN CR R5s210048 if (pc_noOf_PDUsNewConnection >0 ){ // @sic R5s200026 sic@ // @sic R5-199075 sic@ f_NR5GC_RRC_Idle_Extension(p_NR_CellId, p_TestMode, p_ReleaseConnection, p_UE_TestLoopModeA_NR_LB_Setup, p_IpPduDelayTime); } } }

After Change:

function f_NR5GC_RRC_Idle_Steps16_20(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := noRrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, boolean p_IdleExtensionProcedure := true, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC //Anritsu TTCN CR R5s210048 { //Steps 16 - 19a1 // No SRB2 so must be on SRB1 if (p_ExpectedNumberOfNewPDUSessions < 0 ) { // @sic R5-194797, R5-199075, R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection // @sic R5s201358 sic@ < 0"); } f_NR5GC_PDUSessionEstablishment (p_NR_CellId, p_ExpectedNumberOfNewPDUSessions, tsc_NR_RbId_SRB1); //Step 20a1 if (p_ReleaseConnection == rrcConnectionRelease or pc_noOf_PDUsNewConnection >0) {// @sic R5-199075, R5s200489 sic@ // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // @sic R5s200489 sic@ // Step 19Aa1 - 19Aa2 select (p_TestMode) { // @sic R5-194797 sic@ case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } if (p_IdleExtensionProcedure) { //Anritsu TTCN CR R5s210048 if (pc_noOf_PDUsNewConnection >0 ){ // @sic R5s200026 sic@ // @sic R5-199075 sic@ f_NR5GC_RRC_Idle_Extension(p_NR_CellId, p_TestMode, p_ReleaseConnection, p_UE_TestLoopModeA_NR_LB_Setup, p_IpPduDelayTime, p_ServiceType); } } }