3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210265

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1591 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In this function the second parameter is defined as "ENDC_Bearer_Type", this is defined as type ENDC_DRB_Configuration_Type ENDC_Bearer_Type (MCG_SCG, MCG_Split, MCG_Only); In some cases (8.2.2.4.1 for .e.g) the second parameter that is passed into this function is not limited to ENDC_Bearer_Type this results in Runtime exception OutOfValueConstraintException f_TC_8_2_2_4_1_ENDC_EUTRA-> fl_TC_8_2_2_4_1and5_ENDC_EUTRA_TestBody -> f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only) -> v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(p_CellId, p_DRB_Type, p_RRC_State); |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the second parameter to ENDC_DRB_Configuration_Type |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test execution will result in runtime error due to OutOfValueConstraintException |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.4.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections	3

1.1	Correction to f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated	3

# Corrections

## Correction to f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated

| Template name | f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated |
| --- | --- |
| Reason for change | In this function the second parameter is defined as "ENDC_Bearer_Type", this is defined as type ENDC_DRB_Configuration_Type ENDC_Bearer_Type (MCG_SCG, MCG_Split, MCG_Only); In some cases (8.2.2.4.1 for .e.g) the second parameter that is passed into this function is not limited to ENDC_Bearer_Type this results in Runtime exception OutOfValueConstraintException f_TC_8_2_2_4_1_ENDC_EUTRA-> fl_TC_8_2_2_4_1and5_ENDC_EUTRA_TestBody -> f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only) -> v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(p_CellId, p_DRB_Type, p_RRC_State); |
| Summary of change | Changed the second parameter to ENDC_DRB_Configuration_Type |
| TTCN module | EUTRA38_ENDC_CommonProcedures |
| MCC160 Comment | Alternative solution implemented: only call f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated when this is to be included in the Reconfiguration message |

Before Change:

function f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(EUTRA_CellId_Type p_CellId, ENDC_Bearer_Type p_DRB_Type, EUTRA_RRC_STATE_Type p_RRC_State := RRC_IDLE) runs on EUTRA_PTC return template (omit) RadioResourceConfigDedicated { var template (omit) RadioResourceConfigDedicated v_RRCMsg := omit; var template (value) DRB_ToAddModList v_DRBList; var template (omit) EPS_BearerIdentityList v_ListOfIds := f_EUTRA_MobileInfo_GetEPS_BearerList (); // @sic R5-199103 sic@ var EPS_BearerIdentityList v_BearerIds; var template (value) EUTRA_FDD_TDD_CellInfo_Type v_EUTRA_FDD_TDD_Info := f_EUTRA_CellInfo_GetFDD_TDD_Info (p_CellId); //Get FDD or TDD mode from Cell-configuration var template (omit) PhysicalConfigDedicated v_PhysicalConfigDedicated; var template (value) PhysicalConfigDedicated_AntennaInfo_Type v_AntennaInfo := f_EUTRA_CellInfo_GetAntennaInfoDedicated (p_CellId); var integer i; v_PhysicalConfigDedicated := cs_508_PhysicalConfigDedicated_Default_RBC(v_EUTRA_FDD_TDD_Info.cqi_ReportConfig, v_EUTRA_FDD_TDD_Info.soundingRS_UL_Config, v_AntennaInfo); if (isvalue(v_ListOfIds)) { v_BearerIds := valueof(v_ListOfIds); } else { FatalError(__FILE__, __LINE__, "Bearer Id not stored"); } for (i:= 0; i < lengthof(v_BearerIds); i := i+1) { v_DRBList[i] := cs_508_DRB_ToAddMod_DEFAULT_AM (f_EUTRA_NB_EpsBearerAssociatedDRB(v_BearerIds[i])); } select (p_RRC_State) { case (RRC_IDLE) { // Need to configure SRB2 + MAC + Phy too select (p_DRB_Type) { case (MCG_Split) { // add DRB2 to the end of the list v_DRBList[lengthof(v_DRBList)] := cs_508_DRB_ToAddMod_ENDC_Split; v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig({cs_508_SRB2_ToAddMod_DEFAULT}, v_DRBList, cs_508_MAC_MainConfig_Explicit_RBC_DrxL_DC, v_PhysicalConfigDedicated); //@sic R5-200989 sic@ } case(MCG_Only){ // Same configuration on LTE side - no dualConnectivityPHR configured in MAC-MainConfig @sic R5-200989 sic@ v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig({cs_508_SRB2_ToAddMod_DEFAULT}, v_DRBList, cs_508_MAC_MainConfig_Explicit_RBC_DrxL, v_PhysicalConfigDedicated); } case else { // Same configuration on LTE side - with dualConnectivityPHR configured in MAC-MainConfig @sic R5-200989 sic@ v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig({cs_508_SRB2_ToAddMod_DEFAULT}, v_DRBList, cs_508_MAC_MainConfig_Explicit_RBC_DrxL_DC, v_PhysicalConfigDedicated); } } } case else { // RRC_CONNECTED select (p_DRB_Type) { case (MCG_Split) { // only the new DRB to be included v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig(-, {cs_508_DRB_ToAddMod_ENDC_Split}, cs_508_MAC_MainConfig_Explicit_RBC_DrxL_DC); //@sic R5-200989 sic@ } } } } return v_RRCMsg; }

After Change:

function f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(EUTRA_CellId_Type p_CellId, ENDC_DRB_Configuration_Type p_DRB_Type, //WA# changed from ENDC_Bearer_Type p_DRB_Type, as the types passed are based on ENDC_DRB_Configuration_Type EUTRA_RRC_STATE_Type p_RRC_State := RRC_IDLE) runs on EUTRA_PTC return template (omit) RadioResourceConfigDedicated { var template (omit) RadioResourceConfigDedicated v_RRCMsg := omit; var template (value) DRB_ToAddModList v_DRBList; var template (omit) EPS_BearerIdentityList v_ListOfIds := f_EUTRA_MobileInfo_GetEPS_BearerList (); // @sic R5-199103 sic@ var EPS_BearerIdentityList v_BearerIds; var template (value) EUTRA_FDD_TDD_CellInfo_Type v_EUTRA_FDD_TDD_Info := f_EUTRA_CellInfo_GetFDD_TDD_Info (p_CellId); //Get FDD or TDD mode from Cell-configuration var template (omit) PhysicalConfigDedicated v_PhysicalConfigDedicated; var template (value) PhysicalConfigDedicated_AntennaInfo_Type v_AntennaInfo := f_EUTRA_CellInfo_GetAntennaInfoDedicated (p_CellId); var integer i; v_PhysicalConfigDedicated := cs_508_PhysicalConfigDedicated_Default_RBC(v_EUTRA_FDD_TDD_Info.cqi_ReportConfig, v_EUTRA_FDD_TDD_Info.soundingRS_UL_Config, v_AntennaInfo); if (isvalue(v_ListOfIds)) { v_BearerIds := valueof(v_ListOfIds); } else { FatalError(__FILE__, __LINE__, "Bearer Id not stored"); } for (i:= 0; i < lengthof(v_BearerIds); i := i+1) { v_DRBList[i] := cs_508_DRB_ToAddMod_DEFAULT_AM (f_EUTRA_NB_EpsBearerAssociatedDRB(v_BearerIds[i])); } select (p_RRC_State) { case (RRC_IDLE) { // Need to configure SRB2 + MAC + Phy too select (p_DRB_Type) { case (MCG_Split) { // add DRB2 to the end of the list v_DRBList[lengthof(v_DRBList)] := cs_508_DRB_ToAddMod_ENDC_Split; v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig({cs_508_SRB2_ToAddMod_DEFAULT}, v_DRBList, cs_508_MAC_MainConfig_Explicit_RBC_DrxL_DC, v_PhysicalConfigDedicated); //@sic R5-200989 sic@ } case(MCG_Only){ // Same configuration on LTE side - no dualConnectivityPHR configured in MAC-MainConfig @sic R5-200989 sic@ v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig({cs_508_SRB2_ToAddMod_DEFAULT}, v_DRBList, cs_508_MAC_MainConfig_Explicit_RBC_DrxL, v_PhysicalConfigDedicated); } case else { // Same configuration on LTE side - with dualConnectivityPHR configured in MAC-MainConfig @sic R5-200989 sic@ v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig({cs_508_SRB2_ToAddMod_DEFAULT}, v_DRBList, cs_508_MAC_MainConfig_Explicit_RBC_DrxL_DC, v_PhysicalConfigDedicated); } } } case else { // RRC_CONNECTED select (p_DRB_Type) { case (MCG_Split) { // only the new DRB to be included v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig(-, {cs_508_DRB_ToAddMod_ENDC_Split}, cs_508_MAC_MainConfig_Explicit_RBC_DrxL_DC); //@sic R5-200989 sic@ } } } } return v_RRCMsg; }

MCC160 Implementation:

function f_EUTRA38_508_ENDC_Reconfiguration(EUTRA_CellId_Type p_CellId,

ENDC_DRB_Configuration_Type p_DRB_Type,

template (omit) integer p_SK_Counter := omit,

EUTRA_RRC_STATE_Type p_RRC_State := RRC_CONNECTED,

template (omit) NAS_MSG_RequestList_Type p_NASMsg := omit,

template (omit) RadioResourceConfigDedicated p_RadioResourceConfigDedicated  := omit,

template (omit) RadioBearerList_Type p_DrbConfigAtSS := omit,

ENDC_SRB_DRB_Configuration_Type p_SecondaryConfig := NONE,

template (omit) MobilityControlInfo p_MobilityControlInformation := omit,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

template NAS_UL_Message_Type p_ULNASMsg := omit,

template (omit) MeasConfig p_MeasurementConfiguration := omit,

boolean p_NR_Release := false,

boolean p_ENDC_ReleaseAndAdd_r15 := false

) runs on EUTRA_5GS_PTC

{

var template (omit) octetstring v_NR_SCGConfig := omit;

var template (omit) octetstring v_NR_RBConfig := omit;

var template (omit) octetstring v_NR_RBConfig2 := omit;

var template (omit) RadioResourceConfigDedicated v_RadioResourceConfigDedicated := p_RadioResourceConfigDedicated;

var template (omit) RadioBearerList_Type v_DrbConfigAtSS := omit;

var template (omit) NAS_MSG_RequestList_Type v_ActivateBearer := omit;

var integer v_DRB_Id := f_EUTRA38_ENDC_GetDRBIdOfMCG();

if (isvalue(p_NASMsg)) {

v_ActivateBearer := p_NASMsg;

} else if (p_DRB_Type == MCG_SCG or p_DRB_Type == MCG_Split) { // no bearer is configured @sic R5s190168 sic@

// Dedicated bearer as per default messages

v_ActivateBearer := {

f_EPS_GetActivateDedicatedBearer(tsc_EpsDedicatedBearerId, cds_508_EPS_QoS_Dedicated_6_tlv, f_EUTRA_Get_508_TrafficFlowTemplate(6, tsc_EpsDedicatedBearerId, -, PDN_2)) // @sic R5s190188 sic@

};

}

select (p_SecondaryConfig) { // if changing MCG DRB need to tell NR the Id

case (NR_PDCP_MCG, NR_PDCP_MCG_SRB1_2) {

NR.send (cms_EUTRA_NR_OctetData (int2oct(v_DRB_Id, 1), "DRB ID"));

}

}

select (p_DRB_Type) {

case (MCG_SCG) {

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

// For MCG and SCG no change necessary to LTE SS, only SRB2, DRB1

v_DrbConfigAtSS := omit;

if (not isvalue(v_RadioResourceConfigDedicated)) {

v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(p_CellId, p_DRB_Type, p_RRC_State);

}

select (p_SecondaryConfig) { // Prose currently only specifies secondary config with MCG_SCG

case (NR_PDCP_MCG) {// LTE config to configure NR PDCP on EUTRA PTC

v_DrbConfigAtSS := {

cs_RadioBearer_Reconfig_SplitDRB(v_DRB_Id, nr_Cell1)

};

}

case (SRB3, NONE) {

// Nothing on LTE side

}

}

}

case (MCG_Split) {

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

// DRB1 already configured in preamble, only need to add DRB2 config

v_DrbConfigAtSS := {

cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1)

};

if (not isvalue(v_RadioResourceConfigDedicated)) {

v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(p_CellId, p_DRB_Type, p_RRC_State);

}

}

case (MCG_Only) {

if (not isvalue(v_RadioResourceConfigDedicated)) {

v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(p_CellId, p_DRB_Type, p_RRC_State);

}

}

case (RBConfig_Only) {

// Only wait for one lot of data

v_NR_SCGConfig := omit;

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

// expect both SS config and RadioResourceConfigDedicated to be passed in

v_DrbConfigAtSS := p_DrbConfigAtSS;

}

case (SCG_RBConfig) {

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

// Send the RRC Reconfiguration message and receive Complete

// expect both SS config and RadioResourceConfigDedicated to be passed in

v_DrbConfigAtSS := p_DrbConfigAtSS;

}

case (Meas_SCGOnly) {

// Only wait for one lot of data

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig := omit;

// expect both SS config and RadioResourceConfigDedicated to be passed in

v_DrbConfigAtSS := p_DrbConfigAtSS;

//Configure Measurement gap on E-UTRA SS side if it is available

if (isvalue(p_MeasurementConfiguration)){ //@sic R5-199097 sic@

if ( ischosen(p_MeasurementConfiguration.measGapConfig.setup)) {

f_EUTRA_SS_MeasGapCtrlConfig(p_CellId, cs_MeasGapCtrl_Config(valueof(p_MeasurementConfiguration.measGapConfig)), p_TimingInfo);

}

}

}

}

if (p_RRC_State == RRC_IDLE) {

// Page the UE and establish the RRC connection

// Send the RRC Reconfiguration message and receive Complete

f_EUTRA38_508_ENDC_RbEst(p_CellId,

v_NR_SCGConfig,

v_NR_RBConfig,

-,

v_ActivateBearer,

v_DrbConfigAtSS,

v_RadioResourceConfigDedicated,

-,

v_NR_RBConfig2,

p_ULNASMsg,

p_DRB_Type); // @sic R5s200647 sic@

} else { // RRC_CONNECTED

// Send the RRC Reconfiguration message and receive Complete

// Bearer already activated

f_EUTRA38_508_ENDC_SendReconfiguration(p_CellId,

v_DrbConfigAtSS,

v_RadioResourceConfigDedicated,

v_NR_SCGConfig,

v_NR_RBConfig,

v_ActivateBearer, // @sic R5s190093 sic@

p_SK_Counter,

-, -,

p_MeasurementConfiguration,

p_MobilityControlInformation,

p_ULNASMsg, // @sic R5s190093 sic@

p_TimingInfo,

-, -, -,

p_NR_Release,

p_ENDC_ReleaseAndAdd_r15);  // @sic R5s200149 sic@

}

}