3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210277

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1597 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_ReconfigureDRBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of the function f_ReconfigureDRBs(), PDCP security is configured by calling functions f_NR_RRC_CipherActTime_GetInitial() and f_NR_SS_RRC_EnableIntProt_CiphULandDL. This change was introduced as part of the implementation of R5s190382 Change 4.2 of R5s190382 was intended to configure DRB Ciphering . However, the implementation is making use of the function f_NR_RRC_CipherActTime_GetInitial()f_NR_RRC_CipherActTime_GetCurrent(p_NR_CellId, true); Second parameter in function f_NR_RRC_CipherActTime_GetCurrent is set as true which is for p_IncrSRB1. Along with the configuration of DRB , this function also modifies the next Ciphering Activation SN for SRB1 which is not the purpose of this change. This needs to be corrected/improved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to configure the DRB security inside the function f_ReconfigureDRBs(), it is proposed to use a new function f_NR_RRC_CipherActTime_GetCurrent_DRB() instead of f_NR_RRC_CipherActTime_GetInitial() . This new function only configures the DRB security without affecting the earlier SRB configuration. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All the NR5GC and NR5GC IRAT test cases where DRB reconfiguration is involved |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Correction to function f_ReconfigureDRBs ()	4

1.2	Addition of function  f_NR_RRC_CipherActTime_GetCurrent_DRB ()	6

# Corrections required

## Correction to function f_ReconfigureDRBs ()

| Function name | function f_ReconfigureDRBs |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function f_ReconfigureDRBs(), Configure PDCP security is done using the functions f_NR_RRC_CipherActTime_GetInitial() and f_NR_SS_RRC_EnableIntProt_CiphULandDL. This change is introduced as part of the implementation of R5s190382 Change 4.2 of R5s190382 is intended to configure DRB Ciphering .But the implementation is making use of the function f_NR_RRC_CipherActTime_GetInitial() which inturn calls f_NR_RRC_CipherActTime_GetCurrent(p_NR_CellId, true); second parameter is passed as true which is for p_IncrSRB1. Along with the configuration of DRB , this function also modifies the next Ciphering Activation SN for SRB1 which is not the purpose of this change. This needs to be corrected/improved. |
| Summary of change | In order to configure the DRB security inside the function f_ReconfigureDRBs(), it is proposed to use a new function f_NR_RRC_CipherActTime_GetCurrent_DRB() instead of f_NR_RRC_CipherActTime_GetInitial() . This new function only configures the DRB security without affecting the earlier SRB configuration. Please see below. |
| TTCN module | Common\NR5GC\NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted in principle but used existing function f_NR_RRC_CipherActTime_GetCurrent with set to false as this is sometimes called to enable ciphering on SRB2 as well as DRBs. Also corrected f_ENDC_GetKeysActivateSecurity, f_NRL2_ReconfigExistingAndAddNewDRBs, f_ENDC_GetKeysActivateSecurity_KeNB and f_NR_ENDC_PreambleOnEUTRA Additional change: add v_SecurityParams as a parameter so it can be passed in in 8.1.2.1.1 |

Before Change:

function f_ReconfigureDRBs(NR_CellId_Type p_NR_CellId, template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg, template (value) RadioBearerConfig p_RadioBearerConfig, template (value) CellGroupConfig p_CellGroupConfig, NR_RadioBearerList_Type p_SS_Config, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, boolean p_ReceiveForRRCreconfigurationComplete := true, template (omit) NR_RoutingInfoSUL_Type p_NR_RoutingInfoSUL := omit, template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH p_NrofHARQ_ProcessesForPDSCH := omit ) runs on NR_BASE_PTC { // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@ // @sic R5-203533: added p_NrofHARQ_ProcessesForPDSCH sic@ var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); if (isvalue(p_NrofHARQ_ProcessesForPDSCH)) { p_CellGroupConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig := cs_NR_Pdsch_ServingCellConfigSetup(cs_NR_PDSCH_ServingCellConfigDef(-, -, p_NrofHARQ_ProcessesForPDSCH)); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_PDSCH_Config_REQ(p_NR_CellId, p_TimingInfo, cs_NR_PDSCH_CellLevelConfigDef(p_CellGroupConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig.setup))); } v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(p_CellGroupConfig))); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, p_SS_Config, p_TimingInfo); // Configure PDCP security v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(p_NR_CellId); // @sic R5s190382 sic@ f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_NR_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // Send message to UE f_NR_SendRRCReconfiguration (p_NR_CellId, p_SRB, p_RadioBearerConfig, v_V1530Ext, p_NASMsg, -, p_TimingInfo, p_ReceiveForRRCreconfigurationComplete, p_NR_RoutingInfoSUL); }

After Change:

function f_ReconfigureDRBs(NR_CellId_Type p_NR_CellId, template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg, template (value) RadioBearerConfig p_RadioBearerConfig, template (value) CellGroupConfig p_CellGroupConfig, NR_RadioBearerList_Type p_SS_Config, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, boolean p_ReceiveForRRCreconfigurationComplete := true, template (omit) NR_RoutingInfoSUL_Type p_NR_RoutingInfoSUL := omit, template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH p_NrofHARQ_ProcessesForPDSCH := omit ) runs on NR_BASE_PTC { // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@ // @sic R5-203533: added p_NrofHARQ_ProcessesForPDSCH sic@ var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); if (isvalue(p_NrofHARQ_ProcessesForPDSCH)) { p_CellGroupConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig := cs_NR_Pdsch_ServingCellConfigSetup(cs_NR_PDSCH_ServingCellConfigDef(-, -, p_NrofHARQ_ProcessesForPDSCH)); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_PDSCH_Config_REQ(p_NR_CellId, p_TimingInfo, cs_NR_PDSCH_CellLevelConfigDef(p_CellGroupConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig.setup))); } v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(p_CellGroupConfig))); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, p_SS_Config, p_TimingInfo); // Configure PDCP security /*v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(p_NR_CellId); // @sic R5s190382 sic@ f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_NR_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering);*/ // Read PDCP SQN for DRB and calculate Ciphering Activation times v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent_DRB(p_NR_CellId); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_NR_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // Send message to UE f_NR_SendRRCReconfiguration (p_NR_CellId, p_SRB, p_RadioBearerConfig, v_V1530Ext, p_NASMsg, -, p_TimingInfo, p_ReceiveForRRCreconfigurationComplete, p_NR_RoutingInfoSUL); }

MCC160 Implementation:

function f_ReconfigureDRBs(NR_CellId_Type p_NR_CellId,

template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg,

template (value) RadioBearerConfig p_RadioBearerConfig,

template (value) CellGroupConfig p_CellGroupConfig,

template (omit) NR_RadioBearerList_Type p_SS_Config,

NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

boolean p_ReceiveForRRCreconfigurationComplete := true,

template (omit) NR_RoutingInfoSUL_Type p_NR_RoutingInfoSUL := omit,

template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH  p_NrofHARQ_ProcessesForPDSCH := omit,

template (omit) NR_SecurityParams_Type p_SecurityParams := omit

) runs on NR_BASE_PTC

{ // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@

// @sic R5-203533: added p_NrofHARQ_ProcessesForPDSCH sic@

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext;

var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get();

if (isvalue(p_NrofHARQ_ProcessesForPDSCH)) {

p_CellGroupConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig := cs_NR_Pdsch_ServingCellConfigSetup(cs_NR_PDSCH_ServingCellConfigDef(-, -, p_NrofHARQ_ProcessesForPDSCH));

f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_PDSCH_Config_REQ(p_NR_CellId, p_TimingInfo, cs_NR_PDSCH_CellLevelConfigDef(p_CellGroupConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig.setup)));

}

v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(p_CellGroupConfig)));

//Reconfigure DRB in SS - existing DRBs are not reconfigured during release

f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, p_SS_Config, p_TimingInfo);

// Configure PDCP security

if (isvalue(p_SecurityParams)){

v_SecurityParams := valueof(p_SecurityParams);

} else {

v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_NR_CellId, false);  // @sic R5s190382 sic@

}

f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_NR_CellId,

v_SecurityParams.AS_Integrity,

v_SecurityParams.AS_Ciphering);

// Send message to UE

f_NR_SendRRCReconfiguration (p_NR_CellId, p_SRB, p_RadioBearerConfig, v_V1530Ext, p_NASMsg, -, p_TimingInfo, p_ReceiveForRRCreconfigurationComplete, p_NR_RoutingInfoSUL);

}

function f_NR_RRCReconfigExistingDRBs(NR_CellId_Type p_NR_CellId,

template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg,

template (omit) NR_RLC_BearerToAddModList_Type p_RLCConfigList := omit,

template (omit) MAC_CellGroupConfig p_MACConfig := omit,

template (omit) DRB_ToAddModList p_DRBToAddMod := omit,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

template (omit) NR_RadioBearerList_Type p_SS_Drb_Config := omit,

boolean p_ReceiveForRRCreconfigurationComplete := true,

SRBs_AlreadyConfigured p_SRBs := SRB1,

template (omit) NR_RoutingInfoSUL_Type p_NR_RoutingInfoSUL := omit,

template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH  p_NrofHARQ_ProcessesForPDSCH := omit,

template (omit) NR_SCellToAddModList_Type p_SCellConfig := omit,

template (omit) NR_SecurityParams_Type p_SecurityParams := omit

) runs on NR5GC_PTC

{ // Split function in 2 so can re-use for L2 reconfig without NAS message @sic R5-196745 sic@

// Split again so list of DRBs can be built elsewhere @sic R5-199103 sic@

// @sic R5-203533: added p_NrofHARQ_ProcessesForPDSCH sic@

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var NR_RadioBearerList_Type v_SS_Config;

fl_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, p_NR_CellId, p_RLCConfigList, p_MACConfig, p_DRBToAddMod, p_TimingInfo, p_SS_Drb_Config, p_SRBs, p_SCellConfig);

f_ReconfigureDRBs (p_NR_CellId, p_NASMsg, v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, -, p_TimingInfo, p_ReceiveForRRCreconfigurationComplete, p_NR_RoutingInfoSUL, p_NrofHARQ_ProcessesForPDSCH, p_SecurityParams);

}

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC

{ // @sic R5-197057 R5s200044 sic@

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get();

var NR_SRB_COMMON_IND v_ReceivedAsp;

var integer v_NoOfDRBs := f_NR5GC_MobileInfo_GetNoOfDRBs();

var template (omit) NR_DRBInfo_Type v_DRBInfo;

var integer v_DRBId;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext;

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

// @siclog "Step 1" siclog@

// SS transmits a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList.

f_NR_Paging_Def(nr_Cell1, RRC_IDLE);

// @siclog "Step 2-4" siclog@

// UE transmits an RRCSetupRequest message

// SS transmit an RRCSetup message

// The UE transmits an RRCSetupComplete to confirm the successful completion of the connection establishment.

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?);

if (v_NoOfDRBs == 0) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DRBs configured");

}

//Get DRB Info for DRB associated with 1st Internet PDU session

v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); // @sic R5-200256 sic@

v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId);

if (not (isvalue(v_DRBInfo))) { // @sic R5s200571 Ch. 4 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Non-IMS DRBs configured");

}

// @siclog "Step 5" siclog@

// SS transmits a SecurityModeCommand message to activate AS security.

v_SecurityParams := f_NR_Authentication_InitAS (v_SecurityParams, v_ReceivedMsg.SecurityProtection.NasCount);

f_NR_Security_Set(v_SecurityParams);

// Read PDCP SQN for SRB1 and calculate Ciphering Activation times

v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(nr_Cell1);

f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1,

v_SecurityParams.AS_Integrity,

v_SecurityParams.AS_Ciphering);

// send SecurityModeCommand on SRB1. Follow on flag is set to TRUE to ensure the RRCConnectionReconfiguration message is sent on the same TTI and therefore

SRB.send(cas_NR_SRB1_RrcPdu_REQ_FollowOnFlag(nr_Cell1,

-, // cs_TimingInfo_Now is needed for FollowOn = true to work

cs_38508_RRCSecurityModeCommand(tsc_NR_RRC_TI_Def,

cs_38508_SecurityAlgorithmConfig(v_SecurityParams.AS_Ciphering.Algorithm,

v_SecurityParams.AS_Integrity.Algorithm))));

// Sent before it sends SecurityModeComplete message.

// @siclog "Step 6" siclog@

// Using the same slot as the SecurityModeCommand message in step 5, the SS transmits an RRCReconfiguration message to establish a data radio bearer, DRB1

f_NR_RRCReconfigExistingDRBs(nr_Cell1,

{cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg())},  // @sic R5s190388, R5s201387 Baseline Moving sic@

-, -, -,

cs_TimingInfo_Now,

-,

false, // Don't handle complete here

-, -, -, -,// SRB1 only

v_SecurityParams);

. . .

## Addition of function  f_NR_RRC_CipherActTime_GetCurrent_DRB ()

| Function name | function f_NR_RRC_CipherActTime_GetCurrent_DRB |
| --- | --- |
| Reason for change | Please see change 1.1 above |
| Summary of change | New function |
| TTCN module | Common\NR\NR_SecuritySteps.ttcn |
| MCC160 Comment | Not needed |

New Function Implementation:

/* * @desc Common functon to enquire PDCP sequence numbers and generate a ciphering activation list for DRBs which are configured * @param p_NR_CellId * @return NR_SecurityActTimeList_Type * @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ function f_NR_RRC_CipherActTime_GetCurrent_DRB(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return NR_SecurityActTimeList_Type { var NR_PdcpCountInfoList_Type v_PdcpCountInfoList; var NR_PdcpCountInfo_Type v_PdcpCountInfo; var NR_PDCP_ActTime_Type v_PDCP_ActTimeUL; var NR_PDCP_ActTime_Type v_PDCP_ActTimeDL; var NR_RadioBearerId_Type v_RadioBearerId; var NR_SecurityActTime_Type v_SecurityActTime; var integer I; var integer drb_Index := 0 ; var NR_SecurityActTimeList_Type v_SecurityActTimeList := {}; v_PdcpCountInfoList := f_NR_SS_PdcpCount_Get(p_NR_CellId); for (I:=0; I < lengthof(v_PdcpCountInfoList); I:=I+1) { v_PdcpCountInfo := v_PdcpCountInfoList[I]; if ((not ispresent(v_PdcpCountInfo.UL)) or (not ispresent(v_PdcpCountInfo.DL))) { FatalError(__FILE__, __LINE__, "missing PdcpCountInfo for UL or DL"); } v_RadioBearerId := v_PdcpCountInfo.RadioBearerId; // set activation time if(ischosen(v_RadioBearerId.Drb)) { // Convert PdcpCount to integer value (UL an DL) v_PDCP_ActTimeUL.SQN := f_NR_SQN_ConvertFromPdcpCount(v_PdcpCountInfo.UL); v_PDCP_ActTimeDL.SQN := f_NR_SQN_ConvertFromPdcpCount(v_PdcpCountInfo.DL); v_SecurityActTime.RadioBearerId := v_RadioBearerId; v_SecurityActTime.UL := v_PDCP_ActTimeUL; v_SecurityActTime.DL := v_PDCP_ActTimeDL; v_SecurityActTimeList[drb_Index] := v_SecurityActTime; drb_Index := drb_Index + 1 ; } } return v_SecurityActTimeList; }