3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210279

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4593 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA_Init for CAT-M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - When non-IMS suite CAT-M1 testcases are run with IMS enabled, UE fails to send REGISTER message on time due to insufficient UL grant type provided. Presently SS is configured to provide UL grant of type OnSR_Reception but according to clause D.16.2 from 36.523-3, cell must be configured to provide PeriodicOnSR_Reception - 36.523-3 D.16.2 specifies the following for PeriodicOnSR_Reception type of UL_GrantConfig_Type. “SS transmits UL Grant as configured by DciInfoUL_Type periodically; the periodic grant transmission is started/restarted on reception of a SR from UE to be used in non L2 Test to enable large UL data transmission for lower category UEs (Cat<=1)” New function is added to identify L2 testcases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Cell is configured to provide UL grant of type PeriodicOnSR_Reception instead of OnSR_Reception, by using the USE_BIG_GRANTS while cell initialization - New function f_GetTestcaseAttrib_L2_testcase added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTE CAT-M1 TCs cannot be tested on IMS enabled UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Many non-IMS suite CAT-M1 TCs |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_EUTRA_Init |
| --- | --- |
| Reason for change | When non-IMS suite CAT-M1 testcases are run with IMS enabled, UE fails to send REGISTER message on time due to insufficient UL grant type provided. Presently SS is configured to provide UL grant of type OnSR_Reception but according to clause D.16.2 from 36.523-3, cell must be configured to provide PeriodicOnSR_ Reception |
| Summary of change | Cell is configured to provide UL grant of type PeriodicOnSR_Reception instead of OnSR_Reception, by using the USE_BIG_GRANTS while cell initialization |
| TTCN module | EUTRA_CellInfoInit.ttcn |
| MCC160 Comment | Accepted in principle but implemented as per R5-213652 – see proposed implementation below |

Before Change:

function f_EUTRA_Init(Combination_Type p_SysinfoCombination, template (omit) EUTRA_HandleUlDataUM_Type p_HandleULDataUM := omit, template (omit) EUTRA_UseBigGrants_Type p_UseBigGrants := omit, template (omit) integer p_EUTRA_ENDC_PrimaryBand := omit, template (omit) integer p_EUTRA_ENDC_SecondaryBand := omit) runs on EUTRA_PTC { /* @sic R5s170597 - Cleanup of UL grant Configuration: p_DiscardULDataUM, p_UseBigGrants sic@ */ var InitParams_Local_Type v_Params := { NonNas := {SysinfoCombination := p_SysinfoCombination} }; fl_EUTRA_Common_Init(v_Params, p_HandleULDataUM, p_UseBigGrants, p_EUTRA_ENDC_PrimaryBand, p_EUTRA_ENDC_SecondaryBand); fl_EUTRA_IRAT_Init(p_SysinfoCombination); }

After Change:

function f_EUTRA_Init(Combination_Type p_SysinfoCombination, template (omit) EUTRA_HandleUlDataUM_Type p_HandleULDataUM := omit, template (omit) EUTRA_UseBigGrants_Type p_UseBigGrants := omit, template (omit) integer p_EUTRA_ENDC_PrimaryBand := omit, template (omit) integer p_EUTRA_ENDC_SecondaryBand := omit) runs on EUTRA_PTC { /* @sic R5s170597 - Cleanup of UL grant Configuration: p_DiscardULDataUM, p_UseBigGrants sic@ */ var InitParams_Local_Type v_Params := { NonNas := {SysinfoCombination := p_SysinfoCombination} }; if(pc_ue_CategoryDL_M1 and pc_IMS and not(f_GetTestcaseAttrib_L2_testcase(testcasename()))){ p_UseBigGrants := USE_BIG_GRANTS; // 36.523-3 D.16.2 } fl_EUTRA_Common_Init(v_Params, p_HandleULDataUM, p_UseBigGrants, p_EUTRA_ENDC_PrimaryBand, p_EUTRA_ENDC_SecondaryBand); fl_EUTRA_IRAT_Init(p_SysinfoCombination); }

MCC160 proposed implementation

function f_EUTRA_GrantType_Init(template (omit) EUTRA_UseBigGrants_Type p_UseBigGrants) return EUTRA_UseBigGrants_Type

{ /* NOTE: p_UseBigGrants shall be either omit or USE_BIG_GRANTS; other grant types are set based on UE category */

/* NOTE: as per 36.523-3 Table 7.2.0-1 and  Table 7.2.0-2 */

//@sic R5-213652 sic@

var EUTRA_UseBigGrants_Type v_GrantType;

var UE_CategoryInfo_Type v_UE_CategoryInfo := valueof(f_EUTRA_UE_CategoryInfo_Init());

var boolean v_IsCat0 := ispresent(v_UE_CategoryInfo.ue_Category_v1250);

var boolean v_IsCatM1:= ispresent(v_UE_CategoryInfo.ue_CategoryDL_v1310);

var boolean v_IsCat1 := (v_UE_CategoryInfo.ue_Category == 1);

var boolean v_IsCatGreater2 := not(v_IsCat0) and not (v_IsCat1) and not(v_IsCatM1);

var boolean v_EnhancedCoverage := f_GetTestcaseAttrib_EnhancedCoverage(testcasename());

var template (omit) EUTRA_UseBigGrants_Type v_UseBigGrants := p_UseBigGrants;

// Set UL Grant as per 36.523-3 Table 7.2.0-2: Test cases configured with big grant size

if ( (v_IsCat0 or v_IsCatM1) and pc_IMS ) {

//CatM1/0 - Condition to be fulfilled to use big grant in a test case: if the UE supports IMS

v_UseBigGrants := USE_BIG_GRANTS;

} else if ( v_IsCatGreater2 and v_EnhancedCoverage and pc_IMS ) {

// CatX with X≥1 - 	CE mode A test case:  yes - Condition to be fulfilled to use big grant in a test case: if the UE supports IMS

v_UseBigGrants := USE_BIG_GRANTS;

}

//as per 36.523-3 Table 7.2.0-1

if (not ispresent(v_UseBigGrants)) {         /* @sic R5s150338 sic@ */

//Grant size type: Default

if (v_IsCatM1) {                                // CatM1

v_GrantType := USE_CEorBL_GRANTS;

} else if (v_IsCat0) {                          // Cat0

v_GrantType := USE_CAT0_GRANTS;

} else if (v_IsCat1 and v_EnhancedCoverage) {   // Cat1/1bis and CE mode A test case

v_GrantType := USE_CEorBL_GRANTS;

} else if (v_IsCat1) {

v_GrantType := USE_DEF_GRANTS;

} else  if (v_EnhancedCoverage) {               // CatX with X≥2

v_GrantType := USE_CEorBL_GRANTS;

} else {

v_GrantType := USE_DEF_GRANTS;

}

} else {

//Grant size type: Big

if (v_IsCatM1) {                                // CatM1

v_GrantType := USE_CEorBL_BIG_GRANTS;

} else if (v_IsCat0) {                          // Cat0

v_GrantType := USE_CAT0_BIG_GRANTS;

} else if (v_IsCat1 and v_EnhancedCoverage) {   // Cat1/1bis and CE mode A test case

v_GrantType := USE_CEorBL_BIG_GRANTS;

} else if (v_IsCat1) {

v_GrantType := USE_CAT1_BIG_GRANTS;

} else  if (v_EnhancedCoverage) {               // CatX with X≥2

v_GrantType := USE_CEorBL_BIG_GRANTS;

} else {

v_GrantType := USE_BIG_GRANTS;              /* @sic R5s140676 Change 2 sic@ */

}

}

return v_GrantType;

}

## Change 2

| Function name | New function - f_GetTestcaseAttrib_L2_testcase |
| --- | --- |
| Reason for change | 36.523-3 D.16.2 specifies the following for PeriodicOnSR_Reception type of UL_GrantConfig_Type. “SS transmits UL Grant as configured by DciInfoUL_Type periodically; the periodic grant transmission is started/restarted on reception of a SR from UE to be used in non L2 Test to enable large UL data transmission for lower category UEs (Cat<=1)” This new function is added to identify L2 testcases. Note: This function is given as a reference and is only for LTE_A_PRO test suite. Similar functions should be added for all suites. |
| Summary of change | New function f_GetTestcaseAttrib_L2_testcase added |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment | Not needed – see change 1 |

function f_GetTestcaseAttrib_L2_testcase(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_7_1_2_1a") { return true; } case ("TC_7_1_2_3a") { return true; } case ("TC_7_1_2_3b") { return true; } case ("TC_7_1_3_4a") { return true; } case ("TC_7_1_3_5a") { return true; } case ("TC_7_1_4_1a") { return true; } case ("TC_7_1_4_2a") { return true; } case ("TC_7_1_4_2b") { return true; } case ("TC_7_1_4_11a") { return true; } case ("TC_7_1_4_12a") { return true; } case ("TC_7_1_4_24b") { return true; } case ("TC_7_1_4_24c") { return true; } case ("TC_7_1_4_31") { return true; } case ("TC_7_1_4_32_1") { return true; } case ("TC_7_1_4_32_2") { return true; } case ("TC_7_1_4_32_3") { return true; } case ("TC_7_1_4_32_4") { return true; } case ("TC_7_1_4a_1") { return true; } case ("TC_7_1_6_1a") { return true; } case ("TC_7_1_6_5") { return true; } case ("TC_7_1_7_1_13") { return true; } case ("TC_7_1_7_1_14") { return true; } case ("TC_7_1_7_2_2") { return true; } case ("TC_7_1_7_2_3") { return true; } case ("TC_7_1_7_2_4") { return true; } case ("TC_7_1_11_1") { return true; } case ("TC_7_1_12_1") { return true; } case ("TC_7_3_9_1") { return true; } case ("TC_7_3_10_1") { return true; } case ("TC_7_3_10_2") { return true; } case ("TC_7_3_10_3") { return true; } } return false; }