3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210283

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1599 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN-DC FR1 : Addition of EN-DC RRC test case 8.2.2.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add EN-DC RRC test case 8.2.2.2.1 to the 5G/ENDC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to EN-DC RRC test case 8.2.2.2.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	6

4	Execution Log Files	6

4.1	MediaTek MT6297	6

5	References	7

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.2.2.2.1 which is part of the 5G ENDC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	8.2.2.2.1

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	MediaTek MT6297

Verification Status:	PASS

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_8_2_2_2_1_ENDC_NR_TestBody |
| --- | --- |
| Reason for change | - Currently template template cs_NR_RlcBearerConfig_AM is being used for SRB1 and SRB2, this template will configure SN Field Length as 18 instead of 12. - NR PTC is waiting for "Complete" coord message from EUTRA PTC which is not being sent (step 6) |
| Summary of change | - Modified SS Configurationto use template cs_NR_RlcBearerConfig_SRB1SRB2SRB3 (see also change 2) - Pass p_WaitForRRCReconfigurationComplete as false in call to f_NR_SendRRCReconfigurationContentsToEUTRA at step 5, so that NR PTC will not wait for “Complete” |
| TTCN module | RRCReconfig_ENDC_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_8_2_2_2_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC

{ //@sic R5-201204 sic@

var template (value) CellGroupConfig v_SCGConfig;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) NR_RLC_BearerToReleaseList_Type v_Rlc_BearerToReleaseList := {1, 2};

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList;

var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1);

var template (value) NR_RadioBearerList_Type v_RadioBearerList;

var template (value) SRB_ToAddModList v_SRB_ToAddModList;

// @siclog "Step 1" siclog@

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertCiphAlg2Bitstring(f_NR_AS_CipheringAlgorithm_Get())), "NR Ciphering Alg"));

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertIntAlg2Bitstring(f_NR_AS_IntegrityAlgorithm_Get())), "NR Integrity Alg"));

//Set the NR ENDC Configuration for SRB1/2

v_SRB_ToAddModList := { //@sic R5-201203 sic@

cs_NR_SRB_ToAddMod(tsc_SRB1, cs_NR_PDCP_Config_SplitSRB(-, f_NR_LogicalChannelId_SRB(tsc_SRB1), 0, true)),

cs_NR_SRB_ToAddMod(tsc_SRB2, cs_NR_PDCP_Config_SplitSRB(-, f_NR_LogicalChannelId_SRB(tsc_SRB2), 0, true))

};

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(v_SRB_ToAddModList,

-, -,

cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master));

v_RLC_BearerToAddModList := {

cs_38508_RLC_BearerConfig_SRB(tsc_SRB1),

cs_38508_RLC_BearerConfig_SRB(tsc_SRB2)

};

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, v_RLC_BearerToAddModList);

//SS : configure SRB1 and SRB2 with RLCAckProhibit

//@82221

v_RadioBearerList := {

cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB1, cs_NR_PDCP_Configuration_Proxy(crs_RlcBearerRouting_Eutra(eutra_Cell1)), cs_NR_RlcBearerConfig_AM(v_IsFR1, cs_NR_RLC_TestModeConfig_AckProhibit(Prohibit), f_NR_LogicalChannelId_SRB(tsc_SRB1))),

cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB2, cs_NR_PDCP_Configuration_Proxy(crs_RlcBearerRouting_Eutra(eutra_Cell1)), cs_NR_RlcBearerConfig_AM(v_IsFR1, cs_NR_RLC_TestModeConfig_AckProhibit(Prohibit), f_NR_LogicalChannelId_SRB(tsc_SRB2)))

};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_RadioBearerList, cs_TimingInfo_Now); //Configure SRB1/2 in NR with PDCP set to proxy

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig, -, -, false);  // Now send octetstrings to EUTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

// @siclog "Step 5" siclog@

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, v_Rlc_BearerToReleaseList);

//Keep SRB1/2 configured on SS side to ensure Ue is not transmiiting the RRCReconfiguration msg on SCg side

f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig);  // Now send octetstrings to EUTRA //@82221

}

After Change

function fl_TC_8_2_2_2_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC

{ //@sic R5-201204 sic@

var template (value) CellGroupConfig v_SCGConfig;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) NR_RLC_BearerToReleaseList_Type v_Rlc_BearerToReleaseList := {1, 2};

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList;

var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1);

var template (value) NR_RadioBearerList_Type v_RadioBearerList;

var template (value) SRB_ToAddModList v_SRB_ToAddModList;

// @siclog "Step 1" siclog@

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertCiphAlg2Bitstring(f_NR_AS_CipheringAlgorithm_Get())), "NR Ciphering Alg"));

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertIntAlg2Bitstring(f_NR_AS_IntegrityAlgorithm_Get())), "NR Integrity Alg"));

//Set the NR ENDC Configuration for SRB1/2

v_SRB_ToAddModList := { //@sic R5-201203 sic@

cs_NR_SRB_ToAddMod(tsc_SRB1, cs_NR_PDCP_Config_SplitSRB(-, f_NR_LogicalChannelId_SRB(tsc_SRB1), 0, true)),

cs_NR_SRB_ToAddMod(tsc_SRB2, cs_NR_PDCP_Config_SplitSRB(-, f_NR_LogicalChannelId_SRB(tsc_SRB2), 0, true))

};

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(v_SRB_ToAddModList,

-, -,

cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master));

v_RLC_BearerToAddModList := {

cs_38508_RLC_BearerConfig_SRB(tsc_SRB1),

cs_38508_RLC_BearerConfig_SRB(tsc_SRB2)

};

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, v_RLC_BearerToAddModList);

//SS : configure SRB1 and SRB2 with RLCAckProhibit

//@82221

v_RadioBearerList := {

cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB1, cs_NR_PDCP_Configuration_Proxy(crs_RlcBearerRouting_Eutra(eutra_Cell1)), cs_NR_RlcBearerConfig_SRB1SRB2SRB3(tsc_NR_SRB1_LogicalChannelId, tsc_NR_SRB1_LogicalChannelPriority, cs_NR_RLC_TestModeConfig_AckProhibit(Prohibit))),

cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB2, cs_NR_PDCP_Configuration_Proxy(crs_RlcBearerRouting_Eutra(eutra_Cell1)), cs_NR_RlcBearerConfig_SRB1SRB2SRB3(tsc_NR_SRB2_LogicalChannelId, tsc_NR_SRB2_LogicalChannelPriority, cs_NR_RLC_TestModeConfig_AckProhibit(Prohibit)))

};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_RadioBearerList, cs_TimingInfo_Now); //Configure SRB1/2 in NR with PDCP set to proxy

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig, -, -, false);  // Now send octetstrings to EUTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

// @siclog "Step 5" siclog@

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, v_Rlc_BearerToReleaseList);

//Keep SRB1/2 configured on SS side to ensure Ue is not transmiiting the RRCReconfiguration msg on SCg side

f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig, -, -, false);  // Now send octetstrings to EUTRA //@82221

}

## 2.2 Change 2

| Function name | template cs_NR_RlcBearerConfig_SRB1SRB2SRB3 |
| --- | --- |
| Reason for change | Required to configure test mode AckProhibit for SRB1 and SRB2 |
| Summary of change | Modified cs_NR_RlcBearerConfig_SRB1SRB2SRB3 to accept new parameter p_TestModeConfig to configure RLC test mode. |
| TTCN module | RRCReconfig_ENDC_EUTRA.ttcn |
| MCC160 Comment | Accepted but parameter p_TestModeConfig is added with restriction template(omit) |

Before Change

template (value) NR_RlcBearerConfig_Type cs_NR_RlcBearerConfig_SRB1SRB2SRB3(NR_LogicalChannelId_Type p_LogicalChannelId,

integer p_LogicalChannelPriority) :=

/* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

cs_NR_RlcBearerConfig_Common(cs_NR_SS_RLC_Configuration_AM(cs_38508_UL_RLC_SRB, cs_38508_DL_RLC_SRB, cs_NR_RLC_TestModeConfig_None),

p_LogicalChannelId,

cs_NR_SS_MAC_Configuration(-, p_LogicalChannelPriority));

After Change

template (value) NR_RlcBearerConfig_Type cs_NR_RlcBearerConfig_SRB1SRB2SRB3(NR_LogicalChannelId_Type p_LogicalChannelId,

integer p_LogicalChannelPriority,

template NR_RLC_TestModeConfig_Type p_TestModeConfig := cs_NR_RLC_TestModeConfig_None) := //@82221

/* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

cs_NR_RlcBearerConfig_Common(cs_NR_SS_RLC_Configuration_AM(cs_38508_UL_RLC_SRB, cs_38508_DL_RLC_SRB, p_TestModeConfig),

p_LogicalChannelId,

cs_NR_SS_MAC_Configuration(-, p_LogicalChannelPriority));

# Branches executed

The test case was executed on NR band n78

# Execution Log Files

## MediaTek MT6297

The MediaTek MT6297 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_2_2_2_1_LOG.html

- PICS/PIXIT parameter file:

TC_8_2_2_2_1_PIXIT.html

# References

| [1] | R5s210284 : EN-DC FR1 : Supporting information for addition of EN-DC RRC test case 8.2.2.2.1 |
| --- | --- |