3GPP TSG-RAN5 Meeting #2020-TTCN email	R5s210341

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1622 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 11.1.7 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-15 NR5GC test case 11.1.7 to the NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 11.1.7 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

2.3	Change 3	10

2.4	Change 4	14

3	Branches executed	16

4	Execution Log Files	16

4.1	HiSilicon Balong 5000	16

5	References	16

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.1.7 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Pengchong Shen
	shenpengchong@datangmobile.cn

## Verification Test Summary

Test Case: 	11.1.7

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Datang LinkTester 5G Protocol Conformance Test System ECT9610

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_11_1_7_NR5GC |
| --- | --- |
| Reason for change | - NR side should to use f_NR5GC_IRAT_UECapabilityTransfer to complete LTE registration process at preamble. - It should to add cell power on and UE power on command before NR registration. |
| Summary of change | - Add f_NR5GC_IRAT_UECapabilityTransfer after NR cell setup. - Add f_NR_SetCellPower and f_NR5GC_SwitchOnAndResetIMS at NR preamble. |
| TTCN module | EPS_Fallback_NR5GC_NR.ttcn |
| MCC160 Comment | 1. Accepted 2. Accepted in principle but called f_NR5GC_Preamble_Steps1_13 instead |

Before Change

function f_TC_11_1_7_NR5GC() runs on NR5GC_PTC { //Emergency call setup from NR RRC_IDLE / Emergency services Fallback to EPS with redirection / Single registration mode with N26 interface / Success var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; var NG_NAS_GutiParameters_Type v_GutiParams; var template (value) NG_MobileIdentity v_GUTIToSend; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) NG_NetworkFeatureSupport v_NG_NetworkFeatureSupport := cs_NG_NetworkFeatureSupport_Def; f_NR5GC_IRAT_Init(NR_6); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // Wait until UE has attached on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); // Now register on NR // Steps 2 - 4 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, -, -, -, -, cdr_NG_GMM_Cap_S1)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); v_NG_NetworkFeatureSupport.emf := '01'B; // Steps 5 - 13 f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); // Step 14 // Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTIToSend, v_TAIList, omit, -, -, -, v_NG_NetworkFeatureSupport); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Steps 15 - 20 f_NR5GC_RRC_Idle_Steps15_20(nr_Cell1, TESTMode_OFF); // Now tell EUTRA preamble finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); f_NR_TestBody_Set(true); fl_TC_11_1_7_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_ReleaseAllCells(); }

After Change

function f_TC_11_1_7_NR5GC() runs on NR5GC_PTC { //Emergency call setup from NR RRC_IDLE / Emergency services Fallback to EPS with redirection / Single registration mode with N26 interface / Success var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; var NG_NAS_GutiParameters_Type v_GutiParams; var template (value) NG_MobileIdentity v_GUTIToSend; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) NG_NetworkFeatureSupport v_NG_NetworkFeatureSupport := cs_NG_NetworkFeatureSupport_Def; f_NR5GC_IRAT_Init(NR_6); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_IRAT_UECapabilityTransfer(nr_Cell1); // Wait until UE has attached on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_SwitchOnAndResetIMS(); // Now register on NR // Steps 2 - 4 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, -, -, -, -, cdr_NG_GMM_Cap_S1)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); v_NG_NetworkFeatureSupport.emf := '01'B; // Steps 5 - 13 f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); // Step 14 // Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTIToSend, v_TAIList, omit, -, -, -, v_NG_NetworkFeatureSupport); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Steps 15 - 20 f_NR5GC_RRC_Idle_Steps15_20(nr_Cell1, TESTMode_OFF); // Now tell EUTRA preamble finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); f_NR_TestBody_Set(true); fl_TC_11_1_7_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_ReleaseAllCells(); }

## Change 2

| Function name | fl_TC_11_1_7_NR5GC_TestBody |
| --- | --- |
| Reason for change | - Acoording to 38.331 clause 5.3.8.3, UE will ignore any field included in RRCRelease message except waitTime if the AS security is not activated, so it is should add security activate process after step 5. |
| Summary of change | - Add f_NR_RRC_ActivateSecurity after step5. |
| TTCN module | EPS_Fallback_NR5GC_NR.ttcn |
| MCC160 Comment | Accepted subject to prose CR approval |

Before Change

function fl_TC_11_1_7_NR5GC_TestBody() runs on NR5GC_PTC { var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; //@siclog "Step 2" siclog@ f_UT_RequestIMSEmergencyCall(UT, px_EmergencyCallNumber); //@siclog "Steps 3-5" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, emergency); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); if (f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0100'B)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cell). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo )); // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); // Rest of test on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); }

After Change

function fl_TC_11_1_7_NR5GC_TestBody() runs on NR5GC_PTC { var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; //@siclog "Step 2" siclog@ f_UT_RequestIMSEmergencyCall(UT, px_EmergencyCallNumber); //@siclog "Steps 3-5" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, emergency); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); if (f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0100'B)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } f_NR_RRC_ActivateSecurity (nr_Cell1, v_ReceivedMsg.SecurityProtection.NasCount); //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cell). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo )); // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); // Rest of test on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); }

## Change 3

| Function name | fl_TC_11_1_7_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | - v_NasCount should to be initialized before step 10A. - Need to receive RRCConnectionReconfigurationComplete message in step 10A-10D. - Because SRB2 has been established in step 10A-10D, Nas message of additionl PDN needs to use SRB2 receive. |
| Summary of change | - v_NasCount is equal to Nas sequeues number of TAU. - Add f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg in step 10A-10D. - Set the fourth parameter of f_EUTRA38_MultiPDN_EstablishAdditionalPDNs to srb2 in step 12-16. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment | 1. Already implemented in wk12 2.Accepted 3. Accepted in principle but changed the value of the 4th parameter in f_EUTRA38_MultiPDN_EstablishAdditionalPDNs to tsc_SRB2 |

Before Change

function fl_TC_11_1_7_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var NasCount_Type v_NasCount; var NAS_MSG_Indication_Type v_NasInd; // @siclog "Step 1" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // @siclog "Steps 7-10b2" siclog@ // The UE transmits an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'. SRB.check(receive(car_SRB0_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionRequest(emergency)))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 7"); // @sic R5-204467 sic@ v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency); // @siclog "Step 10A - 10D" siclog@ f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NasCount); // FFS: CR R5 204467 steps 2-4b of 4.9.7 with mapped security context. // Steps 4a1 - 4b2 are not performed for mapped security context, so not included here // @siclog "Step 10E - 11" siclog@ v_NasCount := f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1, v_NasInd, MAPPED_CXT, noRrcConnectionRelease, -, true); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); // @siclog "Steps 12 - 16" siclog@ // Steps 9-15 of 4.5A.4.3-1 f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1); // @siclog "Step 19-20" siclog@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20"); // @siclog "Step 21" siclog@ f_Delay(1.0); // @siclog "Step 22" siclog@ // Tell IMS PTC to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); //IMS call has been released. f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // The SS deactivates the EPS bearer. [This is the rest of C.32] f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); }

After Change

function fl_TC_11_1_7_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var NasCount_Type v_NasCount; var NAS_MSG_Indication_Type v_NasInd; // @siclog "Step 1" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // @siclog "Steps 7-10b2" siclog@ // The UE transmits an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'. SRB.check(receive(car_SRB0_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionRequest(emergency)))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 7"); // @sic R5-204467 sic@ v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency); v_NasCount := v_NasInd.SecurityProtection.NasCount; // @siclog "Step 10A - 10D" siclog@ f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NasCount); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); // FFS: CR R5 204467 steps 2-4b of 4.9.7 with mapped security context. // Steps 4a1 - 4b2 are not performed for mapped security context, so not included here // @siclog "Step 10E - 11" siclog@ v_NasCount := f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1, v_NasInd, MAPPED_CXT, noRrcConnectionRelease, -, true); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); // @siclog "Steps 12 - 16" siclog@ // Steps 9-15 of 4.5A.4.3-1 f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1 , -, tsc_SRB2); // @siclog "Step 19-20" siclog@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20"); // @siclog "Step 21" siclog@ f_Delay(1.0); // @siclog "Step 22" siclog@ // Tell IMS PTC to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); //IMS call has been released. f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // The SS deactivates the EPS bearer. [This is the rest of C.32] f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); }

## Change 4

| Function name | f_TC_11_1_7_NR5GC_IMS2 |
| --- | --- |
| Reason for change | - Since the emergency call is on the LTE side, ims2 PTC needs to be associated with LTE PTC. |
| Summary of change | - Set the fourth parameter of f_IMS_EmergencyCallSetup to OtherIPCAN_E. |
| TTCN module | EPS_Fallback_NR5GC_NR.ttcn |
| MCC160 Comment | Rejected. There is no IPCAN port in this function as it doesn’t interact with any PTC. It does interact with OtherIPCAN before/after the emergency call is released. |

Before Change

function f_TC_11_1_7_NR5GC_IMS2()runs on IMS_PTC { var INVITE_Request v_InviteRequest; f_IMS_PTC_Init(PDN_2); // Test Case Body // Emergency registration: 34.229-1 Annex C.20 f_IMS_EmergencyRegistration(); // Emergency call setup: 34.229-1 Annex C.22 v_InviteRequest := f_IMS_EmergencyCallSetup(); //Release Emergency Call MO //@siclog "Step 22" siclog@ f_IMS_IPCAN_WaitForTrigger(OtherIPCAN); f_IMS_EmergencyCallRelease(v_InviteRequest); // Inform EUTRA: IMS Call released. f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); }

After Change

function f_TC_11_1_7_NR5GC_IMS2()runs on IMS_PTC { var INVITE_Request v_InviteRequest; f_IMS_PTC_Init(PDN_2); // Test Case Body // Emergency registration: 34.229-1 Annex C.20 f_IMS_EmergencyRegistration(); // Emergency call setup: 34.229-1 Annex C.22 v_InviteRequest := f_IMS_EmergencyCallSetup (-, -, OtherIPCAN_E); //Release Emergency Call MO //@siclog "Step 22" siclog@ f_IMS_IPCAN_WaitForTrigger(OtherIPCAN); f_IMS_EmergencyCallRelease(v_InviteRequest); // Inform EUTRA: IMS Call released. f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); }

# Branches executed

NR Band 41,Nea2,Nia2.

LTE Band 3 Uea2 Uia2

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on Datang LinkTester 5G Protocol Conformance Test System ECT9610. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:
\ TC_11_1_7_NR5GC_N41-FDD3.log

- PIXCIT settings used:
\ Pics(Hisilicon-IRAT).xml

\ Pixit(TC_11_1_7_NR5GC).xml

# References

| [1] | R5s210342: Supporting information for agreement of NR5GC test case 11.1.7 in FR1 |
| --- | --- |