3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210384

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0013 | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCPTT test case 5.1 to the MCX ATS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UPV/EHU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-03-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add MCPTT TC 5.1 to the MCX ATS 21WK12 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to the TTCN code required to obtain a pass using the MCX IPCAN test model. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to the MCX ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Logs in R5s210385 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

3	Branches executed	4

4	Execution Log Files	5

4.1	Nemergent Solutions Client v7.0	5

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 5.1 which is part of the ATS iwd-TTCN3-B2020-09_D21wk12_MCX test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Fidel Liberal
	fidel.liberal@ehu.eus

## Verification Test Summary

Test Case: 	5.1

ATS Version:	ATS  iwd-TTCN3-B2020-09_D21wk12_MCX

System Simulator used:	MCSTaaSting Tester

UE used:	Nemergent Client v7.0 on COTS LTE UE

Verification Status:	PASS

# Corrections required

## Change 1

| PIXIT pars | Parameters for EPS bearer Ids |
| --- | --- |
| Reason for change | SS EUTRA HW may need different ids to be assigned to its EPS bearers |
| Summary of change | Constant values changed to PIXITs |
| TTCN module | MCX_CommonIPCAN.ttcn |
| MCC160 Comment | Accepted See prose CR for TS 36.579-5, CR 0017 (R5-213670) |

- Before Change:

const MCX_IPCAN_EpsBearerId_Type tsc_MCX_IPCAN_EpsBearerId_IMS := 5; const MCX_IPCAN_EpsBearerId_Type tsc_MCX_IPCAN_EpsBearerId_INTERNET := 6; const MCX_IPCAN_EpsBearerId_Type tsc_MCX_IPCAN_EpsBearerId_MCX := 7; const MCX_IPCAN_EpsBearerId_Type tsc_MCX_IPCAN_EpsBearerId_MCX_Audio := 8; const MCX_IPCAN_EpsBearerId_Type tsc_MCX_IPCAN_EpsBearerId_MCX_Video := 9;

- After Change:

modulepar MCX_IPCAN_EpsBearerId_Type px_MCX_IPCAN_EpsBearerId_IMS := 5; modulepar MCX_IPCAN_EpsBearerId_Type px_MCX_IPCAN_EpsBearerId_INTERNET := 6; modulepar MCX_IPCAN_EpsBearerId_Type px_MCX_IPCAN_EpsBearerId_MCX := 7; modulepar MCX_IPCAN_EpsBearerId_Type px_MCX_IPCAN_EpsBearerId_MCX_Audio := 8; modulepar MCX_IPCAN_EpsBearerId_Type px_MCX_IPCAN_EpsBearerId_MCX_Video := 9;

## Change 2

| Function name | f_MCX_MCPTTResourceLists_EncodeEncrypt |
| --- | --- |
| Reason for change | Encoding is done on an alias type which results in incorrect encoding. |
| Summary of change | Correct type Resource_lists is used instead of alias type |
| TTCN module | MCX_XML_resource_lists.ttcn |
| MCC160 Comment | Accepted in principle by changing the parameter type |

- Before Change:

function f_MCX_MCPTTResourceLists_EncodeEncrypt(...) { var universal charstring v_XmlDocument := f_Bit2Unichar(encvalue(p_MCPTT_ResourceLists)); if (isvalue(p_CSK)) { // logging needed only in case of encryption f_MCX_LogXML(DOWNLINK, v_XmlDocument); f_XML_Document_EncryptElementContent(v_XmlDocument, "//resource-lists", p_MCServiceID, p_CSK); / } return f_UniCharstring2XmlBody(v_XmlDocument); }

- After Change:

function f_MCX_MCPTTResourceLists_EncodeEncrypt(...) { var Resource_lists v_ResourceLists := valueof(p_MCPTT_ResourceLists); var universal charstring v_XmlDocument := f_Bit2Unichar(encvalue(v_ResourceLists)); if (isvalue(p_CSK)) { // logging needed only in case of encryption f_MCX_LogXML(DOWNLINK, v_XmlDocument); f_XML_Document_EncryptElementContent(v_XmlDocument, "//resource-lists", p_MCServiceID, p_CSK); / } return f_UniCharstring2XmlBody(v_XmlDocument); }

# Branches executed

- TLSv1.2

- IPSec ipsec-3gpp;alg=hmac-sha-1-96;ealg=aes-cbc

- XML integrity and encrpytion according to  http://www.w3.org/2009/xmlenc11#aes128-gcm

- XML signatures according to  http://www.w3.org/2001/04/xmlenc#sha256

- MIKEY SAKKE/ECCSI (RFC 6507, 6508, 6509)

- LTE Band 1

- EEA2/EIA2 RRC/NAS ciphering/integrity algorithms

# Execution Log Files

## Nemergent Solutions Client v7.0

The Nemergent Solutions Client v7.0 on COTS LTE UE passed this test case against the MCSTaaSting Tester. The documentation below is enclosed as evidence of the successful test case run [1]:

- SS_Log_Coversheet.txt

- Signalling-TC5.1.html (IP flow decrypted/decoded)

- MCX-PIXIT.cfg (Titan configuration and PIXITs)

- TLI Interface log

| MCC160 Comment | Insufficient logging details provided. According to RAN5 decision at RAN5#91-e (in R5-213431) the LTE (NAS, RRC) logging must also be provided by the SS. |
| --- | --- |

# References

| [1] | R5s210385: Supporting information for the addition of MCPTT test case 5.1 to the MCX ATS |
| --- | --- |