3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210387

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1644 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS voice call procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For the IMS call release sequences as defined in 38.508-1 4.9.17/4.9.18, the current contents of PDU SESSION MODIFICATION COMMAND are including the QoS rule and QoS flow configuration for IMS signalling bearer, with operation code “add new QoS rule / QoS flow”. This can give an error in UE processing as these IDs will already exist as part of the PDU session. The operations configured in the message should be a delta to the existing PDU session, and should delete the QoS rule and QoS flow that were added in the establishment of IMS voice traffic bearer, through sequences 4.9.15/4.9.16 (function _NR5GC_ModifyPDUSession_AddVoice()). - 2. Following sending of RRCReconfiguration with piggybacked PDU SESSION MODIFICATION COMMAND, the RRCreconfigurationComplete and PDU SESSION MODIFICATION COMPLETE responses can arrive from the UE in any order. Because RRCReconfigurationComplete is handled inside the function f_NR_RRCReconfigNewDRBWithSSConfig(), the test case will fail if PDU SESSION MODIFICATION COMPLETE is received first. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify PDU SESSION MODIFICATION COMMAND contents to release QoS rule ID 3 and QoS flow ID 7. Note : An associated prose CR on 38.508-1 will be raised at RAN5#91. - Pass parameter to f_NR_RRCReconfigNewDRBWithSSConfig() to not wait for RRCReconfgiruationComplete, and handled this in interleave with PDU SESSION MODIFICATION COMPLETE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_NR5GC_ModifyPDUSession_AddVoice(), f_NR5GC_ModifyPDUSession_AddVoice() |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	8

2.3	Change 3	13

2.4	Change 4	13

2.5	Change 5	18

2.6	Change 6	19

2.7	Change 7	19

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_ModifyPDUSession_AddVoice() |
| --- | --- |
| Reason for change | Following sending of RRCReconfiguration with piggybacked PDU SESSION MODIFICATION COMMAND, the RRCreconfigurationComplete and PDU SESSION MODIFICATION COMPLETE responses can arrive from the UE in any order. Because RRCReconfigurationComplete is handled inside the function f_NR_RRCReconfigNewDRBWithSSConfig(), the test case will fail if PDU SESSION MODIFICATION COMPLETE is received first. |
| Summary of change | Pass parameter to f_NR_RRCReconfigNewDRBWithSSConfig() to not wait for RRCReconfgiruationComplete, and handled this in interleave with PDU SESSION MODIFICATION COMPLETE. |
| TTCN module | NR5GC_IMS_Procedures.ttcn |
| MCC160 Comment | These messages are defined as sequential, not parallel, in all procedures in 38.508-1 (4.9.11 – 4.9.18). This must be added to the prose CR. Decision postponed until outcome of prose CR |

Before Change

function f_NR5GC_ModifyPDUSession_AddVoice(NR_CellId_Type p_NR_CellId, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_InterworkWithEPS := false; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var integer v_NewDRBId; var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId); var template (value) QosFlow_Identification_Type v_QosFlowId; if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(p_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules("7_Voice", '7A'O), // @sic R5s210073 sic@ cs_QoSFlowDescr({cs_QoS_Flow5(v_InterworkWithEPS)})); v_NewDRBId := f_NR_GetNextDRBId(p_GSM_MobilityInfo, false); // @sic R5-211171 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7})), // QFI of 7 in flow 5 v_NewDRBId, -, -, cs_38508_PDCP_Config (-, -, -, -, omit))}); // UM v_SS_Config := {f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_NewDRBId, -, -, f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7}))))}; // @sic R5-211371, R5s210330 sic@ v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB(p_NR_CellId, {v_NewDRBId}, false); v_CellGroupConfig.rlc_BearerToAddModList := {cs_38508_RLC_BearerConfig_DRB(v_NewDRBId, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_38508_LogicalChannelConfig_DRB)}; v_QosFlowId := cs_IP_QosFlowId(oct2int(p_GSM_MobilityInfo.SessionId), 7); // @sic R5s210363 sic@ f_IP_RtpRtcpLoopback(IP, p_GSM_MobilityInfo.PdnIndex, tsc_IMS_MediaPort_M1, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s210363 sic@ // then send RRC Reconfiguration message f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }); SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Establishment"); } }

After Change

function f_NR5GC_ModifyPDUSession_AddVoice(NR_CellId_Type p_NR_CellId, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_InterworkWithEPS := false; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var integer v_NewDRBId; var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId); var template (value) QosFlow_Identification_Type v_QosFlowId; if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(p_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules("7_Voice", '7A'O), // @sic R5s210073 sic@ cs_QoSFlowDescr({cs_QoS_Flow5(v_InterworkWithEPS)})); v_NewDRBId := f_NR_GetNextDRBId(p_GSM_MobilityInfo, false); // @sic R5-211171 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7})), // QFI of 7 in flow 5 v_NewDRBId, -, -, cs_38508_PDCP_Config (-, -, -, -, omit))}); // UM v_SS_Config := {f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_NewDRBId, -, -, f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7}))))}; // @sic R5-211371, R5s210330 sic@ v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB(p_NR_CellId, {v_NewDRBId}, false); v_CellGroupConfig.rlc_BearerToAddModList := {cs_38508_RLC_BearerConfig_DRB(v_NewDRBId, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_38508_LogicalChannelConfig_DRB)}; v_QosFlowId := cs_IP_QosFlowId(oct2int(p_GSM_MobilityInfo.SessionId), 7); // @sic R5s210363 sic@ f_IP_RtpRtcpLoopback(IP, p_GSM_MobilityInfo.PdnIndex, tsc_IMS_MediaPort_M1, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s210363 sic@ // then send RRC Reconfiguration message f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }, -, -, false); interleave { [] SRB.receive(car_NR_SRB_RrcPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) {} [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp {} } if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Establishment"); } }

## Change 2

| Function name | f_NR5GC_ModifyPDUSession_RemoveIMSVoice() |
| --- | --- |
| Reason for change | 1. For the IMS call release sequences as defined in 38.508-1 4.9.17/4.9.18, the current contents of PDU SESSION MODIFICATION COMMAND are including the QoS rule and QoS flow configuration for IMS signalling bearer, with operation code “add new QoS rule / QoS flow”. This can give an error in UE processing as these IDs will already exist as part of the PDU session. The operations configured in the message should be a delta to the existing PDU session, and should delete the QoS rule and QoS flow that were added in the establishment of IMS voice traffic bearer, through sequences 4.9.15/4.9.16 (function _NR5GC_ModifyPDUSession_AddVoice()). 2. Following sending of RRCReconfiguration with piggybacked PDU SESSION MODIFICATION COMMAND, the RRCreconfigurationComplete and PDU SESSION MODIFICATION COMPLETE responses can arrive from the UE in any order. Because RRCReconfigurationComplete is handled inside the function f_NR_RRCReconfigNewDRBWithSSConfig(), the test case will fail if PDU SESSION MODIFICATION COMPLETE is received first. |
| Summary of change | - Modify PDU SESSION MODIFICATION COMMAND contents to release QoS rule ID 3 and QoS flow ID 7. Note : An associated prose CR on 38.508-1 will be raised at RAN5#91. - Pass parameter to f_NR_RRCReconfigNewDRBWithSSConfig() to not wait for RRCReconfgiruationComplete, and handled this in interleave with PDU SESSION MODIFICATION COMPLETE. |
| TTCN module | NR5GC_IMS_Procedures.ttcn |
| MCC160 Comment | 1. Postponed until outcome of prose CR 2. These messages are defined as sequential, not parallel, in all procedures in 38.508-1 (4.9.11 – 4.9.18). Decision postponed until outcome of prose CR |

Before Change

function f_NR5GC_ModifyPDUSession_RemoveIMSVoice(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_InterworkWithEPS := false; var O1_Type v_EPSBearer := '00'O; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var template (value) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForIMSPDUSession(); var template (value) NR_RLC_BearerToReleaseList_Type v_RLC_BearerToReleaseList; var NR_DRBInfo_Type v_DRBInfo; var template (value) DRB_ToReleaseList v_DRB_ToReleaseList; var integer i; var integer j := 0; if (isvalue(v_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; v_EPSBearer := int2oct(valueof(v_GSM_MobilityInfo.EPS_Bearer), 1); } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules("2", '7A'O), // @sic R5s210073 sic@ cs_QoSFlowDescr({cs_QoS_Flow2(v_InterworkWithEPS, v_EPSBearer)})); // @sic R5-211171 sic@ for (i := 0; i < lengthof(v_DRBIdList); i := i+1) { v_DRBInfo := valueof(f_NR_GetDRBInfo_FromId(v_DRBIdList[i])); if (not (v_DRBInfo.DefaultDRB)) { v_DRB_ToReleaseList[j] := v_DRBInfo.DRB_Id; v_RLC_BearerToReleaseList[j] := v_DRBInfo.RLC_Config.logicalChannelIdentity; v_SS_Config[j] := cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBInfo.DRB_Id)); j := j+1; } } v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, v_DRB_ToReleaseList); v_CellGroupConfig := cs_38508_CellGroupConfig(f_NR_CellInfo_Get_CellGroupId(p_NR_CellId), omit, v_RLC_BearerToReleaseList); // The SS transmits an RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION MODIFICATION COMMAND f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }); SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Release"); } f_IP_RtpRtcpLoopback(IP, valueof(v_GSM_MobilityInfo.PdnIndex), tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ }

After Change

function f_NR5GC_ModifyPDUSession_RemoveIMSVoice(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_InterworkWithEPS := false; var O1_Type v_EPSBearer := '00'O; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var template (value) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForIMSPDUSession(); var template (value) NR_RLC_BearerToReleaseList_Type v_RLC_BearerToReleaseList; var NR_DRBInfo_Type v_DRBInfo; var template (value) DRB_ToReleaseList v_DRB_ToReleaseList; var integer i; var integer j := 0; if (isvalue(v_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; v_EPSBearer := int2oct(valueof(v_GSM_MobilityInfo.EPS_Bearer), 1); } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules("7_Voice", '7A'O, true), // @sic R5s210073 sic@ cs_QoSFlowDescr({cs_QoS_FlowDel5(v_InterworkWithEPS, v_EPSBearer)})); // @sic R5-211171 sic@ for (i := 0; i < lengthof(v_DRBIdList); i := i+1) { v_DRBInfo := valueof(f_NR_GetDRBInfo_FromId(v_DRBIdList[i])); if (not (v_DRBInfo.DefaultDRB)) { v_DRB_ToReleaseList[j] := v_DRBInfo.DRB_Id; v_RLC_BearerToReleaseList[j] := v_DRBInfo.RLC_Config.logicalChannelIdentity; v_SS_Config[j] := cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBInfo.DRB_Id)); j := j+1; } } v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, v_DRB_ToReleaseList); v_CellGroupConfig := cs_38508_CellGroupConfig(f_NR_CellInfo_Get_CellGroupId(p_NR_CellId), omit, v_RLC_BearerToReleaseList); // The SS transmits an RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION MODIFICATION COMMAND f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }, -,-,false); interleave { [] SRB.receive(car_NR_SRB_RrcPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) {} [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp {} } if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Release"); } f_IP_RtpRtcpLoopback(IP, valueof(v_GSM_MobilityInfo.PdnIndex), tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ }

## Change 3

| Function name | New template cs_QoS_FlowDel |
| --- | --- |
| Reason for change | As change 2.1 |
| Summary of change | New template for QoS flow with operation code 010 “delete existing QoS flow” |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment | Postponed until outcome of prose CR |

New template

template (value) QoS_Flow cs_QoS_FlowDel (B6_Type p_QFI) := { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ // @sic R5-194896 sic@ spare2 := tsc_Spare2, qfi := p_QFI, operationCode := '010'B, //Delete spare6 := tsc_Spare6, eBit := '0'B, numOfParameters :='000000'B, parameterList := omit };

## Change 4

| Function name | f_BuildDefaultQosFlow |
| --- | --- |
| Reason for change | As change 2.1 |
| Summary of change | Add new parameter to allow QoS flow delete operation |
| TTCN module | NG_NAS_Templated.ttcn |
| MCC160 Comment | Postponed until outcome of prose CR |

Before Change

function f_BuildDefaultQosFlow (charstring p_FlowNumber, boolean p_InterworkWithEPS := false, O1_Type p_EPSBearerId := '00'O) return template (value) QoS_Flow { var template (value) QoSParametersList v_QoSParametersList; var B6_Type v_QFI; select (p_FlowNumber) { case ("1") { v_QFI := '000001'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, p_EPSBearerId); // @sic R5-204400 sic@ } } case ("2") { v_QFI := '000010'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '50'O); } } case ("2a") { v_QFI := '000100'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; } case ("3") { v_QFI := '000101'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; } case ("4") { v_QFI := '000110'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; } case ("5") { v_QFI := '000111'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '01'O), cs_QoSParameters(tsc_QoSFlowParameter_GFBR_UL, '02'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_GFBR_DL, '03'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_UL, '04'O, fl_FlowParameterBitRate(320, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_DL, '05'O, fl_FlowParameterBitRate(320, 1))}; // @sic R5s210330 sic@ if (p_InterworkWithEPS) { v_QoSParametersList[5] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '06'O, '60'O); // @sic R5s210330 sic@ } } case ("6") { v_QFI := '001000'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '02'O)/*, cs_QoSParameters(tsc_QoSFlowParameter_GFBR_UL, '02'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_GFBR_DL, '03'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_UL, '04'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_DL, '05'O, fl_FlowParameterBitRate(128, 1))*/}; if (p_InterworkWithEPS) { v_QoSParametersList[5] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '06'O, '70'O); } } case ("7") { // @sic R5-211499 sic@ v_QFI := '001001'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '52'O)}; // 5QI 82 } } return cs_QoS_Flow (v_QFI, v_QoSParametersList); }

After Change

function f_BuildDefaultQosFlow (charstring p_FlowNumber, boolean p_InterworkWithEPS := false, O1_Type p_EPSBearerId := '00'O , boolean p_Delete := false) return template (value) QoS_Flow { var template (value) QoSParametersList v_QoSParametersList; var B6_Type v_QFI; select (p_FlowNumber) { case ("1") { v_QFI := '000001'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, p_EPSBearerId); // @sic R5-204400 sic@ } } case ("2") { v_QFI := '000010'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '50'O); } } case ("2a") { v_QFI := '000100'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; } case ("3") { v_QFI := '000101'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; } case ("4") { v_QFI := '000110'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; } case ("5") { v_QFI := '000111'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '01'O), cs_QoSParameters(tsc_QoSFlowParameter_GFBR_UL, '02'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_GFBR_DL, '03'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_UL, '04'O, fl_FlowParameterBitRate(320, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_DL, '05'O, fl_FlowParameterBitRate(320, 1))}; // @sic R5s210330 sic@ if (p_InterworkWithEPS) { v_QoSParametersList[5] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '06'O, '60'O); // @sic R5s210330 sic@ } } case ("6") { v_QFI := '001000'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '02'O)/*, cs_QoSParameters(tsc_QoSFlowParameter_GFBR_UL, '02'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_GFBR_DL, '03'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_UL, '04'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_DL, '05'O, fl_FlowParameterBitRate(128, 1))*/}; if (p_InterworkWithEPS) { v_QoSParametersList[5] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '06'O, '70'O); } } case ("7") { // @sic R5-211499 sic@ v_QFI := '001001'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '52'O)}; // 5QI 82 } } if(p_Delete == true) { return cs_QoS_FlowDel (v_QFI); } else { return cs_QoS_Flow (v_QFI, v_QoSParametersList); } }

## Change 5

| Function name | New template cs_QoS_FlowDel5 |
| --- | --- |
| Reason for change | As change 2.1 |
| Summary of change | New template to delete QFI 7 |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment | Postponed until outcome of prose CR |

New template

template (value) QoS_Flow cs_QoS_FlowDel5(boolean p_InterworkWithEPS := false, O1_Type p_EPSBearerId := '00'O) := f_BuildDefaultQosFlow("5", p_InterworkWithEPS, p_EPSBearerId, true);

## Change 6

| Function name | New template cs_QoS_RuleDel |
| --- | --- |
| Reason for change | As change 2.1 |
| Summary of change | New template for QoS rule with operaition code 010 “Delete existing QoS rule” |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment | Postponed until outcome of prose CR |

New template

template (value) QoS_Rule cs_QoS_RuleDel (O1_Type p_Id ) := { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ identifier := p_Id, iel := '0001'O, // For the "delete existing QoS rule" operation, the length of QoS rule field is set to one. ruleOperationCode := '010'B, // Delete dqrBit := '0'B, numOfPacketFilters := '0000'B, packetFilterList := omit, precedence := omit, spare := omit, segregation := omit, flowIdentifier := omit };

## Change 7

| Function name | f_BuildDefaultQoSRules() |
| --- | --- |
| Reason for change | As change 2.l |
| Summary of change | Add new parameter to allow QoS rule delete operation |
| TTCN module | NG_NAS_Templated.ttcn |
| MCC160 Comment | Postponed until outcome of prose CR |

Before Change

function f_BuildDefaultQoSRules (charstring p_RuleNum, template (omit) IEI8_Type p_IEI := omit) return template (value) QoS_Rules { var template (value) QoS_Rules v_QoS_Rules; if (p_RuleNum == "7_Voice") { v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('03'O, int2bit(7, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0110'B), '0'B)}, p_IEI) // @sic R5-210627 sic@ } else if (p_RuleNum == "7_Video") { v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('04'O, int2bit(8, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0111'B), '0'B)}, p_IEI) // @sic R5-210627 sic@ } else { v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule(p_RuleNum)}, p_IEI); } return v_QoS_Rules; }

After Change

function f_BuildDefaultQoSRules (charstring p_RuleNum, template (omit) IEI8_Type p_IEI := omit , boolean p_Delete:= false) return template (value) QoS_Rules { var template (value) QoS_Rules v_QoS_Rules; if (p_RuleNum == "7_Voice") { if(p_Delete) { v_QoS_Rules := cs_QoS_Rules ({cs_QoS_RuleDel('03'O)}, p_IEI); } else{ v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('03'O, int2bit(7, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0110'B), '0'B)}, p_IEI); // @sic R5-210627 sic@ } } else if (p_RuleNum == "7_Video") { if (p_RuleNum == "7_Voice") { if(p_Delete) { v_QoS_Rules := cs_QoS_Rules ({cs_QoS_RuleDel('04'O)}, p_IEI); } Else { v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('04'O, int2bit(8, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0111'B), '0'B)}, p_IEI) // @sic R5-210627 sic@ } } else { v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule(p_RuleNum)}, p_IEI); } return v_QoS_Rules; }