3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210403

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1651 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR 5GC IRAT test case 6.2.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Betweeen step 6 and step 7, the UE may reselect back to E-UTRA Cell 1, for the reasons given in R5-210632: i) The prority of serving cell (NR Cell 1) is 4, and the prority of neighbour cell (E-UTRA Cell 1) is 3. ii) For serving cell (NR Cell 1), Squal-NR Cell 1< threshServingLow-Q is satisfied, where Squal-NR Cell 1=13.04 and threshServingLow-Q=26. iii) For neighbour cell (E-UTRA Cell 1), Squal-E-UTRA Cell 1 > threshX-Low-Q is satisfied, where Squal-E-UTRA Cell 1=8.56 and threshX-Low-Q=5. The solution implemented in R5-210632 does not stably resolve this issue. threshX-Low-Q is changed to 10 giving 10-8.56 = 1.46 which is too narrow to stabilize the UE behaviour. - Between steps 8 and 10, the UE may reselect back to E-UTRA Cell 1, for the reasons given in R5-210632: i) The prority of serving cell (NR Cell 1) is 4, and the prority of neighbour cell (E-UTRA Cell 1) is 5. ii) For neighbour cell (E-UTRA Cell 1), Squal-E-UTRA Cell 1 > threshX-High-Q is satisfied, where Squal-E-UTRA Cell 1=8.56 and threshX-High-Q=5. The solution implemented in R5210632 Does not resolve the issue. ThreshX, HighQ in NR SIB5 should be changed instead of in EUTRA SIB24 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change ThreshServing,LowQ in NR SIB2 to 9 - Change ThreshX, HighQ to 12 in NR cell 1 SIB 5. Note : An associated prose CR on 38.523-1 will be raised at RAN5#91 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	7

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_2_3_4_NR5GC |
| --- | --- |
| Reason for change | Betweeen step 6 and step 7, the UE may reselect back to E-UTRA Cell 1, for the reasons given in R5-210632: - The prority of serving cell (NR Cell 1) is 4, and the prority of neighbour cell (E-UTRA Cell 1) is 3. - For serving cell (NR Cell 1), Squal-NR Cell 1< threshServingLow-Q is satisfied, where Squal-NR Cell 1=13.04 and threshServingLow-Q=26. - For neighbour cell (E-UTRA Cell 1), Squal-E-UTRA Cell 1 > threshX-Low-Q is satisfied, where Squal-E-UTRA Cell 1=8.56 and threshX-Low-Q=5. The solution implemented in R5-210632 does not stably resolve this issue. threshX-Low-Q is changed to 10 giving 10-8.56 = 1.46 which is too narrow to stabilize the UE behaviour. |
| Summary of change | Change ThreshServing,LowQ in NR SIB2 to 9 Note : An associate prose CR on 38.523-1 will be raised at RAN5#90 |
| TTCN module | Idle_CellReSelection_NR5GC_NR |
| MCC160 Comment | Accepted subject to prose CR approval at RAN5#91e |

Before Change

function f_TC_6_2_3_4_NR5GC() runs on NR5GC_PTC { // Inter-RAT cell reselection / From NR RRC_Idle to E-UTRA_IDLE (lower priority & higher priority, Squal based) // @sic R5-210632 sic@ var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var boolean v_IsFR1; var Q_RxLevMin v_Q_RxLevMin; var DeltaValues_Type v_DeltaPrimaryBand; // System information combination NR-6 as defined in TS 38.508-1 [7] clause 4.4.3.1, and message contents defined in clause 4.6.1 and clause 4.6.2 with QBASED condition is used in NR cell. f_NR5GC_IRAT_Init(NR_6); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -80, -80); v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); // E-UTRA Cell 1 information Rx. // Table 6.2.3.4.3.3-1:SIB1 for NR Cell 1 v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { // FR1 f_NR_CellInfo_SetSIB1_CellSelectionInfo(nr_Cell1, cs_38508_SIB1_CellSelectionInfo(-55,-,-25)); } else { // FR2 v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); v_Q_RxLevMin := f_Round((-110.0 + int2float(v_DeltaPrimaryBand.DeltaNRf2))/2.0); f_NR_CellInfo_SetSIB1_CellSelectionInfo(nr_Cell1, cs_38508_SIB1_CellSelectionInfo(v_Q_RxLevMin,-,-25)); } // Table 6.2.3.4.3.3-2:SIB2 for NR Cell 1 f_NR_CellInfo_SetSIB2_ThreshServingLowQ(nr_Cell1, 26); // Table 6.2.3.4.3.3-3:SIB5 NR Cell 1 f_NR_CellInfo_SetSIB5_CarrierFreq(nr_Cell1, 0, v_CoOrd_SysInfo.Eutra[0].Arfcn); f_NR_CellInfo_SetSIB5_ThreshXHighQ(nr_Cell1, 0, cs_InterFreqCarrierFreqInfo_ThreshX_Q(20, 10)); f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_IRAT_PreambleEUTRA_NR (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_6_2_3_4_TestBody(); f_NR_TestBody_Set(false); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); // UE now on EUTRA f_NR_ReleaseAllCells(); }

After Change

function f_TC_6_2_3_4_NR5GC() runs on NR5GC_PTC { // Inter-RAT cell reselection / From NR RRC_Idle to E-UTRA_IDLE (lower priority & higher priority, Squal based) // @sic R5-210632 sic@ var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var boolean v_IsFR1; var Q_RxLevMin v_Q_RxLevMin; var DeltaValues_Type v_DeltaPrimaryBand; // System information combination NR-6 as defined in TS 38.508-1 [7] clause 4.4.3.1, and message contents defined in clause 4.6.1 and clause 4.6.2 with QBASED condition is used in NR cell. f_NR5GC_IRAT_Init(NR_6); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -80, -80); v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); // E-UTRA Cell 1 information Rx. // Table 6.2.3.4.3.3-1:SIB1 for NR Cell 1 v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { // FR1 f_NR_CellInfo_SetSIB1_CellSelectionInfo(nr_Cell1, cs_38508_SIB1_CellSelectionInfo(-55,-,-25)); } else { // FR2 v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); v_Q_RxLevMin := f_Round((-110.0 + int2float(v_DeltaPrimaryBand.DeltaNRf2))/2.0); f_NR_CellInfo_SetSIB1_CellSelectionInfo(nr_Cell1, cs_38508_SIB1_CellSelectionInfo(v_Q_RxLevMin,-,-25)); } // Table 6.2.3.4.3.3-2:SIB2 for NR Cell 1 f_NR_CellInfo_SetSIB2_ThreshServingLowQ(nr_Cell1, 9); // Table 6.2.3.4.3.3-3:SIB5 NR Cell 1 f_NR_CellInfo_SetSIB5_CarrierFreq(nr_Cell1, 0, v_CoOrd_SysInfo.Eutra[0].Arfcn); f_NR_CellInfo_SetSIB5_ThreshXHighQ(nr_Cell1, 0, cs_InterFreqCarrierFreqInfo_ThreshX_Q(20, 10)); f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_IRAT_PreambleEUTRA_NR (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_6_2_3_4_TestBody(); f_NR_TestBody_Set(false); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); // UE now on EUTRA f_NR_ReleaseAllCells(); }

<Indicate the exact TTCN changes performed >

## Change 2

| Function name | fl_TC_6_2_3_4_TestBody |
| --- | --- |
| Reason for change | Between steps 8 and 10, the UE may reselect back to E-UTRA Cell 1, for the reason given in R5-210632: - The prority of serving cell (NR Cell 1) is 4, and the prority of neighbour cell (E-UTRA Cell 1) is 5. - For neighbour cell (E-UTRA Cell 1), Squal-E-UTRA Cell 1 > threshX-High-Q is satisfied, where Squal-E-UTRA Cell 1=8.56 and threshX-High-Q=5. The solution implemented in R5210632 Does not resolve the issue. ThreshX, HighQ in NR SIB5 should be changed instead of in EUTRA SIB24 |
| Summary of change | Change ThreshX, HighQ to 12 in NR cell 1 SIB 5. Note : An associate prose CR on 38.523-1 will be raised at RAN5#90 |
| TTCN module | Idle_CellReSelection_NR5GC_NR |
| MCC160 Comment | Accepted subject to prose CR approval at RAN5#91e |

Before Change

function fl_TC_6_2_3_4_TestBody() runs on NR5GC_PTC { var integer v_Noc := -85; // @sic R5s200723 sic@ //@siclog "Step 1" siclog@ //The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T1" in table 6.2.3.4.3.2-1/2/3. f_NR_SetCellPower (nr_Cell1, -95, -95); // FR2 FFS f_NR_ConfigureAndActivateVNG_MultipleCells({cs_CellNoiseLevel(nr_Cell1, v_Noc)}); // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA); // wait until power level change on Step4 //@siclog "Step 4" siclog@ // The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T2" in table 6.2.3.4.3.2-1/2/3. f_NR_SetCellPower (nr_Cell1, -80, -80); // @siclog "Step 5" siclog@ // The UE selects NR Cell 1 and performs the generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 with condition 'connected without release'. f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, noRrcConnectionRelease, false); // @siclog "Step 6" siclog@ // The SS transmits an RRCRelease message on NR Cell 1. f_NR_RRCRelease(nr_Cell1); // @sic R5s210276 sic@ // @siclog "Step 7" siclog@ // The SS changes the E-UTRA cell priority broadcast in system information on NR Cell 1. f_NR5GC_CellInfo_SetSIB5_CellReselectionPriority(nr_Cell1, 0, 5); // Table 6.2.3.4.3.3-7: SIB5 NR Cell 1 f_NR_CellInfo_SetSIB5_ThreshXHighQ(nr_Cell1, 0, cs_InterFreqCarrierFreqInfo_ThreshX_Q(5, 20)); f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); // @siclog "Step 8" siclog@ // Notify UE change of System Information on NR Cell 1. // Table 6.2.3.4.3.3-8: SIB1 of NR Cell 1 f_NR_IncrementValueTag(nr_Cell1, sibType5); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 9" siclog@ // Wait for 6 s for UE to receive system information. f_Delay(6.0); // @siclog "Step 10" siclog@ // The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T3" in table 6.2.3.4.3.2-1/2/3. f_NR_SetCellPower (nr_Cell1, -95, -95); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); // send coordination to move context to EUTRA // @siclog "Step 11" siclog@ // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); }

After Change

function fl_TC_6_2_3_4_TestBody() runs on NR5GC_PTC { var integer v_Noc := -85; // @sic R5s200723 sic@ //@siclog "Step 1" siclog@ //The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T1" in table 6.2.3.4.3.2-1/2/3. f_NR_SetCellPower (nr_Cell1, -95, -95); // FR2 FFS f_NR_ConfigureAndActivateVNG_MultipleCells({cs_CellNoiseLevel(nr_Cell1, v_Noc)}); // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA); // wait until power level change on Step4 //@siclog "Step 4" siclog@ // The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T2" in table 6.2.3.4.3.2-1/2/3. f_NR_SetCellPower (nr_Cell1, -80, -80); // @siclog "Step 5" siclog@ // The UE selects NR Cell 1 and performs the generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 with condition 'connected without release'. f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, noRrcConnectionRelease, false); // @siclog "Step 6" siclog@ // The SS transmits an RRCRelease message on NR Cell 1. f_NR_RRCRelease(nr_Cell1); // @sic R5s210276 sic@ // @siclog "Step 7" siclog@ // The SS changes the E-UTRA cell priority broadcast in system information on NR Cell 1. f_NR5GC_CellInfo_SetSIB5_CellReselectionPriority(nr_Cell1, 0, 5); // Table 6.2.3.4.3.3-7: SIB5 NR Cell 1 f_NR_CellInfo_SetSIB5_ThreshXHighQ(nr_Cell1, 0, cs_InterFreqCarrierFreqInfo_ThreshX_Q(12, 20)); f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); // @siclog "Step 8" siclog@ // Notify UE change of System Information on NR Cell 1. // Table 6.2.3.4.3.3-8: SIB1 of NR Cell 1 f_NR_IncrementValueTag(nr_Cell1, sibType5); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 9" siclog@ // Wait for 6 s for UE to receive system information. f_Delay(6.0); // @siclog "Step 10" siclog@ // The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T3" in table 6.2.3.4.3.2-1/2/3. f_NR_SetCellPower (nr_Cell1, -95, -95); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); // send coordination to move context to EUTRA // @siclog "Step 11" siclog@ // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); }