3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210480

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1700 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to functions f_NR5GC_ModifyPDUSession_AddVoice and f_NR5GC_ModifyPDUSession_RemoveIMSVoice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of the functions f_NR5GC_ModifyPDUSession_AddVoice and f_NR5GC_ModifyPDUSession_RemoveIMSVoice, a runtime error is observed when the IMS testcase 7.5 is run with a UE that is configured in NR only mode. During the initial PDU Session Establishment, the IE PdnIndex is assigned based on following check ( in function f_NR5GC_PDUSessionEstablishment_InnerLoop) : if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@ if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo)); v_GSM_MobilityInfo.PdnIndex := v_PDN_Index; } } Thus, if the UE does not set the s1 Capability, then the variable v_GSM_MobilityInfo.PdnIndex remains un-initialized. This causes runtime errors when function f_NR5GC_ModifyPDUSession_AddVoice is called. In this function, when function f_IP_RtpRtcpLoopback is called, the second parameter is uninitialized and causes runtime error. Similarly the second parameter in f_NR5GC_ModifyPDUSession_RemoveIMSVoice f_IP_RtpRtcpLoopback is uninitialized causing runtime error. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented a isvalue check for PdnIndex and based on that initialized the parameter setting for v_GSM_MobilityInfo.PdnIndex and p_GSM_MobilityInfo.PdnIndex. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase unfairly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections	4

1.1	Correction to f_NR5GC_ModifyPDUSession_AddVoice	4

1.2	Correction to f_NR5GC_ModifyPDUSession_RemoveIMSVoice	7

# Corrections

## Correction to f_NR5GC_ModifyPDUSession_AddVoice

| Template name | f_NR5GC_ModifyPDUSession_AddVoice |
| --- | --- |
| Reason for change | In the current TTCN implementation of the functions f_NR5GC_ModifyPDUSession_AddVoice and f_NR5GC_ModifyPDUSession_RemoveIMSVoice, a runtime error is observed when the IMS testcase 7.5 is run with a UE that is configured in NR only mode. During the initial PDU Session Establishment, the IE PdnIndex is assigned based on following check ( in function f_NR5GC_PDUSessionEstablishment_InnerLoop) : if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@ if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo)); v_GSM_MobilityInfo.PdnIndex := v_PDN_Index; } } Thus, if the UE does not set the s1 Capability, then the variable v_GSM_MobilityInfo.PdnIndex remains un-initialized. This causes runtime errors when function f_NR5GC_ModifyPDUSession_AddVoice is called. In this function, when function f_IP_RtpRtcpLoopback is called, the second parameter is uninitialized and causes runtime error. This needs to be corrected. |
| Summary of change | Implemented a isvalue check for PdnIndex and based on that initialized the parameter setting for p_GSM_MobilityInfo.PdnIndex. Please see below. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | Accepted in principle, but just moved the assignment of v_GSM_MobilityInfo.PdnIndex := v_PDN_Index to outside the if statement in f_NR5GC_PDUSessionEstablishment_InnerLoop – so this step is always done, regardless of whether the UE supports S1 |

Before Change:

function f_NR5GC_ModifyPDUSession_AddVoice(NR_CellId_Type p_NR_CellId, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_InterworkWithEPS := false; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var integer v_NewDRBId; var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId); var template (value) QosFlow_Identification_Type v_QosFlowId; if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(p_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules("7_Voice", '7A'O), // @sic R5s210073 sic@ cs_QoSFlowDescr({cs_QoS_Flow5(v_InterworkWithEPS)})); v_NewDRBId := f_NR_GetNextDRBId(p_GSM_MobilityInfo, false); // @sic R5-211171 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7})), // QFI of 7 in flow 5 v_NewDRBId, -, -, cs_38508_PDCP_Config (-, -, -, -, omit))}); // UM v_SS_Config := {f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_NewDRBId, -, -, f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7}))))}; // @sic R5-211371, R5s210330 sic@ v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB(p_NR_CellId, {v_NewDRBId}, false); v_CellGroupConfig.rlc_BearerToAddModList := {cs_38508_RLC_BearerConfig_DRB(v_NewDRBId, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_38508_LogicalChannelConfig_DRB)}; v_QosFlowId := cs_IP_QosFlowId(oct2int(p_GSM_MobilityInfo.SessionId), 7); // @sic R5s210363 sic@ f_IP_RtpRtcpLoopback(IP, p_GSM_MobilityInfo.PdnIndex, tsc_IMS_MediaPort_M1, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s210363 sic@ // then send RRC Reconfiguration message f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend)}, -, -, false); // @sic R5s210387 sic@ // The UE transmit an RRCReconfigurationComplete message @sic R5s210387 sic@ SRB.receive(car_NR_SRB_RrcPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Establishment"); } }

After Change:

function f_NR5GC_ModifyPDUSession_AddVoice(NR_CellId_Type p_NR_CellId, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_InterworkWithEPS := false; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var integer v_NewDRBId; var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId); var template (value) QosFlow_Identification_Type v_QosFlowId; var PDN_Index_Type v_PdnIndex := PDN_1; //WA#IMS_Voice if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; } if (isvalue(p_GSM_MobilityInfo.PdnIndex)) { //WA#IMS_Voice v_PdnIndex := p_GSM_MobilityInfo.PdnIndex; } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(p_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules("7_Voice", '7A'O), // @sic R5s210073 sic@ cs_QoSFlowDescr({cs_QoS_Flow5(v_InterworkWithEPS)})); v_NewDRBId := f_NR_GetNextDRBId(p_GSM_MobilityInfo, false); // @sic R5-211171 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7})), // QFI of 7 in flow 5 v_NewDRBId, -, -, cs_38508_PDCP_Config (-, -, -, -, omit))}); // UM v_SS_Config := {f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_NewDRBId, -, -, f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7}))))}; // @sic R5-211371, R5s210330 sic@ v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB(p_NR_CellId, {v_NewDRBId}, false); v_CellGroupConfig.rlc_BearerToAddModList := {cs_38508_RLC_BearerConfig_DRB(v_NewDRBId, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_38508_LogicalChannelConfig_DRB)}; v_QosFlowId := cs_IP_QosFlowId(oct2int(p_GSM_MobilityInfo.SessionId), 7); // @sic R5s210363 sic@ f_IP_RtpRtcpLoopback(IP, v_PdnIndex /*WA#IMS_Voice p_GSM_MobilityInfo.PdnIndex */, tsc_IMS_MediaPort_M1, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s210363 sic@ // then send RRC Reconfiguration message f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }); SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Establishment"); } }

MCC160 Implementation:

v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo);

v_GSM_MobilityInfo.PdnIndex := v_PDN_Index;

// @sic R5-204400 sic@

if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@

if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) {

v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo));

v_GSM_MobilityInfo.PdnIndex := v_PDN_Index;

}

}

## Correction to f_NR5GC_ModifyPDUSession_RemoveIMSVoice

| Template name | f_NR5GC_ModifyPDUSession_RemoveIMSVoice |
| --- | --- |
| Reason for change | In the current TTCN implementation of the functions f_NR5GC_ModifyPDUSession_AddVoice and f_NR5GC_ModifyPDUSession_RemoveIMSVoice, a runtime error is observed when the IMS testcase 7.5 is run with a UE that is configured in NR only mode. During the initial PDU Session Establishment, the IE PdnIndex is assigned based on following check ( in function f_NR5GC_PDUSessionEstablishment_InnerLoop) : if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@ if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo)); v_GSM_MobilityInfo.PdnIndex := v_PDN_Index; } } Thus, if the UE does not set the s1 Capability, then the variable v_GSM_MobilityInfo.PdnIndex remains un-initialized. This causes runtime errors when function f_ NR5GC_ModifyPDUSession_RemoveIMSVoice is called. In this function, when function f_IP_RtpRtcpLoopback is called, the second parameter is uninitialized and causes runtime error. This needs to be corrected. |
| Summary of change | Implemented a isvalue check for PdnIndex and based on that initialized the parameter setting for v_GSM_MobilityInfo.PdnIndex. Please see below. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | Not needed – see above implementation |

Before Change:

function f_NR5GC_ModifyPDUSession_RemoveIMSVoice(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_InterworkWithEPS := false; var O1_Type v_EPSBearer := '00'O; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var template (value) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForIMSPDUSession(); var template (value) NR_RLC_BearerToReleaseList_Type v_RLC_BearerToReleaseList; var NR_DRBInfo_Type v_DRBInfo; var template (value) DRB_ToReleaseList v_DRB_ToReleaseList; var integer i; var integer j := 0; if (isvalue(v_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; v_EPSBearer := int2oct(valueof(v_GSM_MobilityInfo.EPS_Bearer), 1); } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules("2", '7A'O), // @sic R5s210073 sic@ cs_QoSFlowDescr({cs_QoS_Flow2(v_InterworkWithEPS, v_EPSBearer)})); // @sic R5-211171 sic@ for (i := 0; i < lengthof(v_DRBIdList); i := i+1) { v_DRBInfo := valueof(f_NR_GetDRBInfo_FromId(v_DRBIdList[i])); if (not (v_DRBInfo.DefaultDRB)) { v_DRB_ToReleaseList[j] := v_DRBInfo.DRB_Id; v_RLC_BearerToReleaseList[j] := v_DRBInfo.RLC_Config.logicalChannelIdentity; v_SS_Config[j] := cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBInfo.DRB_Id)); j := j+1; } } v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, v_DRB_ToReleaseList); v_CellGroupConfig := cs_38508_CellGroupConfig(f_NR_CellInfo_Get_CellGroupId(p_NR_CellId), omit, v_RLC_BearerToReleaseList); // The SS transmits an RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION MODIFICATION COMMAND f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }, -, -, false); // @sic R5s210387 sic@ // The UE transmit an RRCReconfigurationComplete message @sic R5s210387 sic@ SRB.receive(car_NR_SRB_RrcPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Release"); } f_IP_RtpRtcpLoopback(IP, valueof(v_GSM_MobilityInfo.PdnIndex), tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ }

After Change:

function f_NR5GC_ModifyPDUSession_RemoveIMSVoice(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_InterworkWithEPS := false; var O1_Type v_EPSBearer := '00'O; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var template (value) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForIMSPDUSession(); var template (value) NR_RLC_BearerToReleaseList_Type v_RLC_BearerToReleaseList; var NR_DRBInfo_Type v_DRBInfo; var template (value) DRB_ToReleaseList v_DRB_ToReleaseList; var integer i; var integer j := 0; var PDN_Index_Type v_PdnIndex := PDN_1; //WA#IMS_Voice if (isvalue(v_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; v_EPSBearer := int2oct(valueof(v_GSM_MobilityInfo.EPS_Bearer), 1); } if (isvalue(v_GSM_MobilityInfo.PdnIndex)) { //WA#IMS_Voice v_PdnIndex := valueof(v_GSM_MobilityInfo.PdnIndex); } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules("2", '7A'O, true /*WA#7_5*/), // @sic R5s210073 sic@ cs_QoSFlowDescr({cs_QoS_Flow2(v_InterworkWithEPS, v_EPSBearer, true /*WA#7_5*/)})); // @sic R5-211171 sic@ for (i := 0; i < lengthof(v_DRBIdList); i := i+1) { v_DRBInfo := valueof(f_NR_GetDRBInfo_FromId(v_DRBIdList[i])); if (not (v_DRBInfo.DefaultDRB)) { v_DRB_ToReleaseList[j] := v_DRBInfo.DRB_Id; v_RLC_BearerToReleaseList[j] := v_DRBInfo.RLC_Config.logicalChannelIdentity; v_SS_Config[j] := cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBInfo.DRB_Id)); j := j+1; } } v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, v_DRB_ToReleaseList); v_CellGroupConfig := cs_38508_CellGroupConfig(f_NR_CellInfo_Get_CellGroupId(p_NR_CellId), omit, v_RLC_BearerToReleaseList); // The SS transmits an RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION MODIFICATION COMMAND f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }); SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Release"); } f_IP_RtpRtcpLoopback(IP, v_PdnIndex /*WA#IMS_Voice valueof(v_GSM_MobilityInfo.PdnIndex) */, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ }