3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210524

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1719 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR idle mode test case 6.1.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step 5 the RRCSetupRequest can come any time after the power level change of NR cell 1. With the current implementation the test case can fail if the RRCSetupRequest is received while the SS configuration for VNG is still ongoing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the power level change for time T2 to after SS configuration for VNG is competed, to increase test case stability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_1_2_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | At step 5 the RRCSetupRequest can come any time after the power level change of NR cell 1. With the current implementation the test case can fail if the RRCSetupRequest is received while the SS configuration for VNG is still ongoing. |
| Summary of change | Move the power level change for time T2 to after SS configuration for VNG is competed, to increase test case stability. |
| TTCN module | Idle_CellSelection_NR5GC |
| MCC160 Comment | Accepted. Same correction done for NR5GC IRAT test case 6.2.3.2 on Step 1 |

Before Change

function f_TC_6_1_2_2_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-206318 sic@ ……………… v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); // @siclog "Step 4" siclog@ // SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 level according to row "T2" in table 6.1.2.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); if (not v_IsFR1) {// FR2: q-QualMin on SIB1 and SIB2 are FFS for FR2 v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); // @sic R5s200969 sic@ v_Q_RxLevMin := f_NR_CalculateQ_RxLevMin(-55.0, v_DeltaPrimaryBand.DeltaNRf1); } f_NR_CellInfo_SetSIB1_CellSelectionInfo(nr_Cell11, cs_38508_SIB1_CellSelectionInfo(v_Q_RxLevMin,-,-12)); f_NR_CellInfo_SetSIB2_Q_QualMin(nr_Cell11, -12); f_NR_ConfigureAndActivateVNG_MultipleCells({cs_CellNoiseLevel(nr_Cell11, v_Noc)}); // @siclog "Step 5" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1? f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.2.2 Step 5"); ……………. }

After Change

function f_TC_6_1_2_2_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-206318 sic@ ………….. v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); // @siclog "Step 4" siclog@ // SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 level according to row "T2" in table 6.1.2.2.3.2-1/2. //MOVED f_NR_SetCellPowerList(v_CellPowerList_AtT2); if (not v_IsFR1) {// FR2: q-QualMin on SIB1 and SIB2 are FFS for FR2 v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); // @sic R5s200969 sic@ v_Q_RxLevMin := f_NR_CalculateQ_RxLevMin(-55.0, v_DeltaPrimaryBand.DeltaNRf1); } f_NR_CellInfo_SetSIB1_CellSelectionInfo(nr_Cell11, cs_38508_SIB1_CellSelectionInfo(v_Q_RxLevMin,-,-12)); f_NR_CellInfo_SetSIB2_Q_QualMin(nr_Cell11, -12); f_NR_ConfigureAndActivateVNG_MultipleCells({cs_CellNoiseLevel(nr_Cell11, v_Noc)}); f_NR_SetCellPowerList(v_CellPowerList_AtT2); // @siclog "Step 5" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1? f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.2.2 Step 5"); ……………… }