3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210610

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4615 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE PSM TC 9.2.3.1.5b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test, MTCe-UEPCOP-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During the initialization of the TC 9.2.3.1.5b, UE needs to be switched OFF after configuring the PSM, but in TTCN it is currently using SwitchON function instead. Without this change, test does not fail but switchOFF added for logical completeness. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaced the function f_UT_SwitchOnUE with f_UT_SwitchOffUE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case implementation will not be accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.1.5b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3- B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_9_2_3_1_5b_EUTRA |
| --- | --- |
| Reason for change | During the initialization of the TC 9.2.3.1.5b, UE needs to be switched OFF after configuring the PSM, but in TTCN it is currently using SwitchON function instead. Without this change, test does not fail but switchOFF added for logical completeness. |
| Summary of change | Replaced the function f_UT_SwitchOnUE with f_UT_SwitchOffUE |
| TTCN module | LTE_A_R12\9_2\NAS_Attach_EOnly_R12.ttcn |
| MCC160 Comment | Accepted |

Before change:

function f_TC_9_2_3_1_5b_EUTRA() runs on EUTRA_PTC

{ /* Periodic tracking area update / Accepted / PSM / T3412 Extended Value */

//  create cell and send out system information

f_EUTRA_NAS_Init(LTE_AllCellsOnSamePLMN, c1);

f_UT_SwitchOnUE(UT, false, CNF_REQUIRED); // @sic R5s180153 sic@

f_UT_ConfigurePSM(UT, omit, omit, omit, true, CNF_REQUIRED); // @sic R5s170325, R5-176154, R5s180154 sic@

f_UT_SwitchOnUE(UT, false, CNF_REQUIRED); // @sic R5s180153 sic@

// create cell and send out system information

f_EUTRA_CellConfig_Def(eutra_CellA);

f_UT_ConfigureEPSAttach(UT);

//Preamble: UE is in state Registered, Idle Mode (state 2) on Cell A

f_EUTRA_Preamble(eutra_CellA, STATE2_IDLEUPDATE, EPS_Only);

f_EUTRA_TestBody_Set(true);

fl_TC_9_2_3_1_5b_Body();

// Postamble

f_EUTRA_TestBody_Set(false);

f_EUTRA_Postamble(eutra_CellA, E2_T3440, EPS_Only); //@sic R5s160882 Ch. 2  - R5s180256 sic@

}

After change:

function f_TC_9_2_3_1_5b_EUTRA() runs on EUTRA_PTC

{ /* Periodic tracking area update / Accepted / PSM / T3412 Extended Value */

//  create cell and send out system information

f_EUTRA_NAS_Init(LTE_AllCellsOnSamePLMN, c1);

f_UT_SwitchOnUE(UT, false, CNF_REQUIRED); // @sic R5s180153 sic@

f_UT_ConfigurePSM(UT, omit, omit, omit, true, CNF_REQUIRED); // @sic R5s170325, R5-176154, R5s180154 sic@

f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); // @sic R5s180153 sic@

// create cell and send out system information

f_EUTRA_CellConfig_Def(eutra_CellA);

f_UT_ConfigureEPSAttach(UT);

//Preamble: UE is in state Registered, Idle Mode (state 2) on Cell A

f_EUTRA_Preamble(eutra_CellA, STATE2_IDLEUPDATE, EPS_Only);

f_EUTRA_TestBody_Set(true);

fl_TC_9_2_3_1_5b_Body();

// Postamble

f_EUTRA_TestBody_Set(false);

f_EUTRA_Postamble(eutra_CellA, E2_T3440, EPS_Only); //@sic R5s160882 Ch. 2  - R5s180256 sic@

}