3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210618

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1756 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 11.3.2 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-16 NR5GC test case 11.3.2 to the NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 11.3.2 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	8

2.3	Change 3	9

3	Branches executed	9

4	Execution Log Files	9

4.1	HiSilicon Balong 5000	9

5	References	10

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.3.2 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Pengchong Shen
	shenpengchong@datangmobile.cn

## Verification Test Summary

Test Case: 	11.3.2

ATS Version:	iwd-TTCN3-B2020-09_D21wk12

System Simulator used:	Datang LinkTester 5G Protocol Conformance Test System ECT9610

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | fl_TC_11_3_2_TestBody |
| --- | --- |
| Reason for change | - After emergency call is tried at step 1, if UE supports other RATs it may fallback to place the emergency call to EUTRA or other CS RATs. The procedures at step 3 and step 4 which expects the UE to remain camped on NR cell may fail the TC. Also, if the UE has moved to other RAT it will have moved to 5GMM-DERGISTERED and would want to register again. To avoid handling optional behavior, it is better to switch off the UE after step2. This condition is missed in current TTCN. - UE may retry the emergency call after RRCRelease at step 21. This condition is missed in current TTCN. Note: it’s changed with the prose CR R5-213505. |
| Summary of change | - Added steps to switch off the UE and Registration handling after switch on. - Adding steps to complete the emergency call. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle, but implemented differently |

Before Change

function fl_TC_11_3_2_TestBody () runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var RejectWaitTime v_RejectWaitTime := 16; var float v_WaitForConnectionMin := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_RejectWaitTime)); var float v_TimerValue := 30.0; timer t_WaitTime; //@siclog "Step 1" siclog@ // Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1) f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' within 30 s'? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_TimerValue, cr_NR_EstablishmentCause_emergency, "Step 2"); //@siclog "Step 3" siclog@ //SS changes SIB1 according to Table 11.3.2.3.3-2 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect and T390 expire. (Note 1) // Table 11.3.2.3.3-2: SIB1 for NR Cell 1 f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_7Entry, //@sic R5-205321 sic@ -, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 4" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 5" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'mt-access' ? f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); //@siclog "Steps 6-12" siclog@ // Steps 3 to 9a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0010'B)) { // @sic R5s200654 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); f_NR_PreliminaryPass(__FILE__, __LINE__, "Check 5GC RRC_IDLE"); f_NR_RRCRelease(nr_Cell1); //@siclog "Step 13" siclog@ // SS changes SIB1 according to 38.508-1 [4] Table 4.6.1-28 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Steps 14-15" siclog@ //Steps 1-2 of the NR RRC_CONNECTED procedure in table 4.5.4.2-3 in TS 38.508-1[4] are performed. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 16" siclog@ //The SS responds with RRCReject message with IE waitTime set to 10s. SRB.send(cas_NR_SRB0_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCReject(v_RejectWaitTime))); t_WaitTime.start(v_WaitForConnectionMin); //@siclog "Step 17" siclog@ // Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1) //NOTE 1: This could be done by e.g. MMI or AT command. f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 18" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' (Note 2)? if (f_NR_RRC_SetupRequest_Common(nr_Cell1, t_WaitTime, cr_NR_EstablishmentCause_emergency)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18 PASS"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18 Failed"); } //@siclog "Step 19" siclog@ // The SS transmits an RRCSetup message. f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Step 20" siclog@ // The UE transmits an RRCSetupComplete messsage. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); //@siclog "Step 21" siclog@ //SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. f_NR_RRCRelease(nr_Cell1); }

After Change

function fl_TC_11_3_2_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var RejectWaitTime v_RejectWaitTime := 16; var float v_WaitForConnectionMin := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_RejectWaitTime)); var float v_TimerValue := 30.0; timer t_WaitTime; var NR_SRB_COMMON_IND v_ReceivedAsp; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_s; var ProcedureTransactionIdentifier v_PTI := '00'O; var NG_NAS_UL_Message_Type v_GSMMsg; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); //@siclog "Step 1" siclog@ // Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1) f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' within 30 s'? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_TimerValue, cr_NR_EstablishmentCause_emergency, "Step 2"); //@siclog "Step 2A1" siclog@ //IF pc_SwitchOnOff THEN switch off UE, ELSE IF pc_USIM_Removal THEN remove the USIM, ELSE power off UE in which case steps 2A2 to 2A4 are not performed. if(pc_SwitchOnOff or pc_SwitchOnOff){ if (pc_SwitchOnOff) { // switch off the UE fl_UT_ApplyCommand(UT, cas_UT_Req(SWITCH_OFF, NO_CNF_REQUIRED)); // @sic R5s110395 sic@ } else if (pc_USIM_Removal) { // UE doesn't support switch off, but does support SIM removal, so remove it; @sic R5-104083 sic@ @sic R5s120597r1 sic@ fl_UT_ApplyCommand(UT, cas_UT_Req(REMOVE_USIM, CNF_REQUIRED));} //@siclog "Step 2A2" siclog@ t_WaitTime.start(5.0); alt{ //@siclog "Step 2A3a1" siclog@ []SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_WaitTime.stop; if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } } //@siclog "Step 2A3b1" siclog@ [] t_WaitTime.timeout{ //@siclog "Step 2A4" siclog@ f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1), tsc_CnfReq); } } } else { f_UT_PowerOffUE(UT); } //@siclog "Step 2A5" siclog@ f_NR5GC_MobileInfo_SetDRBInfoList({}); //Switch on the UE //The UE performs Registration procedure as specified in TS 38.508-1 [4] subclause 4.5.2. f_UT_SwitchOnUE(UT, false); f_NR5GC_RRC_IdleState1N_Def(nr_Cell1); //@siclog "Step 3" siclog@ //SS changes SIB1 according to Table 11.3.2.3.3-2 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect and T390 expire. (Note 1) // Table 11.3.2.3.3-2: SIB1 for NR Cell 1 f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_7Entry, //@sic R5-205321 sic@ -, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 4" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 5" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'mt-access' ? f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); //@siclog "Steps 6-12" siclog@ // Steps 3 to 9a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0010'B)) { // @sic R5s200654 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); f_NR_PreliminaryPass(__FILE__, __LINE__, "Check 5GC RRC_IDLE"); f_NR_RRCRelease(nr_Cell1); //@siclog "Step 13" siclog@ // SS changes SIB1 according to 38.508-1 [4] Table 4.6.1-28 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Steps 14-15" siclog@ //Steps 1-2 of the NR RRC_CONNECTED procedure in table 4.5.4.2-3 in TS 38.508-1[4] are performed. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 16" siclog@ //The SS responds with RRCReject message with IE waitTime set to 10s. SRB.send(cas_NR_SRB0_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCReject(v_RejectWaitTime))); t_WaitTime.start(v_WaitForConnectionMin); //@siclog "Step 17" siclog@ // Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1) //NOTE 1: This could be done by e.g. MMI or AT command. f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 18" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' (Note 2)? if (f_NR_RRC_SetupRequest_Common(nr_Cell1, t_WaitTime, cr_NR_EstablishmentCause_emergency)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18 PASS"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18 Failed"); } //@siclog "Step 21-32" siclog@ //Step 2-13 for IMS Emergency call establishment are performed as specified in TS 38.508-1 [4], subclause 4.9.11 on NR Cell 1. f_NR_RRCSetup_Def(nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0011'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // Steps 4-7 Configure SRB2 & DRB f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); // Steps 8-13 f_NR5GC_EmergencyCallPDUSession(nr_Cell1); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); //@siclog "Step 21" siclog@ //SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. f_NR_RRCRelease(nr_Cell1); }

## Change 2

| Function name | TC_11_3_2_NR5GC |
| --- | --- |
| Reason for change | - Add IMS2 for emergency procedure. |
| Summary of change | - Create IMS2 and set alive, add new function f_TC_11_3_2_NR5GC_IMS2. |
| TTCN module | NR5GC_Testsuite.ttcn |
| MCC160 Comment | Accepted |

Before change

testcase TC_11_3_2_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // UAC / Access Identity 0 / 0% access probability / Paging for MT Access/Emergency Call var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_3_2_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After  change

testcase TC_11_3_2_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // UAC / Access Identity 0 / 0% access probability / Paging for MT Access/Emergency Call var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_3_2_NR5GC()); v_IMS2.start(f_TC_11_3_2_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

## Change 3

| Function name | f_TC_11_3_2_NR5GC_IMS2 |
| --- | --- |
| Reason for change | - Add new function for change 2. |
| Summary of change | - Add new function for change 2. |
| TTCN module | Emergency_NR5GC_IMS.ttcn |
| MCC160 Comment | Accepted |

New function

function f_TC_11_3_2_NR5GC_IMS2()runs on IMS_PTC { / // The test case currently sets up 1 IMS Emg Call in Normal service f_IMS_PTC_Init(PDN_2); fl_IMS_EmergencyCallAndRelease(); }

MCC160 Implementation

function fl_TC_11_3_2_TestBody() runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1);

var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams);

var RejectWaitTime v_RejectWaitTime := 16;

var float v_WaitForConnectionMin := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_RejectWaitTime));

var float v_TimerValue := 30.0;

timer t_WaitTime;

//@siclog "Step 1" siclog@

// Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1)

f_UT_RequestIMSEmergencyCall (UT);

//@siclog "Step 2" siclog@

//Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' within 30 s'?

f_NR_CheckNoRRCSetupReq(nr_Cell1, v_TimerValue, cr_NR_EstablishmentCause_emergency, "Step 2");

//@sic R5-213505 sic@

//@siclog "Step 2A1" siclog@

//IF pc_SwitchOnOff THEN switch off UE, ELSE IF pc_USIM_Removal THEN remove the USIM, ELSE power off UE in which case steps 2A2 to 2A4 are not performed

if(pc_SwitchOnOff){

f_UT_SwitchOffUE(UT, true);

//@siclog "Step 2A2" siclog@

//SS starts timer1 = 5 sec to see if UE performs optional Step 10a3a1 (Note 5)

t_WaitTime.start(5.0);

//@siclog "Steps 2A3a1-2A4" siclog@

//The UE transmits a DEREGISTRATION REQUEST message. Stop timer T_Delay

//Timer T_Delay expires. SS locally releases RRC connection

alt {

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1,

tsc_NR_RbId_SRB1,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp{

t_WaitTime.stop;

if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed");

}

f_NR_RRCRelease(nr_Cell1);

}

[] t_WaitTime.timeout {

f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1), tsc_CnfReq);

}

}

}

//@siclog "Step 2A5" siclog@

//ELSE power off UE

else {

f_UT_PowerOffUE (UT);

}

//@siclog "Step 2A6" siclog@

//The UE performs Registration procedure as specified in TS 38.508-1 [4] subclause 4.5.2.

//@siclog "Step 2A5" siclog@

//Switch on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Step 2A6" siclog@

f_NR5GC_RRC_IdleState1N_Def(nr_Cell1);

//@siclog "Step 3" siclog@

//SS changes SIB1 according to Table 11.3.2.3.3-2 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect and T390 expire. (Note 1)

// Table 11.3.2.3.3-2: SIB1 for NR Cell 1

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1,

cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_7Entry, //@sic R5-205321 sic@

-, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B)));

f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS

f_NR_WaitModificationPeriods(nr_Cell1);

//@siclog "Step 4" siclog@

//The SS transmits a Paging message including a matched ng-5G-S-TMSI.

f_NR_Paging_Def(nr_Cell1, RRC_IDLE);

//@siclog "Step 5" siclog@

//Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'mt-access' ?

f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

//@siclog "Steps 6-12" siclog@

// Steps 3 to 9a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 are performed on NR Cell 1.

f_NR_RRCSetup_Def (nr_Cell1);

// receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH):

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def),

tsc_SHT_IntegrityProtected);

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0010'B)) {  // @sic R5s200654 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Check 5GC RRC_IDLE");

f_NR_RRCRelease(nr_Cell1);

//@siclog "Step 13" siclog@

// SS changes SIB1 according to 38.508-1 [4] Table 4.6.1-28 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect.

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit);

f_NR_ModifySysinfo(nr_Cell1);

f_NR_WaitModificationPeriods(nr_Cell1);

//@siclog "Steps 14-15" siclog@

//Steps 1-2 of the NR RRC_CONNECTED procedure in table 4.5.4.2-3 in TS 38.508-1[4] are performed.

f_NR_Paging_Def(nr_Cell1, RRC_IDLE);

f_NR_RRC_SetupRequest_Def(nr_Cell1);

//@siclog "Step 16" siclog@

//The SS responds with RRCReject message with IE waitTime set to 10s.

SRB.send(cas_NR_SRB0_RrcPdu_REQ(nr_Cell1,

-,

cs_38508_RRCReject(v_RejectWaitTime)));

t_WaitTime.start(v_WaitForConnectionMin);

//@siclog "Step 17" siclog@

// Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1)

//NOTE 1: This could be done by e.g. MMI or AT command.

f_UT_RequestIMSEmergencyCall (UT);

//@sic R5-213505 sic@

//@siclog "Step 18 -32" siclog@

//Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' (Note 2)?

//Step 2-13 for IMS Emergency call establishment are performed as specified in TS 38.508-1 [4], subclause 4.9.11 on NR Cell 1.

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, emergency);

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0011'B)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

else{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18");

}

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg);

// Steps 8-13

f_NR5GC_EmergencyCallPDUSession(nr_Cell1);

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);

f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established");

//@siclog "Step 33" siclog@

//SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE.

f_NR_RRCRelease(nr_Cell1);

}

# Branches executed

NR Band 41

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on Datang LinkTester 5G Protocol Conformance Test System ECT9610. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:
\ TC_11_3_2_NR5GC_N41.log

- PIXCIT settings used:
\ Pics(Hisilicon-IRAT).xml

\ Pixit(TC_11_3_2_NR5GC).xml

# References

| [1] | R5s210619: Supporting information for agreement of NR5GC test case 11.3.2 in FR1 |
| --- | --- |