3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210620

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1757 | rev | - | Current version: | 16.1.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 8.1.5.8.2.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of NR5GC test case 8.1.5.8.2.1 in FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 8.1.5.8.2.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Overview	3

2 Corrections required	3

2.1 Change 1	3

3 Branches executed	6

4 Execution Log Files	6

4.1 HiSilicon Balong 5000	6

5 References	6

# Overview

This document lists all the changes needed to correct issues in the ATS NR5GC_IWD_21wk12  related to the title of this CR.

Contact:	Ting Gao
gaoting@starpointcomm.com

# Corrections requireds

## Change 1

| Function name | f_TC_8_1_5_8_2_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | In function f_NR5GC_Rel_1SCell at step4-5，SCell is released locally in SS.The configuration need to add again to consistent with the RRCResume message in step6a5. |
| Summary of change | Config SS to add SCell. |
| TTCN module | RRC_Others_CA_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before change

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type     p_PCell,

NR_CellId_Type     p_SCell) runs on NR5GC_PTC

{

var template(value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_SCellIndex_1);

var template(value) CellGroupConfig v_SCGConfig;

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfigCA)));

var template(value) RadioBearerConfig v_RadioBearerConfig;

var template(omit) DRB_ToAddModList v_DRB_ToAddModList := omit;

var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList();

var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(p_PCell);

var integer i;

// @siclog "Steps 1-3" siclog@

// The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3)

// The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH.

// Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3)

f_NR5GC_RRC_ProcedureDelay(p_PCell,

cas_NR_RrcNasPduList_REQ(p_PCell, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext))),

car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete),

16,   //converted in slots inside the function

"Step 3");

f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_Now, f_NR_CellInfo_GetRNTI(p_PCell));

f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_Now);

// @siclog "Steps 4-5" siclog@

//The SS transmits an RRCReconfiguration message to release SCell(Cell 3)

//UE transmits an RRCReconfigurationComplete message

f_NR5GC_Rel_1SCell(p_PCell, p_SCell);

if ( pc_inactiveState ) {

// @siclog "Step6a1" siclog@

// IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig.

f_NR_RRC_InactiveDef(p_PCell);

// @siclog "Step6a2" siclog@

//Wait 1s to let UE enter RRC_INACTIVE state.

f_Delay(1.0);

// @siclog "Step6a2" siclog@

// The SS transmits a Paging message including a matched ng-5G-S-TMSI.

f_NR_Paging_Def(p_PCell, RRC_INACTIVE);

// @siclog "Step6a3" siclog@

// The UE transmits an RRCResumeRequest message.

f_NR_RRC_ResumeRequest_Def(p_PCell);

// The SS transmits an RRCResume message with sCellToAddModList to add SCell(Cell 3)

// The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 10 to schedule PUSCH.  (Note 1, Note 2)

// Check: Does the UE transmit an RRCResumeComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 9? (Note 1, Note 2, Note 3)

v_SCGConfig := f_NR_GetSCGConfigResume(p_PCell);                          // condition RESUME

v_SCGConfig.mac_CellGroupConfig := v_SCGConfigCA.mac_CellGroupConfig;  // condition SCell_Add

v_SCGConfig.sCellToAddModList  := v_SCGConfigCA.sCellToAddModList;        // condition SCell_Add

for (i:=0; i < lengthof(v_ExistingDRBs); i:=i+1) {

v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit);

}

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList);

f_NR5GC_RRC_ProcedureDelay(p_PCell,

cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, bit2oct(encvalue(v_SCGConfig)), v_RadioBearerConfig)),

car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCResumeComplete),

16,                  //converted in slots inside the function

"Step 6a7");

//DRBs are not reconfigured in SS: that is ok as they are not used

f_NR_SS_RachProcedureConfig(p_PCell, v_NR_RachProcedureConfig);

}

}

After change

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type     p_PCell,

NR_CellId_Type     p_SCell) runs on NR5GC_PTC

{

var template(value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_SCellIndex_1);

var template(value) CellGroupConfig v_SCGConfig;

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfigCA)));

var template(value) RadioBearerConfig v_RadioBearerConfig;

var template(omit) DRB_ToAddModList v_DRB_ToAddModList := omit;

var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList();

var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(p_PCell);

var integer i;

// @siclog "Steps 1-3" siclog@

// The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3)

// The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH.

// Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3)

f_NR5GC_RRC_ProcedureDelay(p_PCell,

cas_NR_RrcNasPduList_REQ(p_PCell, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext))),

car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete),

16,   //converted in slots inside the function

"Step 3");

f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_Now, f_NR_CellInfo_GetRNTI(p_PCell));

f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_Now);

// @siclog "Steps 4-5" siclog@

//The SS transmits an RRCReconfiguration message to release SCell(Cell 3)

//UE transmits an RRCReconfigurationComplete message

f_NR5GC_Rel_1SCell(p_PCell, p_SCell);

if ( pc_inactiveState ) {

// @siclog "Step6a1" siclog@

// IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig.

f_NR_RRC_InactiveDef(p_PCell);

// @siclog "Step6a2" siclog@

//Wait 1s to let UE enter RRC_INACTIVE state.

f_Delay(1.0);

// @siclog "Step6a2" siclog@

// The SS transmits a Paging message including a matched ng-5G-S-TMSI.

f_NR_Paging_Def(p_PCell, RRC_INACTIVE);

// @siclog "Step6a3" siclog@

// The UE transmits an RRCResumeRequest message.

f_NR_RRC_ResumeRequest_Def(p_PCell);

// The SS transmits an RRCResume message with sCellToAddModList to add SCell(Cell 3)

// The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 10 to schedule PUSCH.  (Note 1, Note 2)

// Check: Does the UE transmit an RRCResumeComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 9? (Note 1, Note 2, Note 3)

v_SCGConfig := f_NR_GetSCGConfigResume(p_PCell);                          // condition RESUME

v_SCGConfig.mac_CellGroupConfig := v_SCGConfigCA.mac_CellGroupConfig;  // condition SCell_Add

v_SCGConfig.sCellToAddModList  := v_SCGConfigCA.sCellToAddModList;        // condition SCell_Add

for (i:=0; i < lengthof(v_ExistingDRBs); i:=i+1) {

v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit);

}

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList);

f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_Now, f_NR_CellInfo_GetRNTI(p_PCell));

f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_Now);

f_NR5GC_RRC_ProcedureDelay(p_PCell,

cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, bit2oct(encvalue(v_SCGConfig)), v_RadioBearerConfig)),

car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCResumeComplete),

16,                  //converted in slots inside the function

"Step 6a7");

//DRBs are not reconfigured in SS: that is ok as they are not used

f_NR_SS_RachProcedureConfig(p_PCell, v_NR_RachProcedureConfig);

}

}

# Branches executed

The test case was executed on NR band CA_n41C.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000passed this test case on Starpoint SP9500 Test System. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file: 
\TC_8_1_5_8_2_1_NR5GC_PASS.spm

- PIXCIT settings used:

\PCT_PICS_PIXIT.sppar

# References

| [1] | R5s210621: Supporting information for agreement of NR5GC test case 8.1.5.8.2.1 in FR1. |
| --- | --- |