3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210624

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4616 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 8.6.4.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2021-06-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current definition of template cr_RLF_Report_1Entry has as 1st input parameter a template of type RSRP_Range, in TC 8.6.4.5 ‘*’ is used as input parameter. According ASN.1 definition: RLF-Report-r9 ::= SEQUENCE { measResultLastServCell-r9 SEQUENCE { rsrpResult-r9 RSRP-Range, rsrqResult-r9 RSRQ-Range OPTIONAL }, […] RSRP-Range is a mandatory field hence omit should not be a possibility. In order to fix the issue AnyValueOrOmit ‘*’ should be substituted by AnyValue ‘?’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed input parameter from ‘*’ to ‘?’ as omit is not an allowed value. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case(s) may end with a runttime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Correction to function f_TC_8_6_4_5_EUTRA	4

# Corrections required

## Correction to function f_TC_8_6_4_5_EUTRA

| Function name | f_TC_8_6_4_5_EUTRA |
| --- | --- |
| Reason for change | RSRP-Range is a mandatory field hence omit should not be a possibility. |
| Summary of change | Changed input parameter from ‘*’ to ‘?’ as omit is not an allowed value. |
| TTCN module | LTE_A_R10_R11\8_6\RRC_LoggedRLF.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> alt { //@sic R5s170569 sic@ []SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell2, cr_UEInformationResponse_Common(tsc_RRC_TI_Def, -, cr_RLF_Report_1Entry(*, *, -, *, cr_FailedPCellId_CellGlobalId(v_FailedPCellId), cr_Cgi(v_PLMN_Identity_Cell2, v_CellIdentity_Cell2), *, rlf, cr_Cgi(v_PLMN_Identity_Cell1,v_CellIdentity_Cell1), //@sic R5s140067, R5s140069 sic@ omit //@sic R5-171144 sic@ )))){}; //@sic R5s170569 sic@ []SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell2, cr_UEInformationResponse_Common(tsc_RRC_TI_Def, -, cr_RLF_Report_1Entry(*, *, -, *, cr_FailedPCellId_Pci_Arfcn(v_PhysCellId_Cell2, v_Dl_CarrierFreq_Cell2), cr_Cgi(v_PLMN_Identity_Cell2, v_CellIdentity_Cell2), *, rlf, cr_Cgi(v_PLMN_Identity_Cell1,v_CellIdentity_Cell1), //@sic R5s140067, R5s140069 sic@ v_FailedPCellId_v1090 //@sic R5-171144 sic@ )))){}; //@sic R5s170569 sic@ } f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.6.4.5 Step 17"); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> alt { //@sic R5s170569 sic@ []SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell2, cr_UEInformationResponse_Common(tsc_RRC_TI_Def, -, cr_RLF_Report_1Entry(?, *, -, *, cr_FailedPCellId_CellGlobalId(v_FailedPCellId), cr_Cgi(v_PLMN_Identity_Cell2, v_CellIdentity_Cell2), *, rlf, cr_Cgi(v_PLMN_Identity_Cell1,v_CellIdentity_Cell1), //@sic R5s140067, R5s140069 sic@ omit //@sic R5-171144 sic@ )))){}; //@sic R5s170569 sic@ []SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell2, cr_UEInformationResponse_Common(tsc_RRC_TI_Def, -, cr_RLF_Report_1Entry(?, *, -, *, cr_FailedPCellId_Pci_Arfcn(v_PhysCellId_Cell2, v_Dl_CarrierFreq_Cell2), cr_Cgi(v_PLMN_Identity_Cell2, v_CellIdentity_Cell2), *, rlf, cr_Cgi(v_PLMN_Identity_Cell1,v_CellIdentity_Cell1), //@sic R5s140067, R5s140069 sic@ v_FailedPCellId_v1090 //@sic R5-171144 sic@ )))){}; //@sic R5s170569 sic@ } f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.6.4.5 Step 17"); <<SKIPPED CODE>>