3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210633

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0141 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for multi frequency GNSS support checking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per TS 37571-2 Table 7.3.4.2.3.3-11 and Table 7.3.4.4.3.3-6 , “A-GNSS RequestLocation Information” will have IE “multiFreqMeasReq” TRUE if UE supports multi frequency GNSS. It means multiFreqMeas only applies if UE supports at least one of pc_A_GPS_L2C, pc_A_GPS_L5, pc_QZSS_QZS_L1C, pc_QZSS_QZS_L2C, pc_QZSS_QZS_L5, pc_GLONASS_G2, pc_GLONASS_G3, pc_GALILEO_E5a, pc_GALILEO_E5b, pc_GALILEO_E6 or pc_GALILEO_E5aE5b. Therefore we should check support for multi frequency GNSS in Provide Location Information Message only if UE supports any of above PICSs. Current TTCN implementaion checks for multi frequency GNSS in all cases for ProvideLocationInformation Message. This will fail TC 7.3.4.2.15s and 7.3.4.4.15s for all UEs which don’t support multifrequency GNSS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct checking in f_POS_CheckAgnssMultiFreq() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail some positioning test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 7.3.4.2, 7.3.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

  - Change 1

| Function name | f_POS_CheckAgnssMultiFreq() |
| --- | --- |
| Reason for change | - gnss_MeasurementList should be checked only for those UEs which support multi frequency GNSS signals. - If a device support multi frequency for one GNSS constellation (eg GPS), it doesn’t imply that it supports multiple frequencies for all other GNSS constellation (e.g if device supports multi frequency for GPS it doesn’t imply that device has to support multi frequency for Glonass/Galileo/Qzss) |
| Summary of change | 1.All check has been moved in IF condition. 2.Else part removed if device doesn’t support multi frequency for a GNSS constellation. 3. And condition changed to OR if device supports at least one multi frequency GNSS. |
| TTCN module | CommonPOS_Functions.ttcn |
| MCC160 Comment | - Acc. to the specific message content of LPP Provide Location Information (e.g. Table 7.3.4.2.3.3-9 or Table 7.3.4.4.3.3-11), the content of gnss-MeasurementList has to be checked also for UE which do not support multi freq GNSS signals. As the current name of the function is misleading, it is proposed to rename the function to f_POS_CheckGnssMeasurementList. - The checking of the non multi freq signals is added in the function. Checking for BDS signals is also added. - Further consistency checks are included in the functions checking individual methods.. See proposed implementation below. These changes are also applied to the corresponding NR function f_POS_NR_CheckAgnssMultiFreq, which is renamed to f_POS_NR_CheckGnssMeasurementList. |

Before change

function f_POS_CheckAgnssMultiFreq(GNSS_MeasurementList p_GNSS_MeasList) runs on EUTRA_PTC

{ // NOTE: The multifrequencies checked here are those listed in condition MultiFreqAGNSS of Table 7.3.4.2.3.3-11

// @sic R5-198966 sic@

var integer i;

var boolean v_AgpsFound := false;

var boolean v_GlonassFound := false;

var boolean v_GalileoFound := false;

var boolean v_QzssFound := false;

var boolean v_AgpsSignalSupported := pc_A_GPS_L2C or pc_A_GPS_L5;

var boolean v_GlonassSignalSupported := pc_GLONASS_G2 or pc_GLONASS_G3;

var boolean v_GalileoSignalSupported := pc_GALILEO_E5a or pc_GALILEO_E5b or pc_GALILEO_E6 or pc_GALILEO_E5aE5b;

var boolean v_QzssSignalSupported := pc_QZSS_QZS_L1C or pc_QZSS_QZS_L2C or pc_QZSS_QZS_L5;

//Check the measurements reported by the UE

for (i:=0; i<lengthof(p_GNSS_MeasList); i:=i+1) {

select (p_GNSS_MeasList[i].gnss_ID.gnss_id){

case(gps){

v_AgpsFound := true;

if (v_AgpsSignalSupported){

f_POS_CheckSignalsGps(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for A-GPS but not supported by UE");

}

}

case(glonass){

v_GlonassFound := true;

if (v_GlonassSignalSupported){

f_POS_CheckSignalsGlonass(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for Glonass but not supported by UE");

}

}

case(galileo){

v_GalileoFound := true;

if (v_GalileoSignalSupported){

f_POS_CheckSignalsGalileo(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for Galileo but not supported by UE");

}

}

case(qzss){

v_QzssFound := true;

if (v_QzssSignalSupported){

f_POS_CheckSignalsQzss(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for QZss but not supported by UE");

}

}

case else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "UE reported Multifrequencies not included in condition MultiFreqAGNSS of Table 7.3.4.2.3.3-11");

}

}

}

//Now check that the UE reported all the supported signals

if (not((v_AgpsSignalSupported and v_AgpsFound) and

(v_GlonassSignalSupported and v_GlonassFound) and

(v_GalileoSignalSupported and v_GalileoFound) and

(v_QzssSignalSupported and v_QzssFound)))

{

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Not all signals were reported by the UE");

}

}

After change

function f_POS_CheckAgnssMultiFreq(GNSS_MeasurementList p_GNSS_MeasList) runs on EUTRA_PTC

{ // NOTE: The multifrequencies checked here are those listed in condition MultiFreqAGNSS of Table 7.3.4.2.3.3-11

// @sic R5-198966 sic@

var integer i;

var boolean v_AgpsFound := false;

var boolean v_GlonassFound := false;

var boolean v_GalileoFound := false;

var boolean v_QzssFound := false;

var boolean v_AgpsSignalSupported := pc_A_GPS_L2C or pc_A_GPS_L5;

var boolean v_GlonassSignalSupported := pc_GLONASS_G2 or pc_GLONASS_G3;

var boolean v_GalileoSignalSupported := pc_GALILEO_E5a or pc_GALILEO_E5b or pc_GALILEO_E6 or pc_GALILEO_E5aE5b;

var boolean v_QzssSignalSupported := pc_QZSS_QZS_L1C or pc_QZSS_QZS_L2C or pc_QZSS_QZS_L5;

if(f_MultiFreqAGNSS())

{

//Check the measurements reported by the UE

for (i:=0; i<lengthof(p_GNSS_MeasList); i:=i+1) {

select (p_GNSS_MeasList[i].gnss_ID.gnss_id){

case(gps){

v_AgpsFound := true;

if (v_AgpsSignalSupported){

f_POS_CheckSignalsGps(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

/* REMOVED   else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for A-GPS but not supported by UE");

} */

}

case(glonass){

v_GlonassFound := true;

if (v_GlonassSignalSupported){

f_POS_CheckSignalsGlonass(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

/* REMOVED  else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for Glonass but not supported by UE");

} */

}

case(galileo){

v_GalileoFound := true;

if (v_GalileoSignalSupported){

f_POS_CheckSignalsGalileo(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

/* REMOVED  else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for Galileo but not supported by UE");

}*/

}

case(qzss){

v_QzssFound := true;

if (v_QzssSignalSupported){

f_POS_CheckSignalsQzss(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

/* REMOVED else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for QZss but not supported by UE");

} */

}

case else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "UE reported Multifrequencies not included in condition MultiFreqAGNSS of Table 7.3.4.2.3.3-11");

}

}

}

//Now check that the UE reported all the supported signals

if (not((v_AgpsSignalSupported and v_AgpsFound) or

(v_GlonassSignalSupported and v_GlonassFound) or

(v_GalileoSignalSupported and v_GalileoFound) or

(v_QzssSignalSupported and v_QzssFound)))

{

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Not all signals were reported by the UE");

}

}

}

## MCC TF160 proposed implementation

## function f_POS_CheckGnssMeasurementList (GNSS_MeasurementList p_GNSS_MeasList) runs on EUTRA_PTC

## { // NOTE: Function previously called f_POS_CheckAgnssMultiFreq. Renamed as now also non multifrequency signals are checked

## // @sic R5-198966 sic@

## var integer i;

## var boolean v_AgpsSignalSupported := pc_A_GPS_L1C_A or pc_A_GPS_L1C or pc_A_GPS_L2C or pc_A_GPS_L5;

## var boolean v_GlonassSignalSupported := pc_GLONASS_G1 or pc_GLONASS_G2 or pc_GLONASS_G3;

## var boolean v_GalileoSignalSupported := pc_GALILEO_E1 or pc_GALILEO_E5a or pc_GALILEO_E5b or pc_GALILEO_E6 or pc_GALILEO_E5aE5b;

## var boolean v_QzssSignalSupported := pc_QZSS_QZS_L1 or pc_QZSS_QZS_L1C or pc_QZSS_QZS_L2C or pc_QZSS_QZS_L5;

## var boolean v_BdsSignalSupported := pc_BDS_B1I or pc_BDS_B1C;

## //Check the measurements reported by the UE

## for (i:=0; i<lengthof(p_GNSS_MeasList); i:=i+1) {

## select (p_GNSS_MeasList[i].gnss_ID.gnss_id){

## case(gps){

## if (v_AgpsSignalSupported){

## f_POS_CheckSignalsGps(p_GNSS_MeasList[i].gnss_SgnMeasList);

## }

## else {

## f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for A-GPS but not supported by UE");

## }

## }

## case(glonass){

## if (v_GlonassSignalSupported){

## f_POS_CheckSignalsGlonass(p_GNSS_MeasList[i].gnss_SgnMeasList);

## }

## else {

## f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for Glonass but not supported by UE");

## }

## }

## case(galileo){

## if (v_GalileoSignalSupported){

## f_POS_CheckSignalsGalileo(p_GNSS_MeasList[i].gnss_SgnMeasList);

## }

## else {

## f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for Galileo but not supported by UE");

## }

## }

## case(qzss){

## if (v_QzssSignalSupported){

## f_POS_CheckSignalsQzss(p_GNSS_MeasList[i].gnss_SgnMeasList);

## }

## else {

## f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for QZss but not supported by UE");

## }

## }

## case(bds){

## if (v_BdsSignalSupported){

## f_POS_CheckSignalsBds(p_GNSS_MeasList[i].gnss_SgnMeasList);

## }

## else {

## f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Measurement reported for Bds but not supported by UE");

## }

## }

## case else {

## f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "UE reported Multifrequencies not included in condition MultiFreqAGNSS of Table 7.3.4.2.3.3-11");

## }

## }

## }

## }

## function f_POS_CheckSignalsGps(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

## {// @sic R5-198966 sic@

## var integer i;

## var boolean v_A_GPS_L1C_A_Reported := false;

## var boolean v_A_GPS_L1C_Reported := false;

## var boolean v_A_GPS_L2C_Reported := false;

## var boolean v_A_GPS_L5_Reported := false;

## for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

## v_A_GPS_L1C_A_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

## v_A_GPS_L1C_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

## v_A_GPS_L2C_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

## v_A_GPS_L5_Reported := true;

## }

## if (not((v_A_GPS_L1C_A_Reported == pc_A_GPS_L1C_A) and (v_A_GPS_L1C_Reported == pc_A_GPS_L1C) and (v_A_GPS_L2C_Reported == pc_A_GPS_L2C) and (v_A_GPS_L5_Reported == pc_A_GPS_L5)))

## {

## f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GPS signals not reported correctly");

## }

## }

## }

## function f_POS_CheckSignalsGlonass(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

## {// @sic R5-198966 sic@

## var integer i;

## var boolean v_GLONASS_G1_Reported := false;

## var boolean v_GLONASS_G2_Reported := false;

## var boolean v_GLONASS_G3_Reported := false;

## for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

## v_GLONASS_G1_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

## v_GLONASS_G2_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

## v_GLONASS_G3_Reported := true;

## }

## if (not((v_GLONASS_G1_Reported == pc_GLONASS_G1) and (v_GLONASS_G2_Reported == pc_GLONASS_G2) and (v_GLONASS_G3_Reported == pc_GLONASS_G3)))

## {

## f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "GLONASS signals not reported correctly");

## }

## }

## }

## function f_POS_CheckSignalsGalileo(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

## {// @sic R5-198966 sic@

## var integer i;

## var boolean v_GALILEO_E1_Reported := false;

## var boolean v_GALILEO_E5a_Reported := false;

## var boolean v_GALILEO_E5b_Reported := false;

## var boolean v_GALILEO_E6_Reported := false;

## var boolean v_GALILEO_E5aE5b_Reported := false;

## for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

## v_GALILEO_E1_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

## v_GALILEO_E5a_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

## v_GALILEO_E5b_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

## v_GALILEO_E6_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(4))) {

## v_GALILEO_E5aE5b_Reported := true;

## }

## if (not((v_GALILEO_E1_Reported == pc_GALILEO_E1) and (v_GALILEO_E5a_Reported == pc_GALILEO_E5a) and (v_GALILEO_E5b_Reported == pc_GALILEO_E5b) and

## (v_GALILEO_E6_Reported == pc_GALILEO_E6) and (v_GALILEO_E5aE5b_Reported == pc_GALILEO_E5aE5b)))

## f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Galileo signals not reported correctly");

## }

## }

## }

## function f_POS_CheckSignalsQzss(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

## {// @sic R5-198966 sic@

## var integer i;

## var boolean v_QZSS_QZS_L1_Reported := false;

## var boolean v_QZSS_QZS_L1C_Reported := false;

## var boolean v_QZSS_QZS_L2C_Reported := false;

## var boolean v_QZSS_QZS_L5_Reported := false;

## for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

## v_QZSS_QZS_L1_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

## v_QZSS_QZS_L1C_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

## v_QZSS_QZS_L2C_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

## v_QZSS_QZS_L5_Reported := true;

## }

## if (not ((v_QZSS_QZS_L1_Reported == pc_QZSS_QZS_L1) and (v_QZSS_QZS_L1C_Reported == pc_QZSS_QZS_L1C) and

## (v_QZSS_QZS_L2C_Reported == pc_QZSS_QZS_L2C) and (v_QZSS_QZS_L5_Reported == pc_QZSS_QZS_L5)))

## {

## f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "QZSS signals not reported correctly");

## }

## }

## }

## function f_POS_CheckSignalsBds(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

## {// @sic R5s210633 sic@

## var integer i;

## var boolean v_BDS_B1I_Reported := false;

## var boolean v_BDS_B1C_Reported := false;

## for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

## v_BDS_B1I_Reported := true;

## }

## if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement((9,10,11)))) {

## v_BDS_B1C_Reported := true;

## }

## }

## if (not ((v_BDS_B1I_Reported == pc_BDS_B1I) and (v_BDS_B1C_Reported == pc_BDS_B1C)))

## {

## f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "BDS signals not reported correctly");

## }

## }

## template (present) GNSS_SgnMeasElement cr_GNSS_SgnMeasElement(template integer p_SignalID) :=

## { /* @status    APPROVED (POS) */

## //@sic R5-198966 sic@

## gnss_SignalID := {

## gnss_SignalID := p_SignalID,

## gnss_SignalID_Ext_r15 := *

## },

## gnss_CodePhaseAmbiguity := *,

## gnss_SatMeasList := ?

## }