3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210634

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4618 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT EMM-CIoT TC 22.5.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, NB_IOTenh-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At Step 11, as per prose, SS transmits RRCConnectionRelease-NB message (redirectedCarrierInfo including carrierFreq-r13 of Ncell 1 instead of Ncell 23. |  |  |  |  |  |  |  |  |
| Summary of change: |  | carrierFreq-r13 of Ncell 1 is included instead Ncell 23 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3- B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_22_5_20_Body |
| --- | --- |
| Reason for change | At Step 11, as per prose, SS transmits RRCConnectionRelease-NB message (redirectedCarrierInfo including carrierFreq-r13 of Ncell 1 instead of Ncell 23. |
| Summary of change | carrierFreq-r13 of Ncell 1 is included instead Ncell 23 |
| TTCN module | NBIOT\22_5\ENBIOT_NAS.ttcn |
| MCC160 Comment | Accepted |

Before change:

function fl_TC_22_5_20_Body() runs on NBIOT_PTC

{

var O1_Type v_UserData := 'F0'O;

var NAS_MSG_Indication_Type v_NAS_Ind;

var template (value) NB_CellPowerList_Type v_CellPowerList;

var template (omit) EPS_BearerContextStatus v_EPS_ContextStatus;

var GutiParameters_Type v_Guti_Params_Cell1 := f_NBIOT_CellInfo_GetGuti (nbiot_Cell1);

var NAS_PlmnId v_PLMN_Cell1 := f_Asn2Nas_PlmnId (v_Guti_Params_Cell1.PLMN_Identity);

var EUTRA_ASN1_TrackingAreaCode_Type v_TAC_Cell1 := f_NBIOT_CellInfo_GetTAC (nbiot_Cell1);

//The timer uplink data delay is configured in CLOSE UE TEST LOOP.

//When UE receives downlink ESM DATA TRANSPORT, UE will start this timer and loopback the uplink user data when this timer expires.

//But timer uplink data delay = 8s may be too short when RRC connnection release procedure takes 10s

//Hence this is set to 15s

var NB_CarrierFreq_Type v_CarrierFreq_Cell23 := f_NBIOT_CellInfo_GetEARFCN(nbiot_Cell23); // @sic R5-206436 sic@

var integer v_DelayLoopbackDataTime := 5;

timer t_T3448 := 30.0;

timer t_Wait;

//======skipped code========

//@siclog "Step 11" siclog@

//SS transmits an RRCConnectionRelease-NB message (IE redirectedCarrierInfo including carrierFreq-r13 of Ncell 23).

f_NBIOT_RRC_ConnectionRelease_Common(nbiot_Cell23, CONTROL_PLANE, cs_RRCConnectionRelease_NB_r14(tsc_RRC_TI_Def, other, omit , omit, 30, v_CarrierFreq_Cell23.DL), cs_TimingInfo_Now); // @sic R5-206436 sic@

//======skipped code========

//@siclog "Step 31" siclog@

//The SS transmits an RRCConnectionRelease-NB message

f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1);   // @sic R5s190574 sic@

} // End of fl_TC_22_5_20_Body

After change:

function fl_TC_22_5_20_Body() runs on NBIOT_PTC

{

var O1_Type v_UserData := 'F0'O;

var NAS_MSG_Indication_Type v_NAS_Ind;

var template (value) NB_CellPowerList_Type v_CellPowerList;

var template (omit) EPS_BearerContextStatus v_EPS_ContextStatus;

var GutiParameters_Type v_Guti_Params_Cell1 := f_NBIOT_CellInfo_GetGuti (nbiot_Cell1);

var NAS_PlmnId v_PLMN_Cell1 := f_Asn2Nas_PlmnId (v_Guti_Params_Cell1.PLMN_Identity);

var EUTRA_ASN1_TrackingAreaCode_Type v_TAC_Cell1 := f_NBIOT_CellInfo_GetTAC (nbiot_Cell1);

//The timer uplink data delay is configured in CLOSE UE TEST LOOP.

//When UE receives downlink ESM DATA TRANSPORT, UE will start this timer and loopback the uplink user data when this timer expires.

//But timer uplink data delay = 8s may be too short when RRC connnection release procedure takes 10s

//Hence this is set to 15s

var NB_CarrierFreq_Type v_CarrierFreq_Cell1 := f_NBIOT_CellInfo_GetEARFCN(nbiot_Cell1); // @sic R5-206436 sic@

var integer v_DelayLoopbackDataTime := 5;

timer t_T3448 := 30.0;

timer t_Wait;

//======skipped code========

//@siclog "Step 11" siclog@

//SS transmits an RRCConnectionRelease-NB message (IE redirectedCarrierInfo including carrierFreq-r13 of Ncell 1).

f_NBIOT_RRC_ConnectionRelease_Common(nbiot_Cell23, CONTROL_PLANE, cs_RRCConnectionRelease_NB_r14(tsc_RRC_TI_Def, other, omit , omit, 30, v_CarrierFreq_Cell1.DL), cs_TimingInfo_Now); // @sic R5-206436 sic@

//======skipped code========

//@siclog "Step 31" siclog@

//The SS transmits an RRCConnectionRelease-NB message

f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1);   // @sic R5s190574 sic@

} // End of fl_TC_22_5_20_Body