3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210651

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0142 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC Positioning test case 9.3.1.2.5s |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC Positioning test case 9.3.1.2.5s to the POS_IWD_21wk12 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to 5GS POS test case 9.3.1.2.5s required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail some positioning test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2.5s.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 9.3.1.2.5s  which is part of the POS test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	9.3.1.2.5s

ATS Version:	iwd-TTCN3-B2020-09_D21wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

UE used:	MediaTek MT6297,

Verification Status:	PASS

# Corrections required

  - Change 1

| Function name | f_TC_9_3_1_2_5s_EUTRA_OTDOA_POS |
| --- | --- |
| Reason for change | 1. The EUTRAN cell is not switch on. 2. NR part is still in the preamble when EUTRA already send coordination message with OTDOA information. This casue TC failed |
| Summary of change | 1. Switch on the EUTRA cell 2. Add wait for coordination message send from NR when preamble on NR part finished |
| TTCN module | NR5GC_Positioning_Testcases.ttcn |
| MCC160 Comment | Accepted. |

Before change

function f_TC_9_3_1_2_5s_EUTRA_OTDOA_POS() runs on EUTRA_OTDOA_POS_PTC

{//LPP Abort - OTDOA (LTE)

f_EUTRA_POS_Init(c1);

// create cell for OTDOA and send out system information

f_POS_EUTRA_CellConfig_OTDOA (eutra_Cell1);

f_OTDOA_GetOtdoaInfoAndSendCoordMsg();

// Wait for co-ordination message from NR to finish the test case

f_E_OTDOA_NR_WaitForCoOrd_Trigger(NR);

f_EUTRA_ReleaseAllCells();

}

After change

function f_TC_9_3_1_2_5s_EUTRA_OTDOA_POS() runs on EUTRA_OTDOA_POS_PTC

{//LPP Abort - OTDOA (LTE)

f_EUTRA_POS_Init(c1);

// create cell for OTDOA and send out system information

f_POS_EUTRA_CellConfig_OTDOA (eutra_Cell1);

f_EUTRA_SetCellPower(eutra_Cell1, tsc_SuitableCellRS_EPRE);

f_E_OTDOA_NR_WaitForCoOrd_Trigger(NR);

f_OTDOA_GetOtdoaInfoAndSendCoordMsg();

// Wait for co-ordination message from NR to finish the test case

f_E_OTDOA_NR_WaitForCoOrd_Trigger(NR);

f_EUTRA_ReleaseAllCells();

}

  - Change 2

| Function name | f_TC_9_3_1_2_5s_NR5GC |
| --- | --- |
| Reason for change | The same reason like chang 1 point 2 |
| Summary of change | Send the coordination massage to the EUTRA after preabmble when measurement GAP are configured. |
| TTCN module | NR5GC_Positioning_Testcases.ttcn |
| MCC160 Comment | Accepted. |

Before change

function f_TC_9_3_1_2_5s_NR5GC() runs on NR5GC_POS_PTC

{//LPP Abort - OTDOA (LTE)

var E_OTDOA_NR_CoOrd_OtdoaInfo_Type v_OtdoaInfo;

f_NR5GC_POS_Init(NR_1);

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

//Preamble: The UE is in 5GS state 3N-A

f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A);

// Table 9.3.1.2.3.1-1: Configuration of measurement gaps for OTDOA

//@sic R5-204489 sic@

f_NR_ConfigMeasGap_OTDOA();

//Wait for co-ordination message from EUTRA_OTDOA component

v_OtdoaInfo := f_E_OTDOA_NR_WaitForCoOrd_OtdoaInfo(OTDOA);

f_NR_TestBody_Set(true);

f_TC_9_3_1_2_5s_NR5GC_TestBody(v_OtdoaInfo);

f_NR_TestBody_Set(false);

// Switch/Power off UE

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

//Send co-ordination message to EUTRA_OTDOA component to finish the test case

f_E_OTDOA_NR_SendCoOrd(OTDOA, cms_E_OTDOA_NR_Trigger);

} //end of f_TC_9_3_1_2_15s_NR5GC

After change

function f_TC_9_3_1_2_5s_NR5GC() runs on NR5GC_POS_PTC

{//LPP Abort - OTDOA (LTE)

var E_OTDOA_NR_CoOrd_OtdoaInfo_Type v_OtdoaInfo;

f_NR5GC_POS_Init(NR_1);

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

//Preamble: The UE is in 5GS state 3N-A

f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A);

// Table 9.3.1.2.3.1-1: Configuration of measurement gaps for OTDOA

//@sic R5-204489 sic@

f_NR_ConfigMeasGap_OTDOA();

f_E_OTDOA_NR_SendCoOrd(OTDOA, cms_E_OTDOA_NR_Trigger);

//Wait for co-ordination message from EUTRA_OTDOA component

v_OtdoaInfo := f_E_OTDOA_NR_WaitForCoOrd_OtdoaInfo(OTDOA);

f_NR_TestBody_Set(true);

f_TC_9_3_1_2_5s_NR5GC_TestBody(v_OtdoaInfo);

f_NR_TestBody_Set(false);

// Switch/Power off UE

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

//Send co-ordination message to EUTRA_OTDOA component to finish the test case

f_E_OTDOA_NR_SendCoOrd(OTDOA, cms_E_OTDOA_NR_Trigger);

} //end of f_TC_9_3_1_2_15s_NR5GC

# Branches Executed

The test case was executed on NR band n78 and EUTRA band 3 on Anritsu Platform.

# Execution Log Files

## MediaTek MT6297

The MediaTek MT6297 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_9_3_1_2_5s_LOG.xml

- PICS/PIXIT parameter file:

TC_9_3_1_2_5s_PIXIT.txt

# References

| [1] | R5s210652 |
| --- | --- |