3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210668

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1771 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR MAC testcase 7.1.1.7.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Change 1 - sCellDeactivationTimer needs to be set as per Table 7.1.1.7.1.1.3.3-4 Change 2 - v_SI_Rcv flag was set to true following Steps 5,7. To reuse this on Step 14,16,20, the flag needs to be reset to false. - Current TTCN implementation involves repeats on both alt branches, and since v_Data_Rcv is an unused variable, it will never return true. This leads to watchdog timer always timing out and failing. Since this procedure is identical to the previous procedure (Steps 5,7,10), it’s better to have identical implementations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1 1. Set sCellDeactivationTimer to ms320 Change 2 - Reset v_SI_Rcv value to false - Copy same implementation as Steps 5,7,10 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.7.1.1.NR5GC, 7.1.1.7.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | function f_TC_7_1_1_7_1_ENDC_Common |
| --- | --- |
| Reason for change | sCellDeactivationTimer needs to be set as per Table 7.1.1.7.1.1.3.3-4 |
| Summary of change | Set sCellDeactivationTimer to ms320 |
| TTCN module | MAC_CA_ENDC_NR |
| MCC160 Comment | Accepted |

Before change:

function f_TC_7_1_1_7_1_ENDC_Common(NR_CellId_Type  p_PSCell,

NR_CellId_Type  p_SCell,

NR_CA_Tested_Type p_NR_CA_Tested) runs on ENDC_NR_PTC

{

//Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer

var boolean v_UL_CA := pc_UL_NR_CA_2CC ;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList  := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)};

f_ENDC_NR_Init_CA(p_NR_CA_Tested, {p_PSCell, p_SCell}, v_UL_CA);

//Create and configure NR cell1s

f_NR_CellConfig_PCell_Def(p_PSCell);

f_NR_CellConfig_SCell(p_SCell,Scell_Active);

// Get NR capabilities sent during EUTRA preamble

f_NR_ENDC_PreambleOnEUTRA(p_PSCell, MCG_Only);

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Configure on SS the SCG DRB in test mode

f_NR_ENDC_ConfigAM_UM(p_PSCell, 1, 0, v_SS_Drb_ConfigList);

// Wait for Trigger from EUTRA indicating start of Test Body

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_TestBody_Set(true);

//@siclog "Steps 1 - 2" siclog@

f_NR_AddMod_1SCell_ENDC(p_PSCell, p_SCell,-,-,-,-,v_UL_CA);

//Configure SS in No header manipulation

v_SS_Drb_ConfigList  := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList);

//@siclog "Steps 3-20" siclog@

f_TC_7_1_1_7_1_NR_TestBody(p_PSCell, p_SCell,tsc_SCellIndex_2,tsc_NR_DRB2,v_UL_CA);

f_NR_TestBody_Set(false);

// Send Trigger to EUTRA indicating Finish of Test Body

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger);

//Wait from coordination AFTER the connection is released in E-UTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

//Postamble

f_NR_ReleaseAllCells();

}

After change:

function f_TC_7_1_1_7_1_ENDC_Common(NR_CellId_Type  p_PSCell,

NR_CellId_Type  p_SCell,

NR_CA_Tested_Type p_NR_CA_Tested) runs on ENDC_NR_PTC

{

//Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer

var boolean v_UL_CA := pc_UL_NR_CA_2CC ;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList  := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)};

f_ENDC_NR_Init_CA(p_NR_CA_Tested, {p_PSCell, p_SCell}, v_UL_CA);

//Create and configure NR cell1s

f_NR_CellConfig_PCell_Def(p_PSCell);

f_NR_CellConfig_SCell(p_SCell,Scell_Active);

// Get NR capabilities sent during EUTRA preamble

f_NR_ENDC_PreambleOnEUTRA(p_PSCell, MCG_Only);

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Configure on SS the SCG DRB in test mode

f_NR_ENDC_ConfigAM_UM(p_PSCell, 1, 0, v_SS_Drb_ConfigList);

// Wait for Trigger from EUTRA indicating start of Test Body

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_TestBody_Set(true);

//@siclog "Steps 1 - 2" siclog@

f_NR_AddMod_1SCell_ENDC(p_PSCell, p_SCell,-,-,-,-,v_UL_CA, ms320);

//Configure SS in No header manipulation

v_SS_Drb_ConfigList  := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList);

//@siclog "Steps 3-20" siclog@

f_TC_7_1_1_7_1_NR_TestBody(p_PSCell, p_SCell,tsc_SCellIndex_2,tsc_NR_DRB2,v_UL_CA);

f_NR_TestBody_Set(false);

// Send Trigger to EUTRA indicating Finish of Test Body

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger);

//Wait from coordination AFTER the connection is released in E-UTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

//Postamble

f_NR_ReleaseAllCells();

}

## 2.2 Change 2

| Function name | function f_TC_7_1_1_7_1_NR_TestBody |
| --- | --- |
| Reason for change | 1. v_SI_Rcv flag was set to true following Steps 5,7. To reuse this on Step 14,16,20, the flag needs to be reset to false. 2. Current TTCN implementation involves repeats on both alt branches, and since v_Data_Rcv is an unused variable, it will never return true. This leads to watchdog timer always timing out and failing. Since this procedure is identical to the previous procedure (Steps 5,7,10), it’s better to have identical implementations. |
| Summary of change | 1. Reset v_SI_Rcv value to false 2. Copy same implementation as Steps 5,7,10 |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | - Accepted - Accepted in principle. The CR proposed implementation does not allow for receiving SR and MAC PDU in any order. Hence implemented as shown below. |

Before change:

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type    p_SPCell,

NR_CellId_Type    p_SCell,

SCellIndex p_SCellIndex ,

DRB_Identity p_NR_DRB_Id,

boolean p_UL_CA) runs on NR_BASE_PTC

{ //UE power headroom reporting / SCell activation / DL pathloss change reporting/

const integer const_UL_TBS := 384;

var NR_MAC_SDU_Type v_EncodedRlcPdu1;

var NR_MAC_SDU_Type v_EncodedRlcPdu2;

var NR_MAC_SDU_Type v_EncodedRlcPdu3;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL1;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL2;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL3;

var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL1;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL2;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL3;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL4;

var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation2;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SPCell, tsc_NR_BWP_Id);

var NR_UplinkBWP_Type v_NR_UplinkBWP2 ;

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);

var boolean v_SI_Rcv := false;

var boolean v_Data_Rcv := false;

var B8_Type v_DeactivateSCell := '00000000'B; //

var NR_CellId_Type    v_CellID_UL :=    p_SPCell;

timer t_Watchdog := 5.0;

// Stop UL Grant configuration

f_NR_ULGrantConfiguration_Stop(p_SPCell, -,cs_NR_UplinkTimeAlignment_Keep);

// PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B)

v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) };

v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) };

v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL2 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2) };

v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL2) };

v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL3 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3) };

v_MAC_PDUListDL3 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL3) };

v_MAC_PDUListDL4 := { cs_NR_DL_MAC_PDU(omit, omit,  cs_NR_MAC_Padding_SubPDU('00000000'O)) };   // 1 byte sub headers +4byte Padding header= 40 bits

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP);

v_NR_ResourceAllocation2:= v_NR_ResourceAllocation; //For no UL case

v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) };

v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) };

if (p_UL_CA) { //UL supported case steps 15 & 16 in SCell

v_NR_UplinkBWP2 := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id);

v_NR_ResourceAllocation2 := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP2);

v_CellID_UL := p_SCell;

}

t_Watchdog.start ;

//@siclog "Steps 3" siclog@

//The SS Activates SCell .

v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL

f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL));

//@siclog "Steps 4" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL1));

//@siclog "Steps 6" siclog@

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SCell, v_TimingDL, 40); // 40 ms later

f_NR_OneULGrantTransmission(p_SPCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation);

//@siclog "Steps 8" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100);

f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 1, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) );

//@siclog "Steps 9" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 400);  //

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2));

//@sic R5s210066 sic@

//enable reception on sp_Cell

f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(enable));

//@siclog "Steps 5, 7" siclog@

alt {

[] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?))

{

if (not v_SI_Rcv)

{

v_SI_Rcv := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

}

repeat;

};

[]DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(p_SPCell, p_NR_DRB_Id, -, v_MAC_PDUListUL))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

t_Watchdog.stop;

};

};

//@siclog "Step 10" siclog@

t_Watchdog.start (1.0);

alt{

[] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?))

{

t_Watchdog.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: TP2 failed, UE is sending SR  ");

};

[] t_Watchdog.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10");

}

};

t_Watchdog.start;

//@siclog "Steps 11" siclog@

//The SS Activates SCell .

v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL

f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL));

//@siclog "Steps 12" siclog@

//Padding MAC PDU Only

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL4));

//@siclog "Steps 13" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200);  // 200+200 =400

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2));

//@siclog "Steps 15" siclog@

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SCell, v_TimingDL, 40); // 40 ms later

f_NR_OneULGrantTransmission(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation2);

//@siclog "Steps 17" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100);

f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) );

//@siclog "Step 18" siclog@

//Deactivate SCell

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200);

f_NR_SS_CommonCellConfig(p_SPCell, cas_NR_MacCommandTrigger_REQ(p_SPCell, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_SCellActDeact_CE(v_DeactivateSCell)));

//@siclog "Step 19" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL3));

//@siclog "Steps 14, 16, 20" siclog@

alt {

[] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?))

{

if (not v_SI_Rcv)

{

v_SI_Rcv := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

} else if(v_Data_Rcv)

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20: TP4 failed, UE is sending SR ");

t_Watchdog.stop;

break;

}

repeat;

};

[]DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(v_CellID_UL, p_NR_DRB_Id, -, v_MAC_PDUListUL))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16");

repeat;

};

};

//@sic R5s210066 sic@

//disable reception on sp_Cell

f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(disable));

}; //End of f_TC_7_1_1_7_1_NR_TestBody

After change:

function f_TC_7_1_1_7_1_NR_TestBody(NR_CellId_Type    p_SPCell,

NR_CellId_Type    p_SCell,

SCellIndex p_SCellIndex ,

DRB_Identity p_NR_DRB_Id,

boolean p_UL_CA) runs on NR_BASE_PTC

{ //UE power headroom reporting / SCell activation / DL pathloss change reporting/

const integer const_UL_TBS := 384;

var NR_MAC_SDU_Type v_EncodedRlcPdu1;

var NR_MAC_SDU_Type v_EncodedRlcPdu2;

var NR_MAC_SDU_Type v_EncodedRlcPdu3;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL1;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL2;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL3;

var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL1;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL2;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL3;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL4;

var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation2;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SPCell, tsc_NR_BWP_Id);

var NR_UplinkBWP_Type v_NR_UplinkBWP2 ;

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);

var boolean v_SI_Rcv := false;

var boolean v_Data_Rcv := false;

var B8_Type v_DeactivateSCell := '00000000'B; //

var NR_CellId_Type    v_CellID_UL :=    p_SPCell;

timer t_Watchdog := 5.0;

// Stop UL Grant configuration

f_NR_ULGrantConfiguration_Stop(p_SPCell, -,cs_NR_UplinkTimeAlignment_Keep);

// PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B)

v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL1 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) };

v_MAC_PDUListDL1 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL1) };

v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL2 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2) };

v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL2) };

v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL3 := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3) };

v_MAC_PDUListDL3 := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL3) };

v_MAC_PDUListDL4 := { cs_NR_DL_MAC_PDU(omit, omit,  cs_NR_MAC_Padding_SubPDU('00000000'O)) };   // 1 byte sub headers +4byte Padding header= 40 bits

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP);

v_NR_ResourceAllocation2:= v_NR_ResourceAllocation; //For no UL case

v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) };

v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) };

if (p_UL_CA) { //UL supported case steps 15 & 16 in SCell

v_NR_UplinkBWP2 := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id);

v_NR_ResourceAllocation2 := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP2);

v_CellID_UL := p_SCell;

}

t_Watchdog.start ;

//@siclog "Steps 3" siclog@

//The SS Activates SCell .

v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL

f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL));

//@siclog "Steps 4" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL1));

//@siclog "Steps 6" siclog@

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SCell, v_TimingDL, 40); // 40 ms later

f_NR_OneULGrantTransmission(p_SPCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation);

//@siclog "Steps 8" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100);

f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 1, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) );

//@siclog "Steps 9" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 400);  //

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2));

//@sic R5s210066 sic@

//enable reception on sp_Cell

f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(enable));

//@siclog "Steps 5, 7" siclog@

alt {

[] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?))

{

if (not v_SI_Rcv)

{

v_SI_Rcv := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

}

repeat;

};

[]DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(p_SPCell, p_NR_DRB_Id, -, v_MAC_PDUListUL))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

t_Watchdog.stop;

};

};

//@siclog "Step 10" siclog@

t_Watchdog.start (1.0);

alt{

[] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?))

{

t_Watchdog.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: TP2 failed, UE is sending SR  ");

};

[] t_Watchdog.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10");

}

};

t_Watchdog.start;

//@siclog "Steps 11" siclog@

//The SS Activates SCell .

v_TimingDL := f_NR_GetNextSendOccasion_DL(p_SCell, 300); // Timing of DL

f_SS_CellConfig_ActivateSCell_CA(p_SPCell, p_SCellIndex,cs_TimingInfo_NR(v_TimingDL));

//@siclog "Steps 12" siclog@

//Padding MAC PDU Only

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL4));

//@siclog "Steps 13" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200);  // 200+200 =400

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL2));

//@siclog "Steps 15" siclog@

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(p_SCell, v_TimingDL, 40); // 40 ms later

f_NR_OneULGrantTransmission(v_CellID_UL, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation2);

//@siclog "Steps 17" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 100);

f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 2, p_NR_DRB_Id,p_SCell,cs_TimingInfo_NR(v_TimingDL) );

//@siclog "Step 18" siclog@

//Deactivate SCell

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200);

f_NR_SS_CommonCellConfig(p_SPCell, cas_NR_MacCommandTrigger_REQ(p_SPCell, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_SCellActDeact_CE(v_DeactivateSCell)));

//@siclog "Step 19" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 200);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL3));

//@siclog "Steps 14, 16" siclog@

v_SI_Rcv := false;

alt {

[] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?))

{

if (not v_SI_Rcv)

{

v_SI_Rcv := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

}

repeat;

};

[]DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(v_CellID_UL, p_NR_DRB_Id, -, v_MAC_PDUListUL))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16");

t_Watchdog.stop;

};

};

//@siclog "Step 20" siclog@

t_Watchdog.start (1.0);

alt{

[] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?))

{

t_Watchdog.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20: TP4 failed, UE is sending SR  ");

};

[] t_Watchdog.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20");

}

};

//@sic R5s210066 sic@

//disable reception on sp_Cell

f_NR_SS_SystemIndCtrlConfig(p_SPCell, cds_NR_SystemIndCtrl_SchedReq(disable));

}; //End of f_TC_7_1_1_7_1_NR_TestBody

MCC 160 Implementation

//@siclog "Steps 14, 16, 20" siclog@ v_SI_Rcv := false; v_Data_Rcv := false; alt { [] SYSIND.receive (car_NR_SchedulingReq_IND (p_SPCell, ?)) { if (not v_SI_Rcv) { v_SI_Rcv := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20: TP4 failed, UE is sending SR "); t_Watchdog.stop; break; } repeat; }; []DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(v_CellID_UL, p_NR_DRB_Id, -, v_MAC_PDUListUL)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); v_Data_Rcv := true; repeat; }; [] t_Watchdog.timeout { if (v_Data_Rcv and v_SI_Rcv) {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20"); }; }; };