3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210674

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1776 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test cases 7.1.2.3.3, 7.1.2.3.4 ,7.1.3.1.1 and 7.1.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1) In the current TTCN implementation of 7.1.2.3.3/4, 7.1.3.1.1/2 Initialization of v_TimingPreviousSlot is done after the transmission of each PDU List, instead it is proposed to initialize v_TimingPreviousSlot at the beginning, then v_TimingPreviousSlot is set with v_TimingCurrentSlot after the reception of PDU List in each iteration 2) In the current TTCN implementation of 7.1.3.1.1/2, v_TimingCurrentSlot is set by querying current system time after the reception of PDU List , instead it is proposed to make use of v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame field to set v_TimingCurrentSlot on the reception of PDU List |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Initialize v_TimingPreviousSlot at the beginning, then v_TimingPreviousSlot is set with v_TimingCurrentSlot after the reception of PDU List in each iteration ( for test cases 7.1.2.3.3/4,7.1.3.1.1/2) 2) v_TimingCurrentSlot is set with the value received in v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame field to set v_TimingCurrentSlot ( for test cases 7.1.3.1.1/2) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | v_TimingPreviousSlot is set unexpectedly after the transmission of each PDU List v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame is left unutilized for setting v_TimingCurrentSlot |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.3.ENDC,7.1.2.3.4.ENDC,7.1.3.1.1.ENDC,7.1.3.1.2.ENDC,7.1.2.3.3.NR5GC, 7.1.2.3.4.NR5GC, 7.1.3.1.1.NR5GC and 7.1.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to function fl_TC_7_1_2_3_3and4	3

1.2	Correction to function fl_TC_7_1_3_1_1and2	6

1.3	Correction to function f_NR_PDCP_PDU_GetList	12

# Corrections required

## Correction to function fl_TC_7_1_2_3_3and4

| Function name | fl_TC_7_1_2_3_3and4 |
| --- | --- |
| Reason for change | In the current TTCN implementation of 7.1.2.3.3/4, 7.1.3.1.1/2 Initialization of v_TimingPreviousSlot is done after the transmission of each PDU List, instead it is proposed to initialize v_TimingPreviousSlot at the beginning, then v_TimingPreviousSlot is set with v_TimingCurrentSlot after the reception of PDU List in each iteration |
| Summary of change | Initialize v_TimingPreviousSlot at the beginning, then v_TimingPreviousSlot is set with v_TimingCurrentSlot after the reception of PDU List in each iteration |
| TTCN module | RLC_TC_Common_NR |
| MCC160 Comment | Accepted as an improvement |

Before Change:

function fl_TC_7_1_2_3_3and4(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC { var integer v_RLC_SDUsize; //@sic R5s190358 sic@ var B1_Type v_Poll; var integer i; var integer v_ListOffset:=0; // @sic R5s210579 sic@ var integer j; var template (value) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU; var template (present) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_Expected; var integer v_VerdictFilter; // @sic R5-185083 sic@ var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; var NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_Received; var NR_L2DataList_Type v_NR_L2DataList; var integer v_LenPDUList; var template (value) NR_RLC_PDUList_Type v_RlcPduList; // @sic R5-213460 sic@ var SubFrameTiming_Type v_TimingCurrentSlot; // @sic R5-213460 sic@ var SubFrameTiming_Type v_TimingPreviousSlot; // @sic R5-213460 sic@ var integer k; if (p_RLC_Rec.AM_SN_Size == 18) { v_VerdictFilter := 4096; // @sic R5s200315 sic@ } else { //SN12 v_VerdictFilter := 256; } v_RLC_SDUsize := 4; // @sic R5-185083, R5s190358, R5-202613 sic@ // EXCEPTION: Steps 1 - 3a1 shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration_size). for (j := 0; j < p_OuterLoop; j:= j + 1) { // @sic R5-197663, R5-197664, R5-202613 sic@ // generate p_InnerLoop RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, p_InnerLoop); // list offset is 0, TX_NEXT administered when generating // put those p_InnerLoop AMD PDUs into a list for transmission v_Poll := tsc_NR_P_NoPoll; // no indication of Poll bit usage in the prose for (i := 0; i < p_InnerLoop; i:= i + 1) { v_NR_RLC_AMD_PDU := f_NR_RLC_Get_AMD_FullSDU_TX(p_RLC_Rec, v_Poll, v_NR_RLC_DataList[i]); v_RlcPduList[i] := {AMD := v_NR_RLC_AMD_PDU}; // @sic R5-202613, R5-213460 sic@ } // @siclog "Step 1" siclog@ // SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size. Each RLC Data PDU contains one RLC SDU. f_NR_RLCPDUList_Send(nr_Cell1, p_NR_DRB_Id, v_RlcPduList); // @sic R5-213460 sic@ v_TimingPreviousSlot := f_NR_GetCurrentTiming(nr_Cell1); // initialize slot @sic R5-213460 sic@ // EXCEPTION: In Step 2, SS shall receive a RLC PDU and step 2 is repeated from SN=j*iteration_size to SN=(((j+1)*iteration_size)-1). // @siclog "Step 2" siclog@ // CHECK: Does UE transmit a RLC Data PDU with SN=0 for the first iteration and then incremented by 1 at each iteration? for (i := 0; i < p_InnerLoop; i:= i + 1) { // @sic R5-202613, R5-206326 sic@ <<< SKIPPED CODE >>>

After Change:

function fl_TC_7_1_2_3_3and4(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC { var integer v_RLC_SDUsize; //@sic R5s190358 sic@ var B1_Type v_Poll; var integer i; var integer v_ListOffset:=0; // @sic R5s210579 sic@ var integer j; var template (value) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU; var template (present) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_Expected; var integer v_VerdictFilter; // @sic R5-185083 sic@ var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; var NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_Received; var NR_L2DataList_Type v_NR_L2DataList; var integer v_LenPDUList; var template (value) NR_RLC_PDUList_Type v_RlcPduList; // @sic R5-213460 sic@ var SubFrameTiming_Type v_TimingCurrentSlot; // @sic R5-213460 sic@ var SubFrameTiming_Type v_TimingPreviousSlot; // @sic R5-213460 sic@ var integer k; if (p_RLC_Rec.AM_SN_Size == 18) { v_VerdictFilter := 4096; // @sic R5s200315 sic@ } else { //SN12 v_VerdictFilter := 256; } v_RLC_SDUsize := 4; // @sic R5-185083, R5s190358, R5-202613 sic@ v_TimingPreviousSlot := f_NR_GetCurrentTiming(nr_Cell1); // initialize slot @sic R5-213460 sic@ // EXCEPTION: Steps 1 - 3a1 shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration_size). for (j := 0; j < p_OuterLoop; j:= j + 1) { // @sic R5-197663, R5-197664, R5-202613 sic@ // generate p_InnerLoop RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, p_InnerLoop); // list offset is 0, TX_NEXT administered when generating // put those p_InnerLoop AMD PDUs into a list for transmission v_Poll := tsc_NR_P_NoPoll; // no indication of Poll bit usage in the prose for (i := 0; i < p_InnerLoop; i:= i + 1) { v_NR_RLC_AMD_PDU := f_NR_RLC_Get_AMD_FullSDU_TX(p_RLC_Rec, v_Poll, v_NR_RLC_DataList[i]); v_RlcPduList[i] := {AMD := v_NR_RLC_AMD_PDU}; // @sic R5-202613, R5-213460 sic@ } // @siclog "Step 1" siclog@ // SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size. Each RLC Data PDU contains one RLC SDU. f_NR_RLCPDUList_Send(nr_Cell1, p_NR_DRB_Id, v_RlcPduList); // @sic R5-213460 sic@ //v_TimingPreviousSlot := f_NR_GetCurrentTiming(nr_Cell1); // initialize slot @sic R5-213460 sic@ // EXCEPTION: In Step 2, SS shall receive a RLC PDU and step 2 is repeated from SN=j*iteration_size to SN=(((j+1)*iteration_size)-1). // @siclog "Step 2" siclog@ // CHECK: Does UE transmit a RLC Data PDU with SN=0 for the first iteration and then incremented by 1 at each iteration? for (i := 0; i < p_InnerLoop; i:= i + 1) { // @sic R5-202613, R5-206326 sic@

## Correction to function fl_TC_7_1_3_1_1and2

| Function name | fl_TC_7_1_3_1_1and2 |
| --- | --- |
| Reason for change | 1) In the current TTCN implementation of 7.1.3.1.1/2 Initialization of v_TimingPreviousSlot is done after the transmission of each PDU List, instead it is proposed to initialize v_TimingPreviousSlot at the beginning, then v_TimingPreviousSlot is set with v_TimingCurrentSlot after the reception of PDU List in each iteration 2) In the current TTCN implementation of 7.1.3.1.1/2, v_TimingCurrentSlot is set by querying current system time after the reception of PDU List , instead it is proposed to make use of v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame field to set v_TimingCurrentSlot |
| Summary of change | 1) Initialize v_TimingPreviousSlot at the beginning, then v_TimingPreviousSlot is set with v_TimingCurrentSlot after the reception of PDU List in each iteration 2) v_TimingCurrentSlot is set with the value received in v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame field to set v_TimingCurrentSlot |
| TTCN module | PDCP_TC_Common_NR |
| MCC160 Comment | 1) Accepted as an improvement 2) Accepted |

Before Change:

function fl_TC_7_1_3_1_1and2(NR_PDCP_SS_State_Type p_NR_PDCP_State, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC {//@sic R5-211387 sic@ var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var integer i:=0; var integer j:=0; var integer k:=0; var integer v_NumberOfPdu := p_InnerLoop; var integer v_SduSize := 1; var NR_PDCP_PDUList_Type v_PdcpPduList; var NR_PDCP_SDU_Type v_Sdu := f_GetN_OctetsFromPRBS (20,(v_SduSize)); var integer v_VerdictFilter; // @sic R5-211387 sic@ var NR_L2DataList_Type v_NR_L2DataList; var NR_PDCP_PDU_Type v_NR_PDCP_PDU_Received; var integer v_LenPDUList; var integer v_ListOffset:=0; // @sic R5s210579 sic@ var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var template (present) NR_PDCP_PDU_Type v_NR_PDCP_PDU_Expected; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; if (p_NR_PDCP_State.PDCP_SN_Size == PDCP_SNLength18) { v_VerdictFilter := 4096; // @sic R5-211387 sic@ } else { //SN12 v_VerdictFilter := 256; } // EXCEPTION: : Steps 1 - 2 shall be repeated from j = 0 to j = FLOOR(Maximum_PDCP_SN/iteration_size). (Note 1, 4) for (j := 0; j < p_OuterLoop; j:= j + 1) { // @sic R5-197663, R5-197664, R5-202613 sic@ //@siclog "Step 1" siclog@ // SS transmits in one slot several PDCP PDUs in a PDCP PDU list without header compression, the number of PDCP PDUs sent in each PDCP PDU list is defined by the iteration_size. (Note 4, 5). v_PdcpPduList := f_GeneratePdcpPDUList(p_NR_PDCP_State, v_NumberOfPdu,v_Sdu); f_NR_PDCPPDUList_Send(p_NR_PDCP_State,nr_Cell1,v_PdcpPduList); v_TimingPreviousSlot := f_NR_GetCurrentTiming(nr_Cell1); // initialize slot @sic R5-213462 sic@ // EXCEPTION: In Step 2, SS may receive a PDCP PDU or several PDCP PDUs, then step 2 may be repeated multiple times until all PDCP PDUs with SN=j*iteration_size to SN=(((j+1)* iteration_size)-1) for each iteration are received. // @siclog "Step 2" siclog@ // CHECK: Does UE transmit PDCP PDUs with SN=0 for the first iteration and all PDCP PDUs for each iteration?(Note 2)(Note 6) // @sic R5-213462 sic@ for (i := 0; i < p_InnerLoop; i:= i + 1) { // @sic R5-211387 sic@ // Get next list of PDCP PDUs received v_NR_L2DataList := f_NR_PDCP_PDU_GetList(p_NR_PDCP_State); // @sic R5-211387 sic@ v_TimingCurrentSlot := f_NR_GetCurrentTiming(nr_Cell1); // Note timing of the slot of the list received @sic R5-213462 sic@ f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); // both slots must be different @sic R5-213462 sic@ v_TimingPreviousSlot := v_TimingCurrentSlot; // Keep the info on the present slot @sic R5-213462 sic@ v_LenPDUList := lengthof(v_NR_L2DataList.PdcpPdu); v_ListOffset := i; // note i at start of PDU list @sic R5s210579 sic@ for (k := 0; k < v_LenPDUList; k := k + 1) { // set i to account for k i := v_ListOffset + k; // @sic R5-213462 R5s210579 sic@ if (i == p_InnerLoop) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, too many PDCP PDUs received");}; // check ok ??? // extract the PDCP PDU to be handled v_NR_PDCP_PDU_Received := v_NR_L2DataList.PdcpPdu[k]; // build the PDCP PDU expected (template) v_NR_PDCP_PDU_Expected := v_PdcpPduList[i]; // ith element in the currently used v_PdcpPduList // match the PDCP PDU to be handled against the PDCP PDU expected if (not match (v_NR_PDCP_PDU_Received, v_NR_PDCP_PDU_Expected)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, mismatch of received and expected PDUs"); }; if ((i == 0) and (j == 0)) { // SN = 0 f_NR_PreliminaryPass(__FILE__, __LINE__, " SN=0 "); } else { // SN = 1 .. if ((((j * p_InnerLoop) + i + 1) mod v_VerdictFilter) == 0) {f_NR_PreliminaryPass(__FILE__, __LINE__, " SN = " & int2str((j * p_InnerLoop) + i) )}; // @sic R5-185083, R5-187106, R5s200315, R5s200957 sic@ } } } } //@siclog "Step 3" siclog@ // SS transmits a PDCP PDU containing one PDCP SDU without header compression. f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_Sdu); //@siclog "Step 4" siclog@ // CHECK: Does UE transmit a PDCP PDU with SN=0? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_Sdu, cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //SS transmits a PDCP PDU containing one PDCP SDU without header compression. f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_Sdu); //@siclog "Step 6" siclog@ //CHECK: Does UE transmit a PDCP PDU with SN=1? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_Sdu, cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); }

After Change:

function fl_TC_7_1_3_1_1and2(NR_PDCP_SS_State_Type p_NR_PDCP_State, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC {//@sic R5-211387 sic@ var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var integer i:=0; var integer j:=0; var integer k:=0; var integer v_NumberOfPdu := p_InnerLoop; var integer v_SduSize := 1; var NR_PDCP_PDUList_Type v_PdcpPduList; var NR_PDCP_SDU_Type v_Sdu := f_GetN_OctetsFromPRBS (20,(v_SduSize)); var integer v_VerdictFilter; // @sic R5-211387 sic@ var NR_L2DataList_Type v_NR_L2DataList; var NR_PDCP_PDU_Type v_NR_PDCP_PDU_Received; var integer v_LenPDUList; var integer v_ListOffset:=0; // @sic R5s210579 sic@ var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var template (present) NR_PDCP_PDU_Type v_NR_PDCP_PDU_Expected; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; if (p_NR_PDCP_State.PDCP_SN_Size == PDCP_SNLength18) { v_VerdictFilter := 4096; // @sic R5-211387 sic@ } else { //SN12 v_VerdictFilter := 256; } v_TimingPreviousSlot := f_NR_GetCurrentTiming(nr_Cell1); // initialize slot @sic R5-213462 sic@ // EXCEPTION: : Steps 1 - 2 shall be repeated from j = 0 to j = FLOOR(Maximum_PDCP_SN/iteration_size). (Note 1, 4) for (j := 0; j < p_OuterLoop; j:= j + 1) { // @sic R5-197663, R5-197664, R5-202613 sic@ //@siclog "Step 1" siclog@ // SS transmits in one slot several PDCP PDUs in a PDCP PDU list without header compression, the number of PDCP PDUs sent in each PDCP PDU list is defined by the iteration_size. (Note 4, 5). v_PdcpPduList := f_GeneratePdcpPDUList(p_NR_PDCP_State, v_NumberOfPdu,v_Sdu); f_NR_PDCPPDUList_Send(p_NR_PDCP_State,nr_Cell1,v_PdcpPduList); //v_TimingPreviousSlot := f_NR_GetCurrentTiming(nr_Cell1); // initialize slot @sic R5-213462 sic@ // EXCEPTION: In Step 2, SS may receive a PDCP PDU or several PDCP PDUs, then step 2 may be repeated multiple times until all PDCP PDUs with SN=j*iteration_size to SN=(((j+1)* iteration_size)-1) for each iteration are received. // @siclog "Step 2" siclog@ // CHECK: Does UE transmit PDCP PDUs with SN=0 for the first iteration and all PDCP PDUs for each iteration?(Note 2)(Note 6) // @sic R5-213462 sic@ for (i := 0; i < p_InnerLoop; i:= i + 1) { // @sic R5-211387 sic@ // Get next list of PDCP PDUs received v_NR_L2DataList := f_NR_PDCP_PDU_GetList(p_NR_PDCP_State,v_TimingCurrentSlot); // @sic R5-211387 sic@ //v_TimingCurrentSlot := f_NR_GetCurrentTiming(nr_Cell1); // Note timing of the slot of the list received @sic R5-213462 sic@ f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); // both slots must be different @sic R5-213462 sic@ v_TimingPreviousSlot := v_TimingCurrentSlot; // Keep the info on the present slot @sic R5-213462 sic@ v_LenPDUList := lengthof(v_NR_L2DataList.PdcpPdu); v_ListOffset := i; // note i at start of PDU list @sic R5s210579 sic@ for (k := 0; k < v_LenPDUList; k := k + 1) { // set i to account for k i := v_ListOffset + k; // @sic R5-213462 R5s210579 sic@ if (i == p_InnerLoop) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, too many PDCP PDUs received");}; // check ok ??? // extract the PDCP PDU to be handled v_NR_PDCP_PDU_Received := v_NR_L2DataList.PdcpPdu[k]; // build the PDCP PDU expected (template) v_NR_PDCP_PDU_Expected := v_PdcpPduList[i]; // ith element in the currently used v_PdcpPduList // match the PDCP PDU to be handled against the PDCP PDU expected if (not match (v_NR_PDCP_PDU_Received, v_NR_PDCP_PDU_Expected)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, mismatch of received and expected PDUs"); }; if ((i == 0) and (j == 0)) { // SN = 0 f_NR_PreliminaryPass(__FILE__, __LINE__, " SN=0 "); } else { // SN = 1 .. if ((((j * p_InnerLoop) + i + 1) mod v_VerdictFilter) == 0) {f_NR_PreliminaryPass(__FILE__, __LINE__, " SN = " & int2str((j * p_InnerLoop) + i) )}; // @sic R5-185083, R5-187106, R5s200315, R5s200957 sic@ } } } } //@siclog "Step 3" siclog@ // SS transmits a PDCP PDU containing one PDCP SDU without header compression. f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_Sdu); //@siclog "Step 4" siclog@ // CHECK: Does UE transmit a PDCP PDU with SN=0? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_Sdu, cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //SS transmits a PDCP PDU containing one PDCP SDU without header compression. f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_Sdu); //@siclog "Step 6" siclog@ //CHECK: Does UE transmit a PDCP PDU with SN=1? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_Sdu, cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); }

## Correction to function f_NR_PDCP_PDU_GetList

| Function name | f_NR_PDCP_PDU_GetList |
| --- | --- |
| Reason for change | In the current TTCN implementation of 7.1.3.1.1/2, v_TimingCurrentSlot is set by querying current system time after the reception of PDU List , instead it is proposed to make use of v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame field to set v_TimingCurrentSlot |
| Summary of change | v_TimingCurrentSlot is set with the value received in v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame field to set v_TimingCurrentSlot – to achieve this the below function is introduced with the second parameter which outputs the time in which PDCP PDU list is received |
| TTCN module | PDCP_TC_Common_NR |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_PDCP_PDU_GetList(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC return NR_L2DataList_Type { // @sic R5-211387 R5s210395 sic@ var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var NR_L2DataList_Type v_NR_L2DataList; DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList(nr_Cell1, p_NR_PDCP_State.DRB_Id)) -> value v_NR_DRB_COMMON_IND; v_NR_L2DataList := v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList; return v_NR_L2DataList; }

After Change:

function f_NR_PDCP_PDU_GetList(NR_PDCP_SS_State_Type p_NR_PDCP_State, out SubFrameTiming_Type p_TimingCurrentSlot) runs on NR_BASE_PTC return NR_L2DataList_Type { // @sic R5-211387 R5s210395 sic@ var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var NR_L2DataList_Type v_NR_L2DataList; DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList(nr_Cell1, p_NR_PDCP_State.DRB_Id)) -> value v_NR_DRB_COMMON_IND; v_NR_L2DataList := v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList; p_TimingCurrentSlot := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; return v_NR_L2DataList; }