3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210689

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1783 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following implementation of R5-213465, the call to f_NR_ModifySysinfo() at step 21 should happen after the PRACH-configurationIndex has been updated at step 22, so that the updated system information is broadcast. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move call to f_NR_ModifySysinfo() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

Change 1

| Function name | fl_TC_8_1_1_4_1_TestBody |
| --- | --- |
| Reason for change | The call to f_NR_ModifySysinfo() at step 21 should happen after the PRACH-configurationIndex has been updated at step 22, so that the updated system information is broadcast. |
| Summary of change | Move call to f_NR_ModifySysinfo() |
| TTCN module | RRC_Resume_NR5GC.ttcn |
| MCC160 Comment | Accepted. A prose CR to TS 38.523-1 is required for RAN5#92-e. |

Before Change:

function fl_TC_8_1_1_4_1_TestBody() runs on NR5GC_PTC { ………………………… //@siclog "Step 21" siclog@ //@sic R5-213465 sic@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell2, true, RRC_INACTIVE); //@siclog "Step 22" siclog@ //@sic R5-213465 sic@ //The SS changes the prach-ConfigurationIndex in the system information. if (f_NR_CellInfo_GetIsFR1(nr_Cell2)){ v_PrachConfigurationIndex := 157; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //FR1 TDD - As per 38.211 Table 6.3.3.2-3 // nSFN mod 1 = 0=> any SFN Subframe 4 v_TimingInfo := cr_TimingInfo(?, (4)); } else { // FR2 v_PrachConfigurationIndex := 157; // slots (3,5,7) - As per 38.211 Table 6.3.3.2-4 v_TimingInfo := ( cr_TimingInfo(?, (3 / 8), -, cs_SlotTimingInfo_Numerology3(3 mod 8)), //Slot 3 cr_TimingInfo(?, (5 / 8), -, cs_SlotTimingInfo_Numerology3(5 mod 8)), //Slot 5 cr_TimingInfo(?, (7 / 8), -, cs_SlotTimingInfo_Numerology3(7 mod 8)) //Slot 7 ); } //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell2, v_PrachConfigurationIndex); //Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell2, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable)); //@siclog "Step 23" siclog@ //@sic R5-213465 sic@ //Wait for 2.1* modification period second for the UE to receive new system information. //Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. f_NR_WaitModificationPeriods(nr_Cell2); ……………………… }

After Change:

function fl_TC_8_1_1_4_1_TestBody() runs on NR5GC_PTC { …………………………………………… //@siclog "Step 21" siclog@ //@sic R5-213465 sic@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. //MOVED f_NR_ModifySysinfo(nr_Cell2, true, RRC_INACTIVE); //@siclog "Step 22" siclog@ //@sic R5-213465 sic@ //The SS changes the prach-ConfigurationIndex in the system information. if (f_NR_CellInfo_GetIsFR1(nr_Cell2)){ v_PrachConfigurationIndex := 157; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //FR1 TDD - As per 38.211 Table 6.3.3.2-3 // nSFN mod 1 = 0=> any SFN Subframe 4 v_TimingInfo := cr_TimingInfo(?, (4)); } else { // FR2 v_PrachConfigurationIndex := 157; // slots (3,5,7) - As per 38.211 Table 6.3.3.2-4 v_TimingInfo := ( cr_TimingInfo(?, (3 / 8), -, cs_SlotTimingInfo_Numerology3(3 mod 8)), //Slot 3 cr_TimingInfo(?, (5 / 8), -, cs_SlotTimingInfo_Numerology3(5 mod 8)), //Slot 5 cr_TimingInfo(?, (7 / 8), -, cs_SlotTimingInfo_Numerology3(7 mod 8)) //Slot 7 ); } //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell2, v_PrachConfigurationIndex); //Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell2, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_ModifySysinfo(nr_Cell2, true, RRC_INACTIVE); //@siclog "Step 23" siclog@ //@sic R5-213465 sic@ //Wait for 2.1* modification period second for the UE to receive new system information. //Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. f_NR_WaitModificationPeriods(nr_Cell2); ……………… }