3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210776

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1826 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-DC test case 8.2.5.2.2 in FR1+FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR-DC TC 8.2.5.2.2 Bearer Modification / Uplink Data Path / Split DRB Reconfiguration / NR-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR-DC test case 8.2.5.2.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications 38.523-1 |  |  |  | TS38.523-1, TS38.508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches Executed	5

4	Execution Log Files	5

4.1	Qualcomm SM 8450 + SDX65	5

5	References	5

- Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 8.2.5.2.2 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

  - Verification Test Summary

Test Case: 	8.2.5.2.2

ATS Version:	iwd-TTCN3-B2020-09_D21wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM 8450 + SDX65

Verification Status:	PASS

- Corrections required

Change 1,5,6 of R5s210716(Addition of NR-DC test case 8.2.2.4.2 in FR1+FR2 ) are also required in addition to the below changes.

  - Change 1

| Function name | f_TC_8_2_5_2_2_NR5GC() |
| --- | --- |
| Reason for change | It has been proposed to use NR-1 to be inline with real deployment. #draft proseCR would be raised for RAN5=92-e meeting To align with prose, RLF Timer for SCG should be set to 10000ms. |
| Summary of change | Change system info to NR_1 when initializing the NR cells Nr_Cell10(PSCell) is provided as input to f_NR_Set_RLF_TimersAndConstants_t311() |
| TTCN module | RRC_Reconfiguration_NRDC_NR5GC.ttcn |
| MCC160 Comment | 1. Accepted conditional to prose CR agreement at RAN5#92-e 2. Not needed |

Before Change:

function f_TC_8_2_5_2_2_NR5GC() runs on NR5GC_PTC { //Radio link failure / PSCell out of sync indication / NR-DC var template (omit) NR_BWP_DownlinkDedicated_RadioLinkMonitoringConfig_Type v_RadioLinkMonitoringConfig; f_NR5GC_Init_NRDC(NR_4, {nr_Cell1, nr_Cell10}); v_RadioLinkMonitoringConfig := cs_38508_RadioLinkMonitoringConfigDef(f_NR_CellInfo_GetSSB_Index(nr_Cell10)); f_NR_CellInfo_SetRadioLinkMonitoringConfig(nr_Cell1, v_RadioLinkMonitoringConfig); f_NR_CellInfo_SetRadioLinkMonitoringConfig(nr_Cell10, v_RadioLinkMonitoringConfig); f_NR_Set_RLF_TimersAndConstants_t311(nr_Cell1); //Create and configure NR cell1 and NR cell 10 f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower(nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_RRC_ConnectedState3N_NRDC(nr_Cell1, nr_Cell10, MCG_SCG); f_NR_TestBody_Set(true); f_TC_8_2_5_2_2_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change:

function f_TC_8_2_5_2_2_NR5GC() runs on NR5GC_PTC { //Radio link failure / PSCell out of sync indication / NR-DC var template (omit) NR_BWP_DownlinkDedicated_RadioLinkMonitoringConfig_Type v_RadioLinkMonitoringConfig; f_NR5GC_Init_NRDC(NR_1, {nr_Cell1, nr_Cell10}); v_RadioLinkMonitoringConfig := cs_38508_RadioLinkMonitoringConfigDef(f_NR_CellInfo_GetSSB_Index(nr_Cell10)); f_NR_CellInfo_SetRadioLinkMonitoringConfig(nr_Cell1, v_RadioLinkMonitoringConfig); f_NR_CellInfo_SetRadioLinkMonitoringConfig(nr_Cell10, v_RadioLinkMonitoringConfig); f_NR_Set_RLF_TimersAndConstants_t311(nr_Cell10); //set t311 for SCG, not MCG //Create and configure NR cell1 and NR cell 10 f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower(nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_RRC_ConnectedState3N_NRDC(nr_Cell1, nr_Cell10, MCG_SCG); f_NR_TestBody_Set(true); f_TC_8_2_5_2_2_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

- Branches Executed

The test case was executed on NR band DC_n79A_n257A.

- Execution Log Files

  - Qualcomm SM 8450 + SDX65

The Qualcomm SM 8450 + SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_2_5_2_2_LOG.xml

- PICS/PIXIT parameter file:

TC_8_2_5_2_2_PICS.txt

- References

| [1] | R5s210777 |
| --- | --- |