3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210788

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0730 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS call control test case 7.26 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Mobile Originating CAT / Forking Model / 5GS test case 7.26 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS registration test case 7.26 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.26.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

3	Branches executed	4

4	Execution Log Files	4

4.1	Qualcomm SM8350+SDX60	4

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.26 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

Shaun, Harry

shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	7.26.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

Keysight 5G Protocol Conformance Toolset

UE used:	Qualcomm SM8350+SDX60

Verification Status:	PASS

# Corrections required

## 2.1 Change 1

| Function name | fl_IMS_CAT_BuildSdpMessage |
| --- | --- |
| Reason for change | The default audio codec in step 9 183SessionProgress (in f_IMS_MOCallSetup_NR5GC_A_4_X_Step3_5) is EVS16000, but default codec in f_IMS_BuildSDP_InitialAnswerAudio_TX for checking the content of PRACK in step 10 is AMR which would cause mismatch in SDP check. |
| Summary of change | Changed the audio codec to be checked in step 10 PRACK |
| TTCN module | IMS_CC_CustomizedAlertingTonesCommon.ttcn |
| MCC160 Comment | Accepted in principle: In order not to break 4G test case 20.1 the parameter EVS_16000 needs to be handed through f_IMS_CAT_Send183SessionProgress and fl_IMS_CAT_BuildSdpMessage |

Before Change:

function fl_IMS_CAT_BuildSdpMessage(SDP_Message p_SDP_MessageInvite) runs on IMS_PTC return template (value) SDP_Message { /* @sic R5s150714: optimization see 4.4 sic@ */ var template (value) SDP_Message v_SDP_Message := f_IMS_BuildSDP_InitialAnswerAudio_TX(p_SDP_MessageInvite, tsc_SDP_WithoutPreconditions); /* preconditions to be added later on */ var template (omit) SDP_attribute_list v_SDP_MediaAttributes; var template (omit) SDP_attribute_list v_SDP_PreconditionAttributes; v_SDP_PreconditionAttributes := cs_SDP_PreconditionAttributes(c_sendrecv, c_none, c_mandatory, c_mandatory); v_SDP_PreconditionAttributes := f_SDP_Attributes_Add_TX(v_SDP_PreconditionAttributes, cs_SDP_Attribute_conf_qos); v_SDP_MediaAttributes := f_SDP_Attributes_Add_TX(v_SDP_Message.media_list[0].attributes, cs_SDP_Attribute_content("g.3gpp.cat")); v_SDP_Message.media_list[0].attributes := f_SDP_Attributes_Concat_TX(v_SDP_MediaAttributes, v_SDP_PreconditionAttributes); return v_SDP_Message; }

After Change:

function fl_IMS_CAT_BuildSdpMessage(SDP_Message p_SDP_MessageInvite) runs on IMS_PTC return template (value) SDP_Message { /* @sic R5s150714: optimization see 4.4 sic@ */ var template (value) SDP_Message v_SDP_Message := f_IMS_BuildSDP_InitialAnswerAudio_TX(p_SDP_MessageInvite, tsc_SDP_WithoutPreconditions, EVS_16000); /* preconditions to be added later on */ var template (omit) SDP_attribute_list v_SDP_MediaAttributes; var template (omit) SDP_attribute_list v_SDP_PreconditionAttributes; v_SDP_PreconditionAttributes := cs_SDP_PreconditionAttributes(c_sendrecv, c_none, c_mandatory, c_mandatory); v_SDP_PreconditionAttributes := f_SDP_Attributes_Add_TX(v_SDP_PreconditionAttributes, cs_SDP_Attribute_conf_qos); v_SDP_MediaAttributes := f_SDP_Attributes_Add_TX(v_SDP_Message.media_list[0].attributes, cs_SDP_Attribute_content("g.3gpp.cat")); v_SDP_Message.media_list[0].attributes := f_SDP_Attributes_Concat_TX(v_SDP_MediaAttributes, v_SDP_PreconditionAttributes); return v_SDP_Message; }

# Branches executed

The test case was executed on NR band n41 on Anritsu and Keysight.

# Execution Log Files

## Qualcomm SM8350+SDX60

The Qualcomm SM8350+SDX60  UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_7_26_LOG.xml

Keysight/TC_7_26_LOG.html

- PICS/PIXIT parameter file:

Anritsu/TC_7_26_PICS_PIXIT.txt

Keysight /TC_7_26_PICS_PIXIT.xml

# References

| [1] | R5s210789 |
| --- | --- |