3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210799

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4634 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EUTRA test frequencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | From 36.508 Table 4.3.1.2.16-1: Test frequencies for E-UTRA channel bandwidth for operating band 48, the value of Low Range test Frequency for ChBandwidth 20 is defined as 55340. Current implementation of function f_EUTRA_InitFrequencyLowMidHigh() sets this value to 56715. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct value of Low Range test Frequency for ChBandwidth 20 for Band 48 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All LTE test cases band 48 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_EUTRA_InitFrequencyLowMidHigh() |
| --- | --- |
| Reason for change | From 36.508 Table 4.3.1.2.16-1: Test frequencies for E-UTRA channel bandwidth for operating band 48, the value of Low Range test Frequency for ChBandwidth 20 is defined as 55340. Current implementation of function f_EUTRA_InitFrequencyLowMidHigh() sets this value to 56715. |
| Summary of change | Correct value of Low Range test Frequency for ChBandwidth 20 for Band 48 |
| TTCN module | EUTRA_CellInfoFrequencyRF.ttcn |
| MCC160 Comment | Accepted |

Before change

function f_EUTRA_InitFrequencyLowMidHigh(integer  p_Band,

Dl_Bandwidth_Type  p_DL_ChBandwidth,

Ul_Bandwidth_Type  p_UL_ChBandwidth ) return FrequencyLowMidHigh_Type

{

//@sic R5-163085 FrequencyBandindicator type changed to integer sic@

//@sic R5-165892 Band extension sic@

var template(value) FrequencyLowMidHigh_Type v_FreqLMH := cs_FrequencyLowMidHighDummy;

select (p_Band){

case (1) {   // Band 1

select (p_DL_ChBandwidth){

case (n25){  // ChBandwidth = 5

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 25;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 300;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 575;

}

case (n50){  // ChBandwidth = 10

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 50;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 300;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 550;

}

case (n75){  // ChBandwidth = 15

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 75;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 300;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 525;

}

case (n100){  // ChBandwidth = 20

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 100;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 300;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 500;

}

case else{    // ChBandwidth = 1.4 or 3 or else

FatalError (__FILE__, __LINE__, "invalid ChannelBandwidth");

}

};

…………………………………………

case (48) {     // Band 48 @sic R5-172672 sic@

select (p_DL_ChBandwidth){

case (n25){  // ChBandwidth = 5

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 55265;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 55990;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 56715;

}

case (n50){  // ChBandwidth = 10

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 55290;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 55990;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 56690;

}

case (n75){  // ChBandwidth = 15

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 55315;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 55990;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 56665;

}

case (n100){  // ChBandwidth = 20

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 56715;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 55990;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 56640;

}

case else{

FatalError (__FILE__, __LINE__, "invalid ChannelBandwidth");

}

}

}

After change

function f_EUTRA_InitFrequencyLowMidHigh(integer  p_Band,

Dl_Bandwidth_Type  p_DL_ChBandwidth,

Ul_Bandwidth_Type  p_UL_ChBandwidth ) return FrequencyLowMidHigh_Type

{

//@sic R5-163085 FrequencyBandindicator type changed to integer sic@

//@sic R5-165892 Band extension sic@

var template(value) FrequencyLowMidHigh_Type v_FreqLMH := cs_FrequencyLowMidHighDummy;

select (p_Band){

case (1) {   // Band 1

select (p_DL_ChBandwidth){

case (n25){  // ChBandwidth = 5

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 25;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 300;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 575;

}

case (n50){  // ChBandwidth = 10

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 50;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 300;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 550;

}

case (n75){  // ChBandwidth = 15

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 75;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 300;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 525;

}

case (n100){  // ChBandwidth = 20

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 100;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 300;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 500;

}

case else{    // ChBandwidth = 1.4 or 3 or else

FatalError (__FILE__, __LINE__, "invalid ChannelBandwidth");

}

};

……………………………………………………….

case (48) {     // Band 48 @sic R5-172672 sic@

select (p_DL_ChBandwidth){

case (n25){  // ChBandwidth = 5

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 55265;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 55990;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 56715;

}

case (n50){  // ChBandwidth = 10

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 55290;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 55990;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 56690;

}

case (n75){  // ChBandwidth = 15

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 55315;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 55990;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 56665;

}

case (n100){  // ChBandwidth = 20

v_FreqLMH.FrequencyLow.dl_CarrierFreq := 55340;

v_FreqLMH.FrequencyMid.dl_CarrierFreq := 55990;

v_FreqLMH.FrequencyHigh.dl_CarrierFreq := 56640;

}

case else{

FatalError (__FILE__, __LINE__, "invalid ChannelBandwidth");

}

}

}