3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210804

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1838 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In the current TTCN implementation, there is no rrcConnection establishment before De-registration at Step 2A3a1. Since the UE is in idle mode after preamble, the UE will perform a new rrcConnection procedure for De-registration. This needs to be handled. Also the timer of 5s is started at Step 2A2 within which the De-registration REQ is expected. If he testcase is run in manual mode, this timer is too short for the operator. The current TTCN also releases the rrc connection at Step 2A4. This is also incorrect. If the UE does not trigger a De-registration REQ, then there is no rrc Conection established, and thus there is nothing to be released. The rrc Release should only happen if the UE established a new rrcConnection for De-registration. This needs to be addressed. - Before Step 2A5, where the UE is Switched ON, the functions f_NR5GC_MobileInfo_Init and f_NR5GC_SwitchOnAndResetIMS should be called so that the DRB info, PDU Session Info, IMS parameters etc would be reset. If this is not done it can cause problems later in the testcase. - At Step 16, the SS transmits the rrcReject, and then at Step 17 the user triggers an IMS emergency call. However, from SS point of view the RACH procedure has been successful when the rrcReject is sent, and thus SS will not be expecting a new RACH when the user initiates an IMS emergency call. This needs to be handled. - In curren TTCN implementation, a timer t-WaitTime is started after Step 16, but is never stopped/cancelled. This needs to be addressed. - In the current TTCN implementation, after the Steps 18 – 32 where the emergency call is established, the rrcConnection is released without releasing the emergency call at Step 33. This needs to be addressed. - After the rrcRelease at Step 33, the IP handling for PDN_2 should be stopped. This needs to be implemented. - This testcase should be included in the function f_GetTestcaseAttrib_EarlyContentionResolution, otherwise a preconfigured rrcSetup will be sent out, thus causing issues when a rrcReject is to be sent at Step 18. This needs to be implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Increased the timer at Step 2A2 to 15s. Implemented the TTCN to handle the rrcConnection before Step 2A3a1. Also moved the rrcRelease after Step 2A3a1. A Prose CR will be raised for this. - Called functions f_NR5GC_MobileInfo_Init and f_NR5GC_SwitchOnAndResetIMS at Step 2A5 instead of just calling f_UT_SwitchOnUE. - Called function f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig); after Step 16. A Prose CR will be raised for this. - Called t_WaitTime.stop; after the call f_UT_RequestIMSEmergencyCall. Please see below. - Called function f_NR5GC_EmergencyCallRelease before Step 33. A Prose CR will be raised for this. - Called function f_IP_Handling_StopPDN after Step 33. Please see below. - Included the tetscase 11.3.2 in the function f_GetTestcaseAttrib_EarlyContentionResolution. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the tetscase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to function fl_TC_11_3_2_TestBody	3

1.2	Correction to function f_GetTestcaseAttrib_EarlyContentionResolution	13

# Corrections required

## Correction to function fl_TC_11_3_2_TestBody

| Function name | fl_TC_11_3_2_TestBody |
| --- | --- |
| Reason for change | - In the current TTCN implementation, there is no rrcConnection establishment before De-registration at Step 2A3a1. Since the UE is in idle mode after preamble, the UE will perform a new rrcConnection procedure for De-registration. This needs to be handled. Also the timer of 5s is started at Step 2A2 within which the De-registration REQ is expected. If he testcase is run in manual mode, this timer is too short for the operator. The current TTCN also releases the rrc connection at Step 2A4. This is also incorrect. If the UE does not trigger a De-registration REQ, then there is no rrc Conection established, and thus there is nothing to be released. The rrc Release should only happen if the UE established a new rrcConnection for De-registration. This needs to be addressed. - Before Step 2A5, where the UE is Switched ON, the functions f_NR5GC_MobileInfo_Init and f_NR5GC_SwitchOnAndResetIMS should be called so that the DRB info, PDU Session Info, IMS parameters etc would be reset. If this is not done it can cause problems later in the testcase. - At Step 16, the SS transmits the rrcReject, and then at Step 17 the user triggers an IMS emergency call. However, from SS point of view the RACH procedure has been successful when the rrcReject is sent, and thus SS will not be expecting a new RACH when the user initiates an IMS emergency call. This needs to be handled. - In curren TTCN implementation, a timer t-WaitTime is started after Step 16, but is never stopped/cancelled. This needs to be addressed. - In the current TTCN implementation, after the Steps 18 – 32 where the emergency call is established, the rrcConnection is released without releasing the emergency call at Step 33. This needs to be addressed. - After the rrcRelease at Step 33, the IP handling for PDN_2 should be stopped. This needs to be implemented. |
| Summary of change | - Increased the timer at Step 2A2 to 15s. Implemented the TTCN to handle the rrcConnection before Step 2A3a1. Also moved the rrcRelease after Step 2A3a1. A Prose CR will be raised for this. - Called functions f_NR5GC_MobileInfo_Init and f_NR5GC_SwitchOnAndResetIMS at Step 2A5 instead of just calling f_UT_SwitchOnUE. - Called function f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig); after Step 16. A Prose CR will be raised for this. - Called t_WaitTime.stop; after the call f_UT_RequestIMSEmergencyCall. Please see below. - Called function f_NR5GC_EmergencyCallRelease before Step 33. A Prose CR will be raised for this. - Called function f_IP_Handling_StopPDN after Step 33. Please see below. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | - Accepted. Implemented according R5-216191 - Accepted - Rejected.Not needed - Accepted - Accepted. Implemented according R5-216191 - Accepted |

Before Change:

function fl_TC_11_3_2_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var RejectWaitTime v_RejectWaitTime := 16; var float v_WaitForConnectionMin := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_RejectWaitTime)); var float v_TimerValue := 30.0; timer t_WaitTime; //@siclog "Step 1" siclog@ // Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1) f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' within 30 s'? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_TimerValue, cr_NR_EstablishmentCause_emergency, "Step 2"); //@sic R5-213505 sic@ //@siclog "Step 2A1" siclog@ //IF pc_SwitchOnOff THEN switch off UE, ELSE IF pc_USIM_Removal THEN remove the USIM, ELSE power off UE in which case steps 2A2 to 2A4 are not performed if(pc_SwitchOnOff){ f_UT_SwitchOffUE(UT, true); //@siclog "Step 2A2" siclog@ //SS starts timer1 = 5 sec to see if UE performs optional Step 10a3a1 (Note 5) t_WaitTime.start(5.0); //@siclog "Steps 2A3a1-2A4" siclog@ //The UE transmits a DEREGISTRATION REQUEST message. Stop timer T_Delay //Timer T_Delay expires. SS locally releases RRC connection alt { [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp{ t_WaitTime.stop; if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } f_NR_RRCRelease(nr_Cell1); } [] t_WaitTime.timeout { f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1), tsc_CnfReq); } } } //@siclog "Step 2A5" siclog@ //ELSE power off UE else { f_UT_PowerOffUE (UT); } //@siclog "Step 2A6" siclog@ //The UE performs Registration procedure as specified in TS 38.508-1 [4] subclause 4.5.2. //@siclog "Step 2A5" siclog@ //Switch on the UE f_UT_SwitchOnUE(UT); //@siclog "Step 2A6" siclog@ f_NR5GC_RRC_IdleState1N_Def(nr_Cell1); //@siclog "Step 3" siclog@ //SS changes SIB1 according to Table 11.3.2.3.3-2 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect and T390 expire. (Note 1) // Table 11.3.2.3.3-2: SIB1 for NR Cell 1 f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_7Entry, //@sic R5-205321 sic@ -, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 4" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 5" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'mt-access' ? f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); //@siclog "Steps 6-12" siclog@ // Steps 3 to 9a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0010'B)) { // @sic R5s200654 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); f_NR_PreliminaryPass(__FILE__, __LINE__, "Check 5GC RRC_IDLE"); f_NR_RRCRelease(nr_Cell1); //@siclog "Step 13" siclog@ // SS changes SIB1 according to 38.508-1 [4] Table 4.6.1-28 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Steps 14-15" siclog@ //Steps 1-2 of the NR RRC_CONNECTED procedure in table 4.5.4.2-3 in TS 38.508-1[4] are performed. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 16" siclog@ //The SS responds with RRCReject message with IE waitTime set to 10s. SRB.send(cas_NR_SRB0_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCReject(v_RejectWaitTime))); t_WaitTime.start(v_WaitForConnectionMin); //@siclog "Step 17" siclog@ // Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1) //NOTE 1: This could be done by e.g. MMI or AT command. f_UT_RequestIMSEmergencyCall (UT); //@sic R5-213505 sic@ //@siclog "Step 18 -32" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' (Note 2)? //Step 2-13 for IMS Emergency call establishment are performed as specified in TS 38.508-1 [4], subclause 4.9.11 on NR Cell 1. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, emergency); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0011'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); // Steps 8-13 f_NR5GC_EmergencyCallPDUSession(nr_Cell1); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); //@siclog "Step 33" siclog@ //SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. f_NR_RRCRelease(nr_Cell1); }

After Change:

import from NR_ASP_TypeDefs all; //WA#11_3_2 import from IP_PTC_CtrlMsgs all; //WA#11_3_2 . . . function fl_TC_11_3_2_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var RejectWaitTime v_RejectWaitTime := 16; var float v_WaitForConnectionMin := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_RejectWaitTime)); var float v_TimerValue := 30.0; var NG_NAS_MSG_Indication_Type v_DetachReq; //WA#11_3_2 var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1); //WA#11_3_2 timer t_WaitTime; //@siclog "Step 1" siclog@ // Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1) f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' within 30 s'? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_TimerValue, cr_NR_EstablishmentCause_emergency, "Step 2"); //@sic R5-213505 sic@ //@siclog "Step 2A1" siclog@ //IF pc_SwitchOnOff THEN switch off UE, ELSE IF pc_USIM_Removal THEN remove the USIM, ELSE power off UE in which case steps 2A2 to 2A4 are not performed if(pc_SwitchOnOff) { f_UT_SwitchOffUE(UT, true); //@siclog "Step 2A2" siclog@ //SS starts timer1 = 5 sec to see if UE performs optional Step 10a3a1 (Note 5) t_WaitTime.start(15.0); //WA#11_3_2 5s-->15s //@siclog "Steps 2A3a1-2A4" siclog@ //The UE transmits a DEREGISTRATION REQUEST message. Stop timer T_Delay //Timer T_Delay expires. SS locally releases RRC connection /*WA#11_3_2 COMMENTED OUT alt { [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp{ t_WaitTime.stop; if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } f_NR_RRCRelease(nr_Cell1); } [] t_WaitTime.timeout { f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1), tsc_CnfReq); } }*/ alt{ //@siclog "Step 2A3a1 - 2A3a2" siclog@ [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp { t_WaitTime.stop; f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), tsc_SHT_IntegrityProtected); v_DetachReq := fl_NR5GC_OptionalIMSDeregisterOnSwitchOff(nr_Cell1, v_ReceivedMsg); //@siclog "Step 2A4" siclog@ f_NR_RRCRelease(nr_Cell1); } //@siclog "Step 2A3b1" siclog@ [] t_WaitTime.timeout{} } } //@siclog "Step 2A5" siclog@ //ELSE power off UE else { f_UT_PowerOffUE (UT); } //@siclog "Step 2A6" siclog@ //The UE performs Registration procedure as specified in TS 38.508-1 [4] subclause 4.5.2. f_NR5GC_MobileInfo_Init(); //WA#11_3_2 //@siclog "Step 2A5" siclog@ //Switch on the UE //WA#11_3_2 COMMENTED OUT f_UT_SwitchOnUE(UT); f_NR5GC_SwitchOnAndResetIMS(); //WA#11_3_2 //@siclog "Step 2A6" siclog@ f_NR5GC_RRC_IdleState1N_Def(nr_Cell1); //@siclog "Step 3" siclog@ //SS changes SIB1 according to Table 11.3.2.3.3-2 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect and T390 expire. (Note 1) // Table 11.3.2.3.3-2: SIB1 for NR Cell 1 f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_7Entry, //@sic R5-205321 sic@ -, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 4" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 5" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'mt-access' ? f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); //@siclog "Steps 6-12" siclog@ // Steps 3 to 9a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0010'B)) { // @sic R5s200654 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); f_NR_PreliminaryPass(__FILE__, __LINE__, "Check 5GC RRC_IDLE"); f_NR_RRCRelease(nr_Cell1); //@siclog "Step 13" siclog@ // SS changes SIB1 according to 38.508-1 [4] Table 4.6.1-28 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Steps 14-15" siclog@ //Steps 1-2 of the NR RRC_CONNECTED procedure in table 4.5.4.2-3 in TS 38.508-1[4] are performed. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 16" siclog@ //The SS responds with RRCReject message with IE waitTime set to 10s. SRB.send(cas_NR_SRB0_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCReject(v_RejectWaitTime))); t_WaitTime.start(v_WaitForConnectionMin); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig); //WA#11_3_2 //@siclog "Step 17" siclog@ // Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1) //NOTE 1: This could be done by e.g. MMI or AT command. f_UT_RequestIMSEmergencyCall (UT); t_WaitTime.stop; //WA#11_3_2 3 //@sic R5-213505 sic@ //@siclog "Step 18 -32" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' (Note 2)? //Step 2-13 for IMS Emergency call establishment are performed as specified in TS 38.508-1 [4], subclause 4.9.11 on NR Cell 1. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, emergency); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0011'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); // Steps 8-13 f_NR5GC_EmergencyCallPDUSession(nr_Cell1); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); f_NR5GC_EmergencyCallRelease(nr_Cell1); //WA#11_3_2 //@siclog "Step 33" siclog@ //SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. f_NR_RRCRelease(nr_Cell1); f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_2)); //WA#11_3_2 }

## Correction to function f_GetTestcaseAttrib_EarlyContentionResolution

| Function name | f_GetTestcaseAttrib_EarlyContentionResolution |
| --- | --- |
| Reason for change | This testcase should be included in the function f_GetTestcaseAttrib_EarlyContentionResolution, otherwise a preconfigured rrcSetup will be sent out, thus causing issues when a rrcReject is to be sent at Step 18. This needs to be implemented. |
| Summary of change | Included the tetscase 11.3.2 in the function f_GetTestcaseAttrib_EarlyContentionResolution. Please see below. |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_6_NR5GC") { return true; } case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } case ("TC_7_1_1_1_5_NR5GC") { return true; } case ("TC_8_1_1_1_1_NR5GC") { return true; } case ("TC_8_1_1_2_1_NR5GC") { return true; } case ("TC_8_1_1_2_3_NR5GC") { return true; } case ("TC_8_1_1_4_1_NR5GC") { return true; } case ("TC_8_1_1_4_2_NR5GC") { return true; } case ("TC_8_1_2_1_4_NR5GC") { return true; } case ("TC_8_1_3_1_23_NR5GC") { return true; } case ("TC_8_1_4_1_5_NR5GC") { return true; } case ("TC_8_1_4_1_6_NR5GC") { return true; } case ("TC_8_1_5_6_1_NR5GC") { return true; } case ("TC_8_1_5_8_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_2_NR5GC") { return true; } case ("TC_8_1_5_8_2_3_NR5GC") { return true; } case ("TC_8_1_6_1_3_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_1_NR5GC") { return true; } case ("TC_8_1_6_1_4_2_NR5GC") { return true; } case ("TC_8_1_6_1_4_4_NR5GC") { return true; } } return false; }

After Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_6_NR5GC") { return true; } case ("TC_11_3_2_NR5GC") { return true; } //WA#11_3_2 case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } case ("TC_7_1_1_1_5_NR5GC") { return true; } case ("TC_8_1_1_1_1_NR5GC") { return true; } case ("TC_8_1_1_2_1_NR5GC") { return true; } case ("TC_8_1_1_2_3_NR5GC") { return true; } case ("TC_8_1_1_4_1_NR5GC") { return true; } case ("TC_8_1_1_4_2_NR5GC") { return true; } case ("TC_8_1_2_1_4_NR5GC") { return true; } case ("TC_8_1_3_1_23_NR5GC") { return true; } case ("TC_8_1_4_1_5_NR5GC") { return true; } case ("TC_8_1_4_1_6_NR5GC") { return true; } case ("TC_8_1_5_6_1_NR5GC") { return true; } case ("TC_8_1_5_8_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_2_NR5GC") { return true; } case ("TC_8_1_5_8_2_3_NR5GC") { return true; } case ("TC_8_1_6_1_3_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_1_NR5GC") { return true; } case ("TC_8_1_6_1_4_2_NR5GC") { return true; } case ("TC_8_1_6_1_4_4_NR5GC") { return true; } } return false; }