3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210849

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0735 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eCall over IMS test case 21.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To verify eCall over IMS test case 21.4 using IWD 21wk24 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to eCall over IMS test case 21.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 34.229-1 CR 1473 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	f_ECall_MSDUpdate_AnnexC47	3

2.2	cs_EmergencyCallData_Control_Request	5

3	Branches executed	6

4	Execution Log Files	6

4.1      Qualcomm SA515M	6

5	References	6

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of IMS test case 21.4 which is part of the iwd-TTCN3-B2020-09_D21wk24.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case:	21.4

ATS Version:	iwd-TTCN3-B2020-09_D21wk24.zip

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SA515M

Verification Status:	PASS

# Corrections required

## f_ECall_MSDUpdate_AnnexC47

| Function name | f_ECall_MSDUpdate_AnnexC47 |
| --- | --- |
| Reason for change | Invalid cidUrl and incorrect contactUrl expected |
| Summary of change | Corrected the expected value of cidUrl and contactUrl |
| TTCN module | IMS_eCall_Functions.ttcn |
| MCC160 Comment | - cid-url Accepted in principle (see below) subject to approval of draft prose CR - v_ContactUrlUEExpected Accepted in principle according to R5-216339 |

Before change

function f_ECall_MSDUpdate_AnnexC47(template (value) EmergencyCallData_Control p_EmergencyCallData_Control_Step4 := cs_EmergencyCallData_Control_Request) runs on IMS_PTC

{

var IMS_DATA_REQ v_IMS_DATA_REQ;

var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL := cs_IMS_RoutingInfo(f_IMS_PTC_ImsInfo_GetTransportProtocol());

var template (value) MessageHeader v_MessageHeader_Step4;

var template (value) MessageBody v_MessageBody_Step4;

var template (present) MessageHeader v_MessageHeader_Step6;

var InternetProtocol_Type v_Protocol;

var SipUrl v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog);

var charstring v_BoundaryStep4 := "boundaryXXX";

var charstring v_CidUrl_Step4 := "test-info@3gpp.org";

var charstring v_CidUrl_Step6;

// Step 4. SS sends SIP_INFO to request MSD update

v_MessageHeader_Step4 := f_IMS_ECall_InfoRequest_MessageHeaderTX(v_BoundaryStep4, v_CidUrl_Step4);

v_MessageBody_Step4 := f_IMS_ECall_MIME_MessageBodyTX(v_BoundaryStep4, v_CidUrl_Step4, p_EmergencyCallData_Control_Step4);

IMS_Client.send(cas_IMS_Info_Request(v_RoutingInfo_DL, cs_INFO_Request(v_ContactUrlUE, v_MessageHeader_Step4, v_MessageBody_Step4)));

//Step 5. UE responds with 200 OK

v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol();

f_IMS_ReceiveResponse(v_MessageHeader_Step4, v_Protocol, cr_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Step4)));

// Step 6. UE sends SIP_INFO with MSD update.

v_MessageHeader_Step6 := f_IMS_ECall_InfoRequest_MessageHeaderRX();

v_IMS_DATA_REQ := f_IMS_ReceiveRequest(car_IMS_Info_Request(cr_INFO_Request(v_ContactUrlUE, v_MessageHeader_Step6, cr_MessageBody_MIME)));

v_CidUrl_Step6 := v_IMS_DATA_REQ.Request.Info.msgHeader.callInfo.callInfoBody[0].url;

f_IMS_MessageHeader_Request_CheckVia(v_IMS_DATA_REQ.Request.Bye.msgHeader, v_IMS_DATA_REQ.RoutingInfo.Protocol);

if (match(tsc_EmergencyCallData_DataType_invalidMSD, p_EmergencyCallData_Control_Step4.choice.request.datatype)) {

// Special case for TC 21.6: A.2.19 with condition A4

f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_IMS_DATA_REQ.Request.Info.messageBody.mimeMessageBody,

tsc_MimeType_EmergencyCallDataControlXML,

v_CidUrl_Step6,

-,

cr_EmergencyCallData_Control_Ack(true, v_CidUrl_Step4));

} else {

// Normal case: UE sends MSD according to A.2.19 with condition A3

f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_IMS_DATA_REQ.Request.Info.messageBody.mimeMessageBody,

tsc_MimeType_EmergencyCallDataMSD,

v_CidUrl_Step6);

}

//Step 7 SS responds to INFO with 200 Ok

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderTX(v_IMS_DATA_REQ.Request.Info.msgHeader))));

}

After change

function f_ECall_MSDUpdate_AnnexC47(template (value) EmergencyCallData_Control p_EmergencyCallData_Control_Step4 := cs_EmergencyCallData_Control_Request) runs on IMS_PTC
{
var IMS_DATA_REQ v_IMS_DATA_REQ;
var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL := cs_IMS_RoutingInfo(f_IMS_PTC_ImsInfo_GetTransportProtocol());
var template (value) MessageHeader v_MessageHeader_Step4;
var template (value) MessageBody v_MessageBody_Step4;
var template (present) MessageHeader v_MessageHeader_Step6;
var InternetProtocol_Type v_Protocol;
var SipUrl v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog);
var charstring v_BoundaryStep4 := "boundaryXXX";
var charstring v_CidUrl_Step4 := "test-info@3gpp.org";
var charstring v_CidUrl_Step4_header := "<cid:test-info@3gpp.org>";
var charstring v_CidUrl_Step6;

var charstring p_cURI := px_IMS_CalleeContactUri;
var SipUrl v_ContactUrlUEExpected := f_SIP_BuildSipUri_TX(p_cURI);

// Step 4. SS sends SIP_INFO to request MSD update
v_MessageHeader_Step4 := f_IMS_ECall_InfoRequest_MessageHeaderTX(v_BoundaryStep4, v_CidUrl_Step4_header);
v_MessageBody_Step4 := f_IMS_ECall_MIME_MessageBodyTX(v_BoundaryStep4, v_CidUrl_Step4, p_EmergencyCallData_Control_Step4);
IMS_Client.send(cas_IMS_Info_Request(v_RoutingInfo_DL, cs_INFO_Request(v_ContactUrlUE, v_MessageHeader_Step4, v_MessageBody_Step4)));

//Step 5. UE responds with 200 OK
v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol();
f_IMS_ReceiveResponse(v_MessageHeader_Step4, v_Protocol, cr_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Step4)));

// Step 6. UE sends SIP_INFO with MSD update.
v_MessageHeader_Step6 := f_IMS_ECall_InfoRequest_MessageHeaderRX();
v_IMS_DATA_REQ := f_IMS_ReceiveRequest(car_IMS_Info_Request(cr_INFO_Request(v_ContactUrlUEExpected, v_MessageHeader_Step6, cr_MessageBody_MIME)));
v_CidUrl_Step6 := v_IMS_DATA_REQ.Request.Info.msgHeader.callInfo.callInfoBody[0].url;

f_IMS_MessageHeader_Request_CheckVia(v_IMS_DATA_REQ.Request.Bye.msgHeader, v_IMS_DATA_REQ.RoutingInfo.Protocol);

if (match(tsc_EmergencyCallData_DataType_invalidMSD, p_EmergencyCallData_Control_Step4.choice.request.datatype)) {
// Special case for TC 21.6: A.2.19 with condition A4
f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_IMS_DATA_REQ.Request.Info.messageBody.mimeMessageBody,
tsc_MimeType_EmergencyCallDataControlXML,
v_CidUrl_Step6,
-,
cr_EmergencyCallData_Control_Ack(true, v_CidUrl_Step4));
} else {
// Normal case: UE sends MSD according to A.2.19 with condition A3
f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_IMS_DATA_REQ.Request.Info.messageBody.mimeMessageBody,
tsc_MimeType_EmergencyCallDataMSD,
v_CidUrl_Step6);
}

//Step 7 SS responds to INFO with 200 Ok
IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderTX(v_IMS_DATA_REQ.Request.Info.msgHeader))));
}

MCC160 Implementation

function f_ECall_MSDUpdate_AnnexC47(template (value) EmergencyCallData_Control p_EmergencyCallData_Control_Step4 := cs_EmergencyCallData_Control_Request, template (present) SipUrl p_ContactUriSS := f_SIP_BuildSipUri_RX(px_IMS_CalleeContactUri)) runs on IMS_PTC { /* @sic R5-216339: p_ContactUriSS - same URI as in 200 OK's Contact sent by the SS (see f_ECall_CallSetup_AnnexC47) sic@ */ var IMS_DATA_REQ v_IMS_DATA_REQ; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL := cs_IMS_RoutingInfo(f_IMS_PTC_ImsInfo_GetTransportProtocol()); var template (value) MessageHeader v_MessageHeader_Step4; var template (value) MessageBody v_MessageBody_Step4; var template (present) MessageHeader v_MessageHeader_Step6; var InternetProtocol_Type v_Protocol; var SipUrl v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); var charstring v_BoundaryStep4 := "boundaryXXX"; var charstring v_ContentId := "test-info@3gpp.org"; var charstring v_CidUrl_Step4 := "cid:" & v_ContentId; var charstring v_CidUrl_Step6; var boolean v_ConditionA4 := match(tsc_EmergencyCallData_DataType_invalidMSD, p_EmergencyCallData_Control_Step4.choice.request.datatype); // Step 4. SS sends SIP_INFO to request MSD update v_MessageHeader_Step4 := f_IMS_ECall_InfoRequest_MessageHeaderTX(v_BoundaryStep4, v_CidUrl_Step4); v_MessageBody_Step4 := f_IMS_ECall_MIME_MessageBodyTX(v_BoundaryStep4, v_ContentId, p_EmergencyCallData_Control_Step4); IMS_Client.send(cas_IMS_Info_Request(v_RoutingInfo_DL, cs_INFO_Request(v_ContactUrlUE, v_MessageHeader_Step4, v_MessageBody_Step4))); //Step 5. UE responds with 200 OK v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); f_IMS_ReceiveResponse(v_MessageHeader_Step4, v_Protocol, cr_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Step4))); // Step 6. UE sends SIP_INFO with MSD update. v_MessageHeader_Step6 := f_IMS_ECall_InfoRequest_MessageHeaderRX(v_ConditionA4); /* @sic R5-216339: v_ConditionA4 sic@ */ v_IMS_DATA_REQ := f_IMS_ReceiveRequest(car_IMS_Info_Request(cr_INFO_Request(p_ContactUriSS, v_MessageHeader_Step6, cr_MessageBody_MIME))); /* @sic R5-216339: p_ContactUriSS sic@ */ v_CidUrl_Step6 := v_IMS_DATA_REQ.Request.Info.msgHeader.callInfo.callInfoBody[0].url; f_IMS_MessageHeader_Request_CheckVia(v_IMS_DATA_REQ.Request.Bye.msgHeader, v_IMS_DATA_REQ.RoutingInfo.Protocol); if (v_ConditionA4) { // Special case for TC 21.6: A.2.19 with condition A4 f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_IMS_DATA_REQ.Request.Info.messageBody.mimeMessageBody, tsc_MimeType_EmergencyCallDataControlXML, v_CidUrl_Step6, -, cr_EmergencyCallData_Control_Ack(v_ContentId)); } else { // Normal case: UE sends MSD according to A.2.19 with condition A3 f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_IMS_DATA_REQ.Request.Info.messageBody.mimeMessageBody, tsc_MimeType_EmergencyCallDataMSD, v_CidUrl_Step6); } //Step 7 SS responds to INFO with 200 Ok IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderTX(v_IMS_DATA_REQ.Request.Info.msgHeader)))); }

function f_IMS_ECall_MIME_MessageBodyTX(charstring p_Boundary, charstring p_XmlPartContentId, template (value) EmergencyCallData_Control p_EmergencyCallData_Control, template (omit) SDP_Message p_SDP_Message := omit) return template (value) MessageBody { var template (value) MIME_Part_List v_MIME_Part_List := {}; var XmlBody v_XmlBody := f_IMS_ECall_EmergencyCallData_Control_Encode(p_EmergencyCallData_Control); var charstring v_MsgId := "<" & p_XmlPartContentId & ">"; v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_EmergencyCallDataControlXML, cs_MimeContentDisposition("by-reference"), cs_MimeContentId(v_MsgId), v_XmlBody); if (isvalue(p_SDP_Message)) { v_MIME_Part_List := f_MIME_Part_List_AddSDP(v_MIME_Part_List, valueof(p_SDP_Message)); } return cs_MessageBody_MIME(cs_MIME_Message(p_Boundary, v_MIME_Part_List)); }

template (value) CallInfo cs_CallInfo(charstring p_CidUrl, charstring p_Purpose) := { /* @status APPROVED (IMS) */ fieldName := CALL_INFO_E, callInfoBody := { { url := "<" & p_CidUrl & ">", infoParams := { cs_GenericParam("purpose", p_Purpose) } } } };

## cs_EmergencyCallData_Control_Request

| Function name | cs_EmergencyCallData_Control_Request |
| --- | --- |
| Reason for change | Incorrected expected value |
| Summary of change | Corrected the expected value of “action” field |
| TTCN module | IMS_XML_eCall_Templates.ttcn |
| MCC160 Comment | Accepted |

Before change

template (value) EmergencyCallData_Control cs_EmergencyCallData_Control_Request(XSD.Token p_Action := "send_data",
XSD.Token p_DataType := tsc_EmergencyCallData_DataType_eCallMSD) :=
{ /* @status APPROVED (IMS) */
attr := omit,
choice := {
request := {
action_ := p_Action,
datatype := p_DataType,
element_id := omit,
int_id := omit,
persistence := omit,
requested_state := omit,
supported_values := omit,
attr := omit,
choice_list := {
cs_EmergencyCallData_Request_Choice_TextList_0
}
}
}
};

After change

template (value) EmergencyCallData_Control cs_EmergencyCallData_Control_Request(XSD.Token p_Action := "send-data",
XSD.Token p_DataType := tsc_EmergencyCallData_DataType_eCallMSD) :=
{ /* @status APPROVED (IMS) */
attr := omit,
choice := {
request := {
action_ := p_Action,
datatype := p_DataType,
element_id := omit,
int_id := omit,
persistence := omit,
requested_state := omit,
supported_values := omit,
attr := omit,
choice_list := {
cs_EmergencyCallData_Request_Choice_TextList_0
}
}
}
};

# Branches executed

The test case was executed on EUTRA band 4

# Execution Log Files

## Qualcomm SA515M

The Qualcomm SA515M IMS UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_21_4_LOG.html

In the log file (in .html format) the complete test case execution can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- PICS/PIXIT parameter file:

TC_21_4_PIXIT.xml

# References

| [1] | R5s210850: Supporting information for addition of eCall over IMS test case 21.4 |
| --- | --- |