3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210869

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1874 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EN-DC RRC test case 8.2.2.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the reconfigurations at steps 7 and 16, The UE will perofrm a RACH procedure on the NR SCG cell (since ReconfigurationWithSync is included in the Reconfiguration message) . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Currently the following steps (sending downlink IP data) could happen before the RACH proceudre is completed. Adding the checking of NR RACH indication after step 7 and 16,.so that the downlink data is not sent too early. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.7.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_2_2_7_1_ENDC_NR_TestBody() |
| --- | --- |
| Reason for change | For the reconfigurations at steps 7 and 16, The UE will perofrm a RACH procedure on the NR SCG cell (since ReconfigurationWithSync is included in the Reconfiguration message) . Currently the following steps (sending downlink IP data) could happen before the RACH proceudre is completed. |
| Summary of change | Adding the checking of NR RACH indication after step 7 and 16,.so that the downlink data is not sent too early. |
| TTCN module | RRCReconfig_ENDC_NR |
| MCC160 Comment | Accepted in principle but simply updated the parameters passed to the function f_NR_SendRRCReconfigurationContentsToEUTRA |

Before Change

function fl_TC_8_2_2_7_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; ………………….. // @siclog "Step 7" siclog@ // MCG to Split DRB f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {f_NR_ENDC_GetRadioBearerConfig_Reconfig (nr_Cell1, MCGToSplit)}); v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell1); // @sic R5-200579 sic@ v_RadioBearerConfig := cs_NR_SplitBearer(tsc_NR_DRB2, secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig); // @siclog "Step 9" siclog@ …………………. // @siclog "Step 16" siclog@ // MCG to SCG DRB f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {f_NR_ENDC_GetRadioBearerConfig_Reconfig (nr_Cell1, MCGToSCG)}); v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell1); // @sic R5-200579 sic@ v_RadioBearerConfig := cs_NR_RadioBearerRecoverPDCP(secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig); // @siclog "Step 18" siclog@ …………… }

After Change

function fl_TC_8_2_2_7_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; …………………. // @siclog "Step 7" siclog@ // MCG to Split DRB f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {f_NR_ENDC_GetRadioBearerConfig_Reconfig (nr_Cell1, MCGToSplit)}); v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell1); // @sic R5-200579 sic@ v_RadioBearerConfig := cs_NR_SplitBearer(tsc_NR_DRB2, secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); SYSIND.receive(car_NR_PRACH_Preamble_IND(?, cr_TimingInfo_Any, ?)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); // @siclog "Step 9" siclog@ ………………. // @siclog "Step 16" siclog@ // MCG to SCG DRB f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {f_NR_ENDC_GetRadioBearerConfig_Reconfig (nr_Cell1, MCGToSCG)}); v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell1); // @sic R5-200579 sic@ v_RadioBearerConfig := cs_NR_RadioBearerRecoverPDCP(secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); SYSIND.receive(car_NR_PRACH_Preamble_IND(?, cr_TimingInfo_Any, ?)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); // @siclog "Step 18" siclog@ …………… }

MCC160 Implementation:

// @siclog "Step 7" siclog@

// MCG to Split DRB

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {f_NR_ENDC_GetRadioBearerConfig_Reconfig (nr_Cell1, MCGToSplit)});

v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell1);  // @sic R5-200579 sic@

v_RadioBearerConfig := cs_NR_SplitBearer(tsc_NR_DRB2, secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get());

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig , -, -, -, reconfigurationWithSync_CheckRACH);

// @siclog "Step 9" siclog@

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_NR_CheckDataPath (nr_Cell1, PDN_2, dedicatedBearer, tsc_NR_DRB2, cs_RlcBearerRouting_NR(nr_Cell1), -,  "Step 9", 9);

// @siclog "Step 10" siclog@

// Split to SCG DRB

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {f_NR_ENDC_GetRadioBearerConfig_Reconfig (nr_Cell1, SplitToSCG)});

v_RadioBearerConfig := cs_NR_RadioBearer_Split_To_MCG(secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); // @sic R5-202646 sic@

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit);

// @siclog "Step 12" siclog@

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_NR_CheckDataPath (nr_Cell1, PDN_2, dedicatedBearer, tsc_NR_DRB2, cs_RlcBearerRouting_NR(nr_Cell1), -,  "Step 12", 12);

// @siclog "Step 13" siclog@

// SCG to MCG DRB

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {f_NR_ENDC_GetRadioBearerConfig_Reconfig (nr_Cell1, SCGToMCG, eutra_Cell1)});

v_RadioBearerConfig :=  cs_38508_RadioBearerConfigDef(-, {cs_NR_DRB_ToAddMod_RecoverPDCP(tsc_NR_DRB2, cs_38508_PDCP_Config)}, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary)); // @sic R5-202646 sic@

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); // @sic R5-200579 sic@

// @siclog "Step 15" siclog@

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_NR_CheckDataPath (nr_Cell1, PDN_2, dedicatedBearer, tsc_NR_DRB2, crs_RlcBearerRouting_Eutra(eutra_Cell1), -, "Step 15", 15);

// @siclog "Step 16" siclog@

// MCG to SCG DRB

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {f_NR_ENDC_GetRadioBearerConfig_Reconfig (nr_Cell1, MCGToSCG)});

v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell1);  // @sic R5-200579 sic@

v_RadioBearerConfig := cs_NR_RadioBearerRecoverPDCP(secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get());

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig) , -, -, -, reconfigurationWithSync_CheckRACH;

// @siclog "Step 18" siclog@