3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210885

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1883 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.4.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The test cases that require short DCI needs to be configured with DCI_0_0 format in the v_NR_RachProcedure for CRNTI Based contention resolution, current TTCN implementation is configuring with DCI_0_1 format - When testing 7.1.1.4.1.1 with SLIV 86 (S=2, L=7), the calculated TBS from TTCN is more than the actual TBS and therefore is resulting in RLC payload given to be sent being more than the TBS. According to 38.211 table 7.4.1.1.2-3 the DMRS positions are l0 7 and 11, therefore with SLIV 86 symbols l0 and 7 will be the DMRS additional position (v_ActualNumberOfSymbolsWithDMRS := 2) - The time domain resource allocation of using SLIV 53 is used in 38.523-3 as part of the default configuration for DL transmission for the preamble procedure and grant allocation. It is therefore proposed to use this for the preamble and during test procedure to test both sliv values (Proposed changes are covered as part of prose CR R5-215134) - The timing (slot) the DL data needs to be sent is not configured by the TTCN when Explicit resource allocation is configured, this could help SS determine which slot to be used to send the DL data, rather than allowing all possible slots to be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified the function to check for short DCI test cases and use new template cs_NR_RachProcedureConfig_CBRA_CrntiBased_DCI_x_0 - Changed the first if conditions to <7 - Due to change 1.4 swapped the order of the symbols - During DL grant configuration introduced cs_TimingInfo_SubFrame(v_SF), to specify the subframe and slot to be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.1.1.NR5GC, 7.1.1.4.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Proposed changes are covered as part of prose CR R5-215134 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required for test case 7.1.1.4.1.1	3

1.1	Correction to f_NR_InitialiseRachProcedure	3

1.2	cs_NR_RachProcedureConfig_CBRA_CrntiBased_DCI_x_0 / cs_NR_ContentionResolutionCtrl_CrntiBased_DCI_x_0	5

1.3	fl_NR_TBS_Calculation_NoOfREsPerPrb_Step1	6

1.4	cs_NR_PDSCH_TimeDomainResourceAllocationList_71141x	8

1.5	f_TC_7_1_1_4_1_1_NR_TestBody	9

1.6	f_NR_DLGrantConfiguration	14

# Corrections required for test case 7.1.1.4.1.1

## Correction to f_NR_InitialiseRachProcedure

| Function name | f_NR_InitialiseRachProcedure |
| --- | --- |
| Reason for change | The test cases that require short DCI needs to be configured with DCI_0_0 format in the v_NR_RachProcedure for CRNTI Based contention resolution, current TTCN implementation is configuring with DCI_0_1 format |
| Summary of change | Modified the function to check for short DCI test cases and use new template cs_NR_RachProcedureConfig_CBRA_CrntiBased_DCI_x_0 |
| TTCN module |  |
| MCC160 Comment | Accepted - implemented differently: adding NR_DciFormat_0_X_SpecificInfo_Type as parameter to cs_NR_RachProcedureConfig_CBRA_CrntiBased |

Before Change:

function f_NR_InitialiseRachProcedure(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return template(value) NR_RachProcedureConfig_Type { var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); var template(value) DL_CCCH_Message v_RrcSetup; var template(value) NR_RachProcedureMsg4RrcMsg_Type v_RachProcedureMsg4RrcMsg; var octetstring v_EncodedRrcSetup; if (f_GetTestcaseAttrib_ENDC(testcasename())) { //ENDC test cases -> C-RNTI based CBRA procedure // As per 38.523-3 Table 7.1.2.3.3-1: Default grants @sic R5-198994 sic@ v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, -,-, tsc_NR_ENDC_Msg3Grant_Def); /* @sic R5-187672, R5-187748: CBRA instead of CFRA sic@ */ } else { //NRSA test cases -> Contention Based with RRCSetupRequest in Msg3 if (not f_NR_EarlyContentionResolution()) { v_RrcSetup := f_NR_38508_RRCSetup(p_NR_CellId); v_EncodedRrcSetup := f_NR_RRC_DL_CCCH_Encvalue(v_RrcSetup); v_RachProcedureMsg4RrcMsg := cs_NR_RachProcedureMsg4WithCcchMsg(v_EncodedRrcSetup); } else { v_RachProcedureMsg4RrcMsg := cs_NR_RachProcedureMsg4WithNoRrcMsg; } v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, v_RachProcedureMsg4RrcMsg); } return v_NR_RachProcedure; }

After Change:

function f_NR_InitialiseRachProcedure(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return template(value) NR_RachProcedureConfig_Type { var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); var template(value) DL_CCCH_Message v_RrcSetup; var template(value) NR_RachProcedureMsg4RrcMsg_Type v_RachProcedureMsg4RrcMsg; var octetstring v_EncodedRrcSetup; if (f_GetTestcaseAttrib_ENDC(testcasename())) { //ENDC test cases -> C-RNTI based CBRA procedure // As per 38.523-3 Table 7.1.2.3.3-1: Default grants @sic R5-198994 sic@ if( not (f_GetTestcaseAttrib_ShortDCI(testcasename()))) { v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, -,-, tsc_NR_ENDC_Msg3Grant_Def); /* @sic R5-187672, R5-187748: CBRA instead of CFRA sic@ */ } else { v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased_DCI_x_0(v_NR_PhysicalParameters,-,-,tsc_NR_ENDC_Msg3Grant_Def); } } else { //NRSA test cases -> Contention Based with RRCSetupRequest in Msg3 if (not f_NR_EarlyContentionResolution()) { v_RrcSetup := f_NR_38508_RRCSetup(p_NR_CellId); v_EncodedRrcSetup := f_NR_RRC_DL_CCCH_Encvalue(v_RrcSetup); v_RachProcedureMsg4RrcMsg := cs_NR_RachProcedureMsg4WithCcchMsg(v_EncodedRrcSetup); } else { v_RachProcedureMsg4RrcMsg := cs_NR_RachProcedureMsg4WithNoRrcMsg; } v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, v_RachProcedureMsg4RrcMsg); } return v_NR_RachProcedure; }

## cs_NR_RachProcedureConfig_CBRA_CrntiBased_DCI_x_0 / cs_NR_ContentionResolutionCtrl_CrntiBased_DCI_x_0

| Function name | cs_NR_RachProcedureConfig_CBRA_CrntiBased_DCI_x_0/ cs_NR_ContentionResolutionCtrl_CrntiBased_DCI_x_0 |
| --- | --- |
| Reason for change | The test cases that require short DCI needs to be configured with DCI_0_0 format in the v_NR_RachProcedure for CRNTI Based contention resolution, current TTCN implementation is configuring with DCI_0_1 format |
| Summary of change | Defined new templates to use x_0 formats |
| TTCN module | NR_CellCfg_Templates |
| MCC160 Comment | Accepted, see change change 1 |

New Template:

template (value) NR_RachProcedureConfig_Type cs_NR_RachProcedureConfig_CBRA_CrntiBased_DCI_x_0(NR_PhysicalParameters_Type p_PhysicalParameters, template (value) NR_ResourceAllocation_Type p_NR_ResourceAllocation := cs_NR_ResourceAllocationMin, RNTI_Value_Type p_TempC_RNTI_Value := tsc_C_RNTI_Value2, integer p_Msg3_TBS := 256 ) := { RachProcedureList := { cs_NR_RachProcedure(p_PhysicalParameters, cs_NR_ContentionResolutionCtrl_CrntiBased_DCI_x_0(p_PhysicalParameters, p_NR_ResourceAllocation), cs_NR_TempC_RNTI_Explicit(p_TempC_RNTI_Value), p_Msg3_TBS ) } }; template (value) NR_ContentionResolutionCtrl_Type cs_NR_ContentionResolutionCtrl_CrntiBased_DCI_x_0(NR_PhysicalParameters_Type p_PhysicalParameters, template (value) NR_ResourceAllocation_Type p_ResourceAllocation, integer p_FirstRbIndex := 0) := { /* @status APPROVED (ENDC, NR5GC) */ CRNTI_Based := cs_NR_SearchSpaceUlDciAssignment(p_PhysicalParameters, cs_NR_DciFormat_0_0_Params, p_ResourceAllocation.Imcs, cs_NR_FreqDomainSchedulExplicit(p_FirstRbIndex, p_ResourceAllocation.Lrbs)) };

## fl_NR_TBS_Calculation_NoOfREsPerPrb_Step1

| Function name | fl_NR_TBS_Calculation_NoOfREsPerPrb_Step1 |
| --- | --- |
| Reason for change | When testing 7.1.1.4.1.1 with SLIV 86 (S=2, L=7), the calculated TBS from TTCN is more than the actual TBS and therefore is resulting in RLC payload given to be sent being more than the TBS. According to 38.211 table 7.4.1.1.2-3 the DMRS positions are l0 7 and 11, therefore with SLIV 86 symbols l0 and 7 will be the DMRS additional position (v_ActualNumberOfSymbolsWithDMRS := 2) |
| Summary of change | Changed the first if conditions to <7 |
| TTCN module | NR_CellCfg_Templates |
| MCC160 Comment | Rejected, the select/case statement is resulting in the correct value of v_ActualNumberOfSymbolsWithDMRS as specified in TS 38.211 Table 6.4.1.1.3-3 and Table 7.4.1.1.2-3 |

Before Change:

function fl_NR_TBS_Calculation_NoOfREsPerPrb_Step1(NR_TBS_CalculationParameters_Type p_Parameters, integer p_NoOfSymbols) return integer { var integer v_ActualNumberOfSymbolsWithDMRS; var integer v_TotalNumberOfREs; var integer v_NumberOfREsWithoutData; select (p_Parameters.MappingType) { case ("TypeA") { // Actual number of symbols with DMRS for mapping type A according to TS 38.211 Table 6.4.1.1.3-3 and Table 7.4.1.1.2-3 if ((p_NoOfSymbols < 8) or (p_Parameters.MaxNoOfDmrsAddPos == 0)) { v_ActualNumberOfSymbolsWithDMRS := 1; } else if ((p_NoOfSymbols < 10) or (p_Parameters.MaxNoOfDmrsAddPos == 1)) { v_ActualNumberOfSymbolsWithDMRS := 2; } else if ((p_NoOfSymbols < 12) or (p_Parameters.MaxNoOfDmrsAddPos == 2)) { v_ActualNumberOfSymbolsWithDMRS := 3; } else { v_ActualNumberOfSymbolsWithDMRS := 4; } }

After Change:

function fl_NR_TBS_Calculation_NoOfREsPerPrb_Step1(NR_TBS_CalculationParameters_Type p_Parameters, integer p_NoOfSymbols) return integer { var integer v_ActualNumberOfSymbolsWithDMRS; var integer v_TotalNumberOfREs; var integer v_NumberOfREsWithoutData; select (p_Parameters.MappingType) { case ("TypeA") { // Actual number of symbols with DMRS for mapping type A according to TS 38.211 Table 6.4.1.1.3-3 and Table 7.4.1.1.2-3 if ((p_NoOfSymbols < 7) or (p_Parameters.MaxNoOfDmrsAddPos == 0)) { v_ActualNumberOfSymbolsWithDMRS := 1; } else if ((p_NoOfSymbols < 10) or (p_Parameters.MaxNoOfDmrsAddPos == 1)) { v_ActualNumberOfSymbolsWithDMRS := 2; } else if ((p_NoOfSymbols < 12) or (p_Parameters.MaxNoOfDmrsAddPos == 2)) { v_ActualNumberOfSymbolsWithDMRS := 3; } else { v_ActualNumberOfSymbolsWithDMRS := 4; } }

## cs_NR_PDSCH_TimeDomainResourceAllocationList_71141x

| Function name | cs_NR_PDSCH_TimeDomainResourceAllocationList_71141x |
| --- | --- |
| Reason for change | The time domain resource allocation of using SLIV 53 is used in 38.523-3 as part of the default configuration for DL transmission for the preamble procedure and grant allocation. It is therefore proposed to use this for the preamble and during test procedure to test both sliv values |
| Summary of change | Swapped the order as the first time domain resource is allocated to be used in preamble Proposed changes require a prose change |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted as improvement |

Before Change:

template (value) PDSCH_TimeDomainResourceAllocationList cs_NR_PDSCH_TimeDomainResourceAllocationList_71141x := { /* @status APPROVED (ENDC, NR5GC) */ // Acc to 38.523-1 Table 7.1.1.4.1.0-1: PDSCH-TimeDomainResourceAllocationList //@sic R5-190677 sic@ cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 86), // S=2, L=7 cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53) // S=2, L=12 };

After Change:

template (value) PDSCH_TimeDomainResourceAllocationList cs_NR_PDSCH_TimeDomainResourceAllocationList_71141x := { /* @status APPROVED (ENDC, NR5GC) */ // Acc to 38.523-1 Table 7.1.1.4.1.0-1: PDSCH-TimeDomainResourceAllocationList //@sic R5-190677 sic@ // changing the order to use Sliv 53 so that preamble can follow automatic configuration as per 38523-3 annex cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53), // S=2, L=12 cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 86) // S=2, L=7 };

## f_TC_7_1_1_4_1_1_NR_TestBody

| Function name | cs_NR_PDSCH_TimeDomainResourceAllocationList_71141x |
| --- | --- |
| Reason for change | The time domain resource allocation of using SLIV 53 is used in 38.523-3 as part of the default configuration for DL transmission for the preamble procedure and grant allocation. It is therefore proposed to use this for the preamble and during test procedure to test both sliv values The timing (slot) the DL data needs to be sent is not configured by the TTCN when Explicit resource allocation is configured, this could help SS determine which slot to be used to send the DL data, rather than allowing all possible slots to be used. |
| Summary of change | Due to change 1.4 swapped the order of the symbols During DL grant configuration introduced cs_TimingInfo_SubFrame(v_SF), to specify the subframe and slot to be used. Note : SLIV swap change will require a change in prose a |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted as improvement |

Before Change:

function f_TC_7_1_1_4_1_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.x: DL-SCH Transport Block Size selection */ var integer v_TBSize:=0; // transport block size in bits var integer v_I_MCS; // Modulation and Coding Scehme Index var integer v_N_PRB; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA; // Time Domain resource allocation var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var NR_ResourceAllocation_Type v_NR_ResourceAllocation := {v_Max_nPRB, 28}; // Max NPRB and Max MCS //@sic R5s190826 sic@ var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_NoOfSymbols; // Configure SS for UL grant for Max TB size f_NR_ULGrantConfiguration_Start( nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation.Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs)); //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..8 //f_NR_ConfigureULGrant_MaxTBSupported(); for (v_N_PRB := 1; v_N_PRB <= v_Max_nPRB; v_N_PRB := v_N_PRB + 1) { //for nPRB 1 to v_Max_nPRB for (v_I_MCS := 0; v_I_MCS <= 28; v_I_MCS := v_I_MCS + 1) { //for iMCS 0 to 28 for (v_TDRA := 0; v_TDRA <= 1 ; v_TDRA := v_TDRA + 1) { //for Time Domain Resource Allocation 0 to 1 if (v_TDRA == 0) { v_NoOfSymbols := 7; } else { v_NoOfSymbols := 12; } //SS looks up iTBS in table 7.1.7.1-1 in TS 36.213 based on the value of iMCS //v_I_TBS := f_NR_Get_iTBS_For_iMCS(v_I_MCS); //The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L, and nPRB. //@sic R5s190826 sic@ v_TBSize := f_NR_TBS_Calculation(p_CalculationParameters, v_NoOfSymbols, v_N_PRB, v_I_MCS,true ); if (v_TBSize < 136 ) { //this results in zero number of PDCP SDUs or TBSize has high coding rate¦ continue; } // skip this nprb, Imcs, TDRA combination. // Steps 2 to 5 are performed if TBsize is More than minimum //@siclog "Step 2" siclog@ // SS creates one or more PDCP SDUs, depending on TBsize, in accordance with Table 7.1.1.4.1.1.3.2-2 //Configure SS for DCI Format x with RA type y and a Resource block assignment (RBA) correspondent //to nPRB as specified in 7.1.6.1 in TS 36.213 and Modulation and coding scheme iMCS f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_Now, resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_N_PRB), int2bit(v_TDRA, 4), cs_NR_TransportBlockRetransmissionList_Def(v_I_MCS), -, //@sic R5s190892 sic@ -, cs_NR_DciFormat_1_0_Params(v_SCS)); //@siclog "Step 3" siclog@ // SS sends MAC PDU (NxPDCP SDUs) v_NR_PDCP_SDUList := fl_NR_GetPdcpSduList_1TB(v_TBSize); //calculates v_N, v_PDCPsize and returns f_NR_Build_N_PDCP_SDUList DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //crs_RlcBearerRouting_NR(nr_Cell1), -, v_NR_PDCP_SDUList )); //@siclog "Step 4" siclog@ // automatically taken by SS //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs(v_NR_PDCP_SDUList, p_NR_DRB_Id); }//End for TDRA 0 to 8 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 }//f_NR_7_1_1_4_1_1

After Change:

function f_TC_7_1_1_4_1_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.x: DL-SCH Transport Block Size selection */ var integer v_TBSize:=0; // transport block size in bits var integer v_I_MCS; // Modulation and Coding Scehme Index var integer v_N_PRB; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA; // Time Domain resource allocation var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var NR_ResourceAllocation_Type v_NR_ResourceAllocation := {v_Max_nPRB, 28}; // Max NPRB and Max MCS //@sic R5s190826 sic@ var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_NoOfSymbols; var SubFrameNumber_Type v_SF := 2; if (v_SCS ==kHz30) {v_SF:= 1;} // Configure SS for UL grant for Max TB size f_NR_ULGrantConfiguration_Start( nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation.Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs)); //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..8 //f_NR_ConfigureULGrant_MaxTBSupported(); for (v_N_PRB := 1; v_N_PRB <= v_Max_nPRB; v_N_PRB := v_N_PRB + 1) { //for nPRB 1 to v_Max_nPRB for (v_I_MCS := 0; v_I_MCS <= 28; v_I_MCS := v_I_MCS + 1) { //for iMCS 0 to 28 for (v_TDRA := 0; v_TDRA <= 1 ; v_TDRA := v_TDRA + 1) { //for Time Domain Resource Allocation 0 to 1 if (v_TDRA == 0) { v_NoOfSymbols := 12; } else { v_NoOfSymbols := 7; } //SS looks up iTBS in table 7.1.7.1-1 in TS 36.213 based on the value of iMCS //v_I_TBS := f_NR_Get_iTBS_For_iMCS(v_I_MCS); //The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L, and nPRB. //@sic R5s190826 sic@ v_TBSize := f_NR_TBS_Calculation(p_CalculationParameters, v_NoOfSymbols, v_N_PRB, v_I_MCS,true ); if (v_TBSize < 136 ) { //this results in zero number of PDCP SDUs or TBSize has high coding rate¦ continue; } // skip this nprb, Imcs, TDRA combination. // Steps 2 to 5 are performed if TBsize is More than minimum //@siclog "Step 2" siclog@ // SS creates one or more PDCP SDUs, depending on TBsize, in accordance with Table 7.1.1.4.1.1.3.2-2 //Configure SS for DCI Format x with RA type y and a Resource block assignment (RBA) correspondent //to nPRB as specified in 7.1.6.1 in TS 36.213 and Modulation and coding scheme iMCS f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF), resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_N_PRB), int2bit(v_TDRA, 4), cs_NR_TransportBlockRetransmissionList_Def(v_I_MCS), -, //@sic R5s190892 sic@ -, cs_NR_DciFormat_1_0_Params(v_SCS)); //@siclog "Step 3" siclog@ // SS sends MAC PDU (NxPDCP SDUs) v_NR_PDCP_SDUList := fl_NR_GetPdcpSduList_1TB(v_TBSize); //calculates v_N, v_PDCPsize and returns f_NR_Build_N_PDCP_SDUList DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //crs_RlcBearerRouting_NR(nr_Cell1), -, v_NR_PDCP_SDUList )); //@siclog "Step 4" siclog@ // automatically taken by SS //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs(v_NR_PDCP_SDUList, p_NR_DRB_Id); }//End for TDRA 0 to 8 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 }//f_NR_7_1_1_4_1_1

## f_NR_DLGrantConfiguration

| Function name | f_NR_DLGrantConfiguration |
| --- | --- |
| Reason for change | Based on change 1.6, when cs_TimingInfo_SubFrame(v_SF) is passed as parameter, the CNF flag needs to set to true. |
| Summary of change | Passed the parameter as true when configuring DCI f_NR_SS_CommonCellConfig(p_NR_CellId, cas_NR_DciDlInfo_REQ…. |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted as improvement |

Before Change:

function f_NR_DLGrantConfiguration(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, NR_ResourceAllocationType_Type p_ResourceAllocationType, template (value) NR_FreqDomainResourceAssignmentDL_Type p_FreqDomainResourceAssignmentDL, bitstring p_Index := '0000'B, template (value) NR_TransportBlockRetransmissionList_Type p_TransportBlockRetransmissionList, template (omit) NR_TransportBlockRetransmissionList_Type p_TransportBlockRetransmissionList2 := omit, template (value) NR_HarqProcessConfig_Type p_HarqProcessConfig := cs_NR_HarqProcessConfig_AnyProcess, template (value) NR_DciFormat_1_X_SpecificInfo_Type p_DciFormatAndParams) runs on NR_BASE_PTC { var template (value) NR_DciDlInfo_Type v_DciDlInfo := cs_NR_DciDlInfo_Init(p_ResourceAllocationType, p_FreqDomainResourceAssignmentDL, cs_NR_DciCommon_TimeDomainResourceAssignment(p_Index), cs_NR_TransportBlockSchedulingDL_Explicit(p_TransportBlockRetransmissionList, p_TransportBlockRetransmissionList2), p_HarqProcessConfig, p_DciFormatAndParams); f_NR_SS_CommonCellConfig(p_NR_CellId, cas_NR_DciDlInfo_REQ(p_NR_CellId, p_TimingInfo, v_DciDlInfo)); }

After Change:

function f_NR_DLGrantConfiguration(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, NR_ResourceAllocationType_Type p_ResourceAllocationType, template (value) NR_FreqDomainResourceAssignmentDL_Type p_FreqDomainResourceAssignmentDL, bitstring p_Index := '0000'B, template (value) NR_TransportBlockRetransmissionList_Type p_TransportBlockRetransmissionList, template (omit) NR_TransportBlockRetransmissionList_Type p_TransportBlockRetransmissionList2 := omit, template (value) NR_HarqProcessConfig_Type p_HarqProcessConfig := cs_NR_HarqProcessConfig_AnyProcess, template (value) NR_DciFormat_1_X_SpecificInfo_Type p_DciFormatAndParams) runs on NR_BASE_PTC { var template (value) NR_DciDlInfo_Type v_DciDlInfo := cs_NR_DciDlInfo_Init(p_ResourceAllocationType, p_FreqDomainResourceAssignmentDL, cs_NR_DciCommon_TimeDomainResourceAssignment(p_Index), cs_NR_TransportBlockSchedulingDL_Explicit(p_TransportBlockRetransmissionList, p_TransportBlockRetransmissionList2), p_HarqProcessConfig, p_DciFormatAndParams); f_NR_SS_CommonCellConfig(p_NR_CellId, cas_NR_DciDlInfo_REQ(p_NR_CellId, p_TimingInfo, v_DciDlInfo, true)); }