3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211169

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2001 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the reestablishment procedure related functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation, 1) In the function : f_NR5GC_RRCReestablishment_Steps1To2 , f_NR_CellInfo_SetRNTI function is updating the RNTI value of the target cell DB . Updating the Target Cell DB with the new RNTI before calculating the MAC-I is creating mismatch between UE and the Network ( especially in the scenario where same source and target cells are used ) 2) In the function : f_NR5GC_RRCReestablishment_Steps3To7 is configuring UL Grant configuration based on SR at steps 3 and 4 , at this point of time UL DCI is in CSS and UE will not send SR to send reestablishment complete message , so it is proposed to shift the UL grant configuration based on SR to the function f_NR5GC_RRCReestablishment_Steps8To13 after UL DCI is switched to USS which will be eventually used for the reconfiguration complete reception 3) In the functions : f_NR5GC_RRCReestablishment_Steps3To7 and in f_NR5GC_RRCReestablishment_Steps8To13, when releasing SRB and DRB (f_NR_SS_SRBs_DRBs_Release ) the follow on flag is set to true. According to the agreed rules (in TTCNsidebar RAN5#89e) it was not intended to use for releasing and seting of SRBs/DRBs (two RadioBearerList (NR_RadioBearerList_Type)). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - f_NR_CellInfo_SetRNTI is moved after the reception of reestablishment request , so that short MAC-I calculated at TTCN matches with UE calculated value - UL grant configuration based on SR , are removed from Step3 and as step 4 from f_NR5GC_RRCReestablishment_Steps3To7 , moved to f_NR5GC_RRCReestablishment_Steps8To13 , after reconfiguration UL DCCH/DTCH DCI on USS is done. - f_NR_SS_SRBs_DRBs_Release is added with tsc_CnfReq and tsc_FollowOnFlag is removed in the functions f_NR5GC_RRCReestablishment_Steps3To7 and f_NR5GC_RRCReestablishment_Steps8To13 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1,2) A Conformant UE may fail the test case 3) FollowOnFlag usage is not in align with what is decided in RAN5#89e |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.4.NR5GC, 8.1.4.1.5.NR5GC and other reestablishment scenarios |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | For Changes 2,3 , Prose CR will be required |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to function f_NR5GC_RRCReestablishment_Steps1To2	3

1.2	Correction to function f_NR5GC_RRCReestablishment_Steps3To7	5

1.3	Correction to function f_NR5GC_RRCReestablishment_Steps8To13	8

# Corrections required

## Correction to function f_NR5GC_RRCReestablishment_Steps1To2

| Function name | f_NR5GC_RRCReestablishment_Steps1To2 |
| --- | --- |
| Reason for change | In the current TTCN implementation, In the function : f_NR5GC_RRCReestablishment_Steps1To2 , f_NR_CellInfo_SetRNTI function is updating the RNTI value of the target cell DB . Updating the Target Cell DB with the new RNTI before calculating the MAC-I is creating mismatch between UE and the Network ( especially in the scenario where same source and target cells are used ) |
| Summary of change | f_NR_CellInfo_SetRNTI is moved after the reception of reestablishment request , so that short MAC-I calculated at TTCN matches with UE calculated value |
| TTCN module | ttcn\develop\Common\NR5GC\ NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted – see proposed implementation below |

Before Change:

function f_NR5GC_RRCReestablishment_Steps1To2(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template(omit) charstring p_VerdictMsg := omit, template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit ) runs on NR_BASE_PTC { //@sic R5-211391 R5-213516: added p_MacBearerRoutingCA sic@ var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId); //C-RNTI used by UE in RRCReestablismentRequest var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_NR_SourceCellId); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId); var ShortMAC_I v_ShortMAC_I; // 1. Target Cell: Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI. f_NR_CellInfo_SetRNTI(p_NR_TargetCellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_NR_TargetCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); // NOTE 1: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, tsc_CnfReq, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg), cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); // 2. Target Cell: Receive RRCReestablismentRequest. v_ShortMAC_I := f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId); SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), p_ReestablishmentCause), p_MacBearerRoutingCA)); if (isvalue(p_VerdictMsg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg)); } }

After Change:

function f_NR5GC_RRCReestablishment_Steps1To2(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template(omit) charstring p_VerdictMsg := omit, template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit ) runs on NR_BASE_PTC { //@sic R5-211391 R5-213516: added p_MacBearerRoutingCA sic@ var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId); //C-RNTI used by UE in RRCReestablismentRequest var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_NR_SourceCellId); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId); var ShortMAC_I v_ShortMAC_I; // 1. Target Cell: Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI. //f_NR_CellInfo_SetRNTI(p_NR_TargetCellId, p_RNTI_Value); //Removed as updating Cell DB here will input updated RNTI for the calculation of Short MAC-I ( especially where source and target cells are same ) f_NR_SS_C_RNTI_Config(p_NR_TargetCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); // NOTE 1: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, tsc_CnfReq, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg), cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); // 2. Target Cell: Receive RRCReestablismentRequest. v_ShortMAC_I := f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId); SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), p_ReestablishmentCause), p_MacBearerRoutingCA)); if (isvalue(p_VerdictMsg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg)); } /*Preferably to configure RNTI on the target cell after the reception of Reest req , as Short MAC-I calculation makes use of RNTI on the source cell which is the value before the updatation */ f_NR_CellInfo_SetRNTI(p_NR_TargetCellId, p_RNTI_Value); }

MCC160 proposed implementation

function f_NR5GC_RRCReestablishment_Steps1To2(NR_CellId_Type            p_NR_SourceCellId,

NR_CellId_Type            p_NR_TargetCellId,

template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure,

RNTI_Value                p_RNTI_Value :=  tsc_C_RNTI_Value4,

template(omit) charstring p_VerdictMsg := omit,

template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit

) runs on NR_BASE_PTC

{ //@sic R5-211391 R5-213516: added p_MacBearerRoutingCA sic@

var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId);  //C-RNTI used by UE in RRCReestablismentRequest

var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_NR_SourceCellId);

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId);

var ShortMAC_I v_ShortMAC_I:= f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId);  //@sic R5s211169 change 1.1 sic@

// 1. Target Cell: Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI.

f_NR_CellInfo_SetRNTI(p_NR_TargetCellId, p_RNTI_Value);

f_NR_SS_C_RNTI_Config(p_NR_TargetCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag);    // NOTE 1: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI.

f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, tsc_CnfReq,

cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg),

cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters),

cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params)));

// 2. Target Cell: Receive RRCReestablismentRequest.

SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), p_ReestablishmentCause), p_MacBearerRoutingCA));

if (isvalue(p_VerdictMsg)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg));

}

}

## Correction to function f_NR5GC_RRCReestablishment_Steps3To7

| Function name | f_NR5GC_RRCReestablishment_Steps3To7 |
| --- | --- |
| Reason for change | In the current TTCN implementation, - In the function : f_NR5GC_RRCReestablishment_Steps3To7 is configuring UL Grant configuration based on SR at steps 3 and 4 , at this point of time UL DCI is in CSS and UE will not send SR to send reestablishment complete message , so it is proposed to shift the UL grant configuration based on SR to the function f_NR5GC_RRCReestablishment_Steps8To13 after UL DCI is switched to USS which will be eventually used for the reconfiguration complete reception - In the functions : f_NR5GC_RRCReestablishment_Steps3To7 and in f_NR5GC_RRCReestablishment_Steps8To13, when releasing SRB and DRB (f_NR_SS_SRBs_DRBs_Release ) the follow on flag is set to true. According to the agreed rules (in TTCNsidebar RAN5#89e) it was not intended to use for releasing and seting of SRBs/DRBs (two RadioBearerList (NR_RadioBearerList_Type). |
| Summary of change | - UL grant configuration based on SR , are removed from Step3 and as step 4 from f_NR5GC_RRCReestablishment_Steps3To7 , moved to f_NR5GC_RRCReestablishment_Steps8To13 , after reconfiguration UL DCCH/DTCH DCI on USS. - f_NR_SS_SRBs_DRBs_Release is added with tsc_CnfReq and tsc_FollowOnFlag is removed in the functions f_NR5GC_RRCReestablishment_Steps3To7 |
| TTCN module | ttcn\develop\Common\NR5GC\ NR5GC_RRCSteps.ttcn |
| MCC160 Comment | 1. Postponed, conditional to prose CR agreement on 38.523-3 clause 7.3.5.4 2. Accepted |

Before Change:

function f_NR5GC_RRCReestablishment_Steps3To7(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, NextHopChainingCount p_NextHopChainingCount := 0, template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := omit ) runs on NR_BASE_PTC { //@sic R5-206369 R5-213516: added p_MacBearerRoutingCA sic@ var IntegerList_Type v_DrbIdList;//WA#WI=866592 // 3. Source Cell: If the Source Cell is the same as the Target Cell: // Reset SRBs and release DRBs // Stop periodic TA if (p_NR_SourceCellId == p_NR_TargetCellId) { v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId); f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-211417 sic@ f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit, true); f_NR_ULGrantConfiguration_Common(p_NR_SourceCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Stop, cs_UL_GrantConfig_OnSR); } // 4.Target Cell: If the Source Cell is different from the Target Cell: Configure UL grant configuration ("OnSR"). if (p_NR_SourceCellId != p_NR_TargetCellId) { f_NR_ULGrantConfiguration_Start(p_NR_TargetCellId, -, -, -, cs_NR_UplinkTimeAlignment_Stop); } // 5. Target Cell: Activate Security f_NR_SS_AS_ActivateSecurity_Current(p_NR_TargetCellId, p_NextHopChainingCount, -, true); // 6. Target : Send RRCReestablishment SRB.send (cas_NR_SRB1_RrcPdu_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def, p_NextHopChainingCount), p_MacBearerRoutingCA)); // 7. Target Cell: Receive RRCReestablishmentComplete SRB.receive ( car_NR_SRB1_RrcPdu_IND ( p_NR_TargetCellId, cr_38508_RRCReestablishmentComplete(-, p_RRCReestablishmentComplete_v1610_IEs), -, p_MacBearerRoutingCA) ); }

After Change:

function f_NR5GC_RRCReestablishment_Steps3To7(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, NextHopChainingCount p_NextHopChainingCount := 0, template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := omit ) runs on NR_BASE_PTC { //@sic R5-206369 R5-213516: added p_MacBearerRoutingCA sic@ var IntegerList_Type v_DrbIdList;//WA#WI=866592 // 3. Source Cell: If the Source Cell is the same as the Target Cell: // Reset SRBs and release DRBs // Stop periodic TA if (p_NR_SourceCellId == p_NR_TargetCellId) { v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId); f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList, tsc_CnfReq/*, tsc_FollowOnFlag*/); //@sic R5-211417 sic@ //WA#WI=875685 f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit, true); //Removing the SR Configuration here as UL DCI is still with CSS and SR is not going to be used for reestablishment complete reception //f_NR_ULGrantConfiguration_Common(p_NR_SourceCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Stop, cs_UL_GrantConfig_OnSR); //WA#WI=875685 } // 4.Target Cell: If the Source Cell is different from the Target Cell: Configure UL grant configuration ("OnSR"). – Step 4 to be removed /*if (p_NR_SourceCellId != p_NR_TargetCellId) { //Removing the SR Configuration here as UL DCI is still with CSS and SR is not going to be used for reestablishment complete reception //f_NR_ULGrantConfiguration_Start(p_NR_TargetCellId, -, -, -, cs_NR_UplinkTimeAlignment_Stop); //WA#WI=875685 }*/ // 4. Target Cell: Activate Security f_NR_SS_AS_ActivateSecurity_Current(p_NR_TargetCellId, p_NextHopChainingCount, -, true); // 5. Target : Send RRCReestablishment SRB.send (cas_NR_SRB1_RrcPdu_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def, p_NextHopChainingCount), p_MacBearerRoutingCA)); // 6. Target Cell: Receive RRCReestablishmentComplete SRB.receive ( car_NR_SRB1_RrcPdu_IND ( p_NR_TargetCellId, cr_38508_RRCReestablishmentComplete(-, p_RRCReestablishmentComplete_v1610_IEs), -, p_MacBearerRoutingCA) ); }

## Correction to function f_NR5GC_RRCReestablishment_Steps8To13

| Function name | f_NR5GC_RRCReestablishment_Steps8To13 |
| --- | --- |
| Reason for change | In the current TTCN implementation, - In the function : f_NR5GC_RRCReestablishment_Steps3To7 is configuring UL Grant configuration based on SR at steps 3 and 4 , at this point of time UL DCI is in CSS and UE will not send SR to send reestablishment complete message , so it is proposed to shift the UL grant configuration based on SR to the function f_NR5GC_RRCReestablishment_Steps8To13 after UL DCI is switched to USS which will be eventually used for the reconfiguration complete reception - In the functions : f_NR5GC_RRCReestablishment_Steps3To7 and in f_NR5GC_RRCReestablishment_Steps8To13, when releasing SRB and DRB (f_NR_SS_SRBs_DRBs_Release ) the follow on flag is set to true. According to the agreed rules (in TTCNsidebar RAN5#89e) it was not intended to use for releasing and seting of SRBs/DRBs (two RadioBearerList (NR_RadioBearerList_Type). |
| Summary of change | - UL grant configuration based on SR , are removed from Step3 and as step 4 from f_NR5GC_RRCReestablishment_Steps3To7 , moved to f_NR5GC_RRCReestablishment_Steps8To13 , after reconfiguration UL DCCH/DTCH DCI on USS. - f_NR_SS_SRBs_DRBs_Release is added with tsc_CnfReq and tsc_FollowOnFlag is removed in the function f_NR5GC_RRCReestablishment_Steps8To13 |
| TTCN module | ttcn\develop\Common\NR5GC\ NR5GC_RRCSteps.ttcn |
| MCC160 Comment | 1. Postponed, conditional to prose CR agreement on 38.523-3 clause 7.3.5.4 2. Accepted - implemented as per R5s211165 |

Before Change:

function f_NR5GC_RRCReestablishment_Steps8To13(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template (omit) NR_SCellToAddModList_Type p_SCellToAddModList := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, boolean p_ReceiveForRRCreconfigurationComplete := true ) runs on NR5GC_PTC { //@sic R5-206369 R5-213516: added p_MacBearerRoutingCA sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId); var IntegerList_Type v_DrbIdList; var template (value) RadioBearerConfig v_RadioBearerConfig := f_NR5GC_GetExisting_RadioBearerConfig(true); var template (value) CellGroupConfig v_CellGroupConfig := fl_NR_GetCellGroupConfigREEST(p_NR_TargetCellId, p_SCellToAddModList, p_RlfTimersAndConstants); var template (value) NR_RadioBearerList_Type v_SS_RadioBearerConfig := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList()); // Step 8. Target Cell: Reconfigure DRBs and reconfigure UL DCCH DCI on USS // Step 9. Target Cell: Send RRCReconfiguration // Step 10. Target Cell: Receive RRCReconfigurationComplete // Set RadioBearerConfig as per Table 4.6.3-132: RadioBearerConfig cond REEST v_RadioBearerConfig.srb_ToAddModList := {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}; //@sic R5-205334 sic@ f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_SearchSpaceUlDciAssignment_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, f_NR_SearchSpaceUlDciAssignment(v_NR_PhysicalParameters))); f_ReconfigureDRBs(p_NR_TargetCellId, omit, v_RadioBearerConfig, v_CellGroupConfig, v_SS_RadioBearerConfig, -, -, p_ReceiveForRRCreconfigurationComplete, -, -, -, p_MacBearerRoutingCA); //@sic R5-205336 sic@ // 11. Target Cell: Reconfigure DL DCCH/DTCH DCI on USS //NOTE 2: The FollowOnFlag is set to true in the ASP reconfiguring the target Cell. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cas_NR_SearchSpaceDlDciAssignment_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, f_NR_SearchSpaceDlDciAssignment_USS(v_NR_PhysicalParameters), tsc_NoCnfReq, tsc_FollowOnFlag)); // 12. Target Cell: Re-configure RACH procedure as for initial access, enable TA transmissions. //NOTE 3: The FollowOnFlag is set to true in the ASP reconfiguring the target PCell. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedureConfig_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg), tsc_NoCnfReq, tsc_FollowOnFlag)); f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cas_NR_UplinkTimeAlignment_REQ(p_NR_TargetCellId, -, cs_NR_UplinkTimeAlignment_Start)); // 13. Source Cell: If the Source Cell is different than the Target Cell: // Reset SRBs and release DRBs. // Stop periodic UL grant and periodic TA. if (p_NR_SourceCellId != p_NR_TargetCellId) { v_DrbIdList := f_NR_GetActiveDRBs(p_NR_TargetCellId); f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-211417 sic@ f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit, true); f_NR_ULGrantConfiguration_Stop(p_NR_SourceCellId, -, cs_NR_UplinkTimeAlignment_Stop); } }

After Change:

function f_NR5GC_RRCReestablishment_Steps8To13(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template (omit) NR_SCellToAddModList_Type p_SCellToAddModList := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, boolean p_ReceiveForRRCreconfigurationComplete := true ) runs on NR5GC_PTC { //@sic R5-206369 R5-213516: added p_MacBearerRoutingCA sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId); var IntegerList_Type v_DrbIdList; var template (value) RadioBearerConfig v_RadioBearerConfig := f_NR5GC_GetExisting_RadioBearerConfig(true); var template (value) CellGroupConfig v_CellGroupConfig := fl_NR_GetCellGroupConfigREEST(p_NR_TargetCellId, p_SCellToAddModList, p_RlfTimersAndConstants); var template (value) NR_RadioBearerList_Type v_SS_RadioBearerConfig := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList()); // Step 7. Target Cell: Reconfigure DRBs and reconfigure UL DCCH DCI on USS // Step 8. Target Cell: Send RRCReconfiguration // Step 9. Target Cell: Receive RRCReconfigurationComplete // Set RadioBearerConfig as per Table 4.6.3-132: RadioBearerConfig cond REEST v_RadioBearerConfig.srb_ToAddModList := {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}; //@sic R5-205334 sic@ f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_SearchSpaceUlDciAssignment_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, f_NR_SearchSpaceUlDciAssignment(v_NR_PhysicalParameters))); //Step 10 :moving the SR Configuration here as UL DCI is switched to USS and SR is going to be used for reconfiguration complete if (p_NR_SourceCellId != p_NR_TargetCellId) { //WA#WI=875685 f_NR_ULGrantConfiguration_Start(p_NR_TargetCellId, -, -, -, cs_NR_UplinkTimeAlignment_Stop); } else{ f_NR_ULGrantConfiguration_Common(p_NR_SourceCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Stop, cs_UL_GrantConfig_OnSR); } //WA#WI=875685 f_ReconfigureDRBs(p_NR_TargetCellId, omit, v_RadioBearerConfig, v_CellGroupConfig, v_SS_RadioBearerConfig, -, -, p_ReceiveForRRCreconfigurationComplete, -, -, -, p_MacBearerRoutingCA); //@sic R5-205336 sic@ // 11. Target Cell: Reconfigure DL DCCH/DTCH DCI on USS //NOTE 2: The FollowOnFlag is set to true in the ASP reconfiguring the target Cell. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cas_NR_SearchSpaceDlDciAssignment_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, f_NR_SearchSpaceDlDciAssignment_USS(v_NR_PhysicalParameters), tsc_NoCnfReq, tsc_FollowOnFlag)); // 12. Target Cell: Re-configure RACH procedure as for initial access, enable TA transmissions. //NOTE 3: The FollowOnFlag is set to true in the ASP reconfiguring the target PCell. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedureConfig_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg), tsc_NoCnfReq, tsc_FollowOnFlag)); f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cas_NR_UplinkTimeAlignment_REQ(p_NR_TargetCellId, -, cs_NR_UplinkTimeAlignment_Start)); // 13. Source Cell: If the Source Cell is different than the Target Cell: // Reset SRBs and release DRBs. // Stop periodic UL grant and periodic TA. if (p_NR_SourceCellId != p_NR_TargetCellId) { v_DrbIdList := f_NR_GetActiveDRBs(p_NR_TargetCellId); f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList, tsc_CnfReq/*, tsc_FollowOnFlag*/); //@sic R5-211417 sic@ //WA#WI=875685 f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit, true); f_NR_ULGrantConfiguration_Stop(p_NR_SourceCellId, -, cs_NR_UplinkTimeAlignment_Stop); } }