3GPP TSG-RAN5 Meeting #92-e	R5-XXXXXX

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR |  | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the reestablishment procedure related steps |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the section 7.3.5.4 , 1) UL Grant configuration based on SR is done at steps 3 and 4 ; at this point of time UL DCI is in CSS and UE will not send SR to send reestablishment complete message , so it is proposed to shift the UL grant configuration based on SR after step 7 where Reconfiguration of UL DCCH/DTCH DCI on USS is done 2) At Step 3 and 13 , Follow On flag is set to true in the ASP configuring the SRBs ; According to the agreed rules (in TTCNsidebar RAN5#89e) FollowOn Flag was not intended to use for releasing and seting of SRBs/DRBs (two RadioBearerList (NR_RadioBearerList_Type)). |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Ul grant configuration based on SR is moved after step 7 , where reconfiguration of UL DCCH/DTCH DCI on USS is completed 2) FollowonFlag is removed in step 3 and 13 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1) A Conformant UE may fail the test case 2) FollowOnFlag usage is not in align with what is decided in TTCNsidebar RAN5#89e |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.4.NR5GC, 8.1.4.1.5.NR5GC and other reestablishment scenarios |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Associated TTCN-CR will be raised |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 7	Test methods and design considerations

## 7.3	NR/5GC

### 7.3.5	RRC connection control

#### 7.3.5.4	RRC connection re-establishment

In general, the re-establishment is done without activation time, i.e. the timing information for configuration of the SS and sending of the RRCReestablishment is ‘Now'.

The Source Cell and Target cell can be the same cell.

1.	Target Cell:	Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI.

NOTE 1:	The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI.

2.	Target Cell:	Receive RRCReestablismentRequest.

3.	Source Cell:	If the Source Cell is the same as the Target Cell:

Reset SRBs and release DRBs.

4.	Target Cell:	Activate Security.

5.	Target Cell:	Send RRCReestablishment.

6.	Target Cell:	Receive RRCReestablishmentComplete.

7.	Target Cell:	Reconfigure DRBs and reconfigure UL DCCH/DTCH DCI on USS.

8.   Target Cell:         If the Source Cell is different/Same from the Target Cell:

Configure UL grant configuration (“On SR”)  and stop periodic TA

9.	Target Cell:	Send RRCReconfiguration.

10.	Target Cell:	Receive RRCReconfigurationComplete.

11. Target Cell: 	Reconfigure DL DCCH/DTCH DCI on USS.

NOTE 3:	The FollowOnFlag is set to true in the ASP reconfiguring the target PCell.

12. Target Cell:	Re-configure RACH procedure as for initial access, enable TA transmissions.

NOTE 4:	The FollowOnFlag is set to true in the ASP reconfiguring the target PCell.

13.	Source Cell:	If the Source Cell is different from the Target Cell:

Reset SRBs and release DRBs.

Stop periodic UL grant and periodic TA.