3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211170

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2002 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RACS NR5GC testcase 9.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add 9.1.9.1RACS / Network assigned UE radio capability ID |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 9.1.9.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	8

2.4	Change 4	10

2.5	Change 5	12

2.6	Change 6	13

3	Branches Executed	14

4	Execution Log Files	15

4.1      Samsung SM-G996B(S22_Palette)	15

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

  - Verification Test Summary

Test Case: 	9.1.9.1

ATS Version:	iwd-TTCN3-B2020-09_D21wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung SM-G996B(S22_Palette)

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | function f_TC_9_1_9_1_NR5GC() |
| --- | --- |
| Reason for change | - NW-assigned UE radio capability ID consists of TF + Version ID + RCI.So, if TE set Version ID as 00, UE radio capability ID in the digit expression is as follows: TF + Version ID + RCI = 1 00 00000000001 - As 5GMM capability is non-cleartext IE and hence not included in the initial NAS message without security It should be included in the Registration request piggyback with Security Mode complete message #draft proseCR would be raised for RAN5#93-e meeting |
| Summary of change | - TTCN uses the encoded value so used “01000000000010” which will be decode as 10000000000001 and leght changed to 7 - Removed GMM cap from IntialRegisration request (also, please refer change 2 and change 4) |
| TTCN module | RACS_NR5GC.ttcn |
| MCC160 Comment | 1. Accepted and implemented according to R5-217901. 2. Accepted in principle, but implementation shown below. |

Before Change

function f_TC_9_1_9_1_NR5GC() runs on NR5GC_PTC { //RACS / Network assigned UE radio capability ID var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) UERadioCapId v_UERadioCapId := { iei := '67'O, iel := '06'O, id := '100000000001'O }; f_NR5GC_Init_NAS(NR_2); … f_NR5GC_SwitchOnAndResetIMS(); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, -, -, -, -, cdr_NG_GMM_Cap_RACS)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } … f_NR_TestBody_Set(true); fl_TC_9_1_9_1_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellB, STATE_IDLE_1A); }

After Change

function f_TC_9_1_9_1_NR5GC() runs on NR5GC_PTC { //RACS / Network assigned UE radio capability ID var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) UERadioCapId v_UERadioCapId := { iei := '67'O, iel := '07'O, id := '01000000000010'O }; f_NR5GC_Init_NAS(NR_2); … v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity/*, -, -, -, -, cdr_NG_GMM_Cap_RACS*/ // Removed)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } … // Finish f_NR_Postamble(v_NGC_NASCellB, STATE_IDLE_1A); }

  - Change 2

| Function name | function fl_TC_9_1_9_1_Body() |
| --- | --- |
| Reason for change | - NW-assigned UE radio capability ID consists of TF + Version ID + RCI.So, if TE set Version ID as 00, UE radio capability ID in the digit expression is as follows: TF + Version ID + RCI = 1 00 00000000001 - As 5GMM capability is non-cleartext IE and hence not included in the initial NAS message without security It should be included in the Registration request piggyback with Security Mode complete message and please refer to the specification |
| Summary of change | - TTCN uses the encoded value so used “01000000000010” and 01000000000020 which will be decode as 10000000000001 and 10000000000002and length changed to 7 - Pass the True argumenent |
| TTCN module | RACS_NR5GC.ttcn |
| MCC160 Comment | 1. Accepted and implemented according to R5-217901. 2. Rejected. R5-217901 states that there is a registration in the preamble and the state is 0N-B, then the Registration Request at step 2 will have a security context. So the Registration Request will NOT be included in the Security Mode Complete at this point. |

Before Change

function fl_TC_9_1_9_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) UERadioCapId v_UERadioCapId1 := { iei := '67'O, iel := '06'O, id := '100000000001'O }; var template (value) UERadioCapId v_UERadioCapId2 := { iei := '67'O, iel := '06'O, id := '100000000002'O }; //@siclog "Step 1" siclog@ f_UT_SwitchOnUE(UT); … v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); //@siclog "Steps 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); … f_NR5GC_MobilityRegistration_Steps4_6 (v_NGC_NASCellB, v_GMM_MobilityInfo); } //End of fl_TC_9_1_9_1_Body

After Change

function fl_TC_9_1_9_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) UERadioCapId v_UERadioCapId1 := { iei := '67'O, iel := '07'O, id := '01000000000010'O }; var template (value) UERadioCapId v_UERadioCapId2 := { iei := '67'O, iel := '07'O, id := '01000000000020'O }; //@siclog "Step 1" siclog@ f_UT_SwitchOnUE(UT); … //@siclog "Steps 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity,-,true); … f_NR5GC_MobilityRegistration_Steps4_6 (v_NGC_NASCellB, v_GMM_MobilityInfo); } //End of fl_TC_9_1_9_1_Body

  - Change 3

| Function name | function f_NR5GC_RRC_Idle_Steps5_13 |
| --- | --- |
| Reason for change | Enabling the GMM capability in registration request piggybacked with Security mode complete. Please refer change 2 |
| Summary of change | Passing the Boolean Variable as argument |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Not needed. Alternative solution implemented as per R5s211357: parameter p_GMM_MobilityInfo_Type changed to inout |

Before Change

function f_NR5GC_RRC_Idle_Steps5_13 (GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NAS_PlmnId p_PLMN, NR_TESTMODE_STATE_Type p_TestMode, TypeOfRegistration_Type p_RegType := Initial_Secure, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC { var NasCount_Type v_NasCount; … // Steps 5 - 6 // Do Authentication v_NasCount := f_NR5GC_RRC_Idle_Steps5_9_AKA (p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, p_PLMN, p_RegType, p_IwkN26); // @sic R5-202564 sic@ … // Steps 12 - 13 // UE Capability f_NR_Capability(p_NR_CellId); }

After Change

function f_NR5GC_RRC_Idle_Steps5_13 (GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NAS_PlmnId p_PLMN, NR_TESTMODE_STATE_Type p_TestMode, TypeOfRegistration_Type p_RegType := Initial_Secure, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, boolean p_RacsEnabled := false ) runs on NR5GC_PTC { var NasCount_Type v_NasCount; … // Steps 5 - 6 // Do Authentication v_NasCount := f_NR5GC_RRC_Idle_Steps5_9_AKA (p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, p_PLMN, p_RegType, p_IwkN26,p_RacsEnabled ); // @sic R5-202564 sic@ … // Steps 12 - 13 // UE Capability f_NR_Capability(p_NR_CellId); }

  - Change 4

| Function name | f_NR5GC_RRC_Idle_Steps5_9_AKA |
| --- | --- |
| Reason for change | Enabling the GMM capability in registration request piggy back with Security mode complete refer to change 2 |
| Summary of change | Passing the Boolean Variable as argument and adding condition to check the GMM capability in Registration request after Security |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Not needed |

Before Change

function f_NR5GC_RRC_Idle_Steps5_9_AKA (inout GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NAS_PlmnId p_PLMN, TypeOfRegistration_Type p_RegType := Initial_Secure, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC return NasCount_Type { var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var NG_NAS_MSG_Indication_Type v_ReceivedMsg := p_ReceivedMsg; var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var SecurityHeaderType v_SentSecurityHeader; var template (present) SecurityHeaderType v_SecurityHeaderResponse; … // Step 8 - 9 v_SecurityParams := f_NR5GC_Security_Get(); // Do NAS Security Mode procedure v_NAS_SecurityReturnValue := f_NR_NASSecurityModeProcedure(p_NR_CellId, v_SecurityParams, p_RegType, p_GMM_MobilityInfo_Type); if (p_RegType == Initial_NoSecurity) { if (isvalue (v_NAS_SecurityReturnValue.NasMsg)) { // As first time Registration, Registration Req should be replayed // overwrite 2nd message received with the replayed msg as this will be the complete message, so it can be checked again v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg := valueof(v_NAS_SecurityReturnValue.NasMsg); v_ReceivedMsg.Pdu.PiggybackedPduList[0].PiggybackedPduList := omit; // @sic R5s210164 sic@ if (not f_Check_NG_RegistrationReqMsg (p_GMM_MobilityInfo_Type, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Replayed Registration Request Message Failed"); } } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Not Replayed"); } } // Step 9Aa1 - 9Aa2 … return valueof(v_NAS_SecurityReturnValue.NasCount); }

After Change

function f_NR5GC_RRC_Idle_Steps5_9_AKA (inout GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NAS_PlmnId p_PLMN, TypeOfRegistration_Type p_RegType := Initial_Secure, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, boolean p_RacsEnabled := false) runs on NR5GC_PTC return NasCount_Type { var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var NG_NAS_MSG_Indication_Type v_ReceivedMsg := p_ReceivedMsg; var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var SecurityHeaderType v_SentSecurityHeader; var template (present) SecurityHeaderType v_SecurityHeaderResponse; … v_SecurityParams := f_NR5GC_Security_Get(); // Do NAS Security Mode procedure v_NAS_SecurityReturnValue := f_NR_NASSecurityModeProcedure(p_NR_CellId, v_SecurityParams, p_RegType, p_GMM_MobilityInfo_Type); if (p_RegType == Initial_NoSecurity) { if (isvalue (v_NAS_SecurityReturnValue.NasMsg)) { // As first time Registration, Registration Req should be replayed // overwrite 2nd message received with the replayed msg as this will be the complete message, so it can be checked again v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg := valueof(v_NAS_SecurityReturnValue.NasMsg); v_ReceivedMsg.Pdu.PiggybackedPduList[0].PiggybackedPduList := omit; // @sic R5s210164 sic@ if(p_RacsEnabled) { if (not f_Check_NG_RegistrationReqMsg (p_GMM_MobilityInfo_Type, v_ReceivedMsg.Pdu, Initial_Secure, -, -, -, -, cdr_NG_GMM_Cap_RACS)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Replayed Registration Request Message Failed"); } } else{ if (not f_Check_NG_RegistrationReqMsg (p_GMM_MobilityInfo_Type, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Replayed Registration Request Message Failed"); } } }else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Not Replayed"); } } // Step 9Aa1 - 9Aa2 … return valueof(v_NAS_SecurityReturnValue.NasCount); }

  - Change 5

| Template name | cdr_NG_GMM_Cap_RACS |
| --- | --- |
| Reason for change | As referred in change 4, cdr_NG_GMM_Cap_RACS is used in function f_NR5GC_RRC_Idle_Steps5_9_AKA |
| Summary of change | Moved template from file RACS_NR5GC.ttcn to NR5GC_NASSteps.ttcn |
| TTCN module | NR5GC_NASSteps.ttcn, RACS_NR5GC.ttcn |
| MCC160 Comment | Not needed |

Before Change

template (present) NG_GMM_Cap cdr_NG_GMM_Cap_RACS modifies cr_NG_GMM_CapAny := { racs := '1'B }; //Removed from File RACS_NR5GC.ttcn

After Change

template (present) NG_GMM_Cap cdr_NG_GMM_Cap_RACS modifies cr_NG_GMM_CapAny := { racs := '1'B }; //Added in File RACS_NR5GC.ttcn

  - Change 6

| Template name | cr_NG_GMM_CapAny |
| --- | --- |
| Reason for change | As referred in change 5, GMM capablilty may contain multiple IE which may cause change in IE length. |
| Summary of change | Can expect any length. |
| TTCN module | NR5GC_NASSteps.ttcn, RACS_NR5GC.ttcn |
| MCC160 Comment | Already implemented in wk37 |

Before Change

template (present) NG_GMM_Cap cr_NG_GMM_CapAny := { /* @status APPROVED (NR5GC_IRAT) */ /* 24.501 cl. 9.11.3.1 */ iei := '10'O, /* present in case of TLV; omit in case of LV */ iel := '01'O, /* present in case of LV or TLV; omit in case of V */ sgc := ?, /* Sep20 @sic R5s201387 Baseline Moving sic@ */ ngIPHC_CP_CIoT := ?, /* Sep20 @sic R5s201387 Baseline Moving sic@ */ n3Data := ?, /* Sep20 @sic R5s201387 Baseline Moving sic@ */ ng_CP_CIoT := ?, /* Sep20 @sic R5s201387 Baseline Moving sic@ */ restrictEC := ?, /* Sep20 @sic R5s201387 Baseline Moving sic@ */ lpp := ?, /* Dec18 */ hoAttach := ?, s1Cap := ?, racs := *, /* Sep20 @sic R5s201387 Baseline Moving, R5s201531 sic@ */ … spare := omit };

After Change

template (present) NG_GMM_Cap cr_NG_GMM_CapAny := { /* @status APPROVED (NR5GC_IRAT) */ /* 24.501 cl. 9.11.3.1 */ iei := '10'O, /* present in case of TLV; omit in case of LV */ iel := ? /* present in case of LV or TLV; omit in case of V */ // TTCN changes for 9.1.9.1 sgc := ?, /* Sep20 @sic R5s201387 Baseline Moving sic@ */ ngIPHC_CP_CIoT := ?, /* Sep20 @sic R5s201387 Baseline Moving sic@ */ n3Data := ?, /* Sep20 @sic R5s201387 Baseline Moving sic@ */ ng_CP_CIoT := ?, /* Sep20 @sic R5s201387 Baseline Moving sic@ */ restrictEC := ?, /* Sep20 @sic R5s201387 Baseline Moving sic@ */ lpp := ?, /* Dec18 */ hoAttach := ?, s1Cap := ?, racs := *, /* Sep20 @sic R5s201387 Baseline Moving, R5s201531 sic@ */ … spare := omit };

MCC160 Implementation

function f_TC_9_1_9_1_NR5GC() runs on NR5GC_PTC

{

//RACS / Network assigned UE radio capability ID

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN);

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var template (value) NG_MobileIdentity v_GUTIToSend;

var NG_NAS_GutiParameters_Type v_GutiParams;

var NAS_PlmnId v_PLMN;

var TrackingAreaCode v_TAC;

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

var template (value) NG_TrackingAreaIdList v_TAIList;

f_NR5GC_Init_NAS(NR_2);

// Create and configure cells

f_NR_CellConfig_Def(v_NGC_NASCellA);

f_NR_CellConfig_Def(v_NGC_NASCellB);

v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13 (v_NGC_NASCellA);

if (not match (v_GMM_MobilityInfo.GMMCap, cdr_NG_GMM_Cap_RACS))   {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA));

v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA);

v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams);

v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA);

v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)});

// Registration Accepted?

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure,

v_GMM_MobilityInfo,

v_GUTIToSend,

v_TAIList,

omit,

-, -, -, -, -,

-, -, -, -, -,

-, -, -, -, -,

-, -, -, -, -,

-, -, -, -, -,

-, -, -, -, -, -, cs_UERadioCapId('67'O, '100000000001'O)

);

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));

f_NR5GC_RRC_Idle_Steps15_20(v_NGC_NASCellA, TESTMode_OFF);

f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellA, STATE_IDLE_1A);

f_NR_TestBody_Set(true);

fl_TC_9_1_9_1_Body();

f_NR_TestBody_Set(false);

// Finish

f_NR_Postamble(v_NGC_NASCellB, STATE_IDLE_1A);

}

- Branches Executed

The test case was executed on NR band n78.

- Execution Log Files

  - Samsung SM-G996B(S22_Palette)

The Samsung SM-G996B(S22_Palette) UE  passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_9_1_9_1_LOG.xml

- PICS/PIXIT parameter file:

TC_9_1_9_1_PICS.txt