3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211245

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0742 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TP parameter indicator template for SMS Status reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TP-PI in SMS-DELIVER-REPORT has bits where TP-UDL and TP-DCS are optional as per 23.040 and UE may or may not signal them. As such their value should not be checked for specific value. Due to a TTCN bug which was fixed by R5s210112 the TP-PI was checked at all. Once fixed by R5s210112 some UEs started failing the MT SMS tests due to signalling values in TP-UDL and TP-DCS. Prose CR associated with this change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TP-UDL and TP-DCS in SMS-DELIVER-REPORT template are now set to ‘ANY’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | .Conformant UEs may unfairly FAIL the SMS MT test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | MT SMS test cases(9.4.NR5GC) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34.229-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Template name | cs_SMS_DELIVER |
| --- | --- |
| Reason for change | . TP-PI in SMS-DELIVER-REPORT has bits where TP-UDL and TP-DCS are optional as per 23.040 and UE may or may not signal them. As such their value should not be checked for specific value. Due to a TTCN bug which was fixed by R5s210112 the TP-PI was checked at all. Once fixed by R5s210112 some UEs started failing the MT SMS tests due to signalling values in TP-UDL and TP-DCS. Prose CR associated with this change. |
| Summary of change | TP-UDL and TP-DCS in SMS-DELIVER-REPORT template are now set to ‘ANY’. |
| TTCN module | Common\NAS\SMS_Templates.ttcn |
| MCC160 Comment | Accepted in principle - conditional to prose CR agreement at RAN5#93 MCC160 implementation see below |

Before Change

template (value) TP_ParameterIndicator_Type crs_TP_ParameterIndicator := // @sic R5s210112 sic@ { /* @status APPROVED (IMS, LTE_A_R12) */ extBit1 := '0'B, spare4 := '0000'B, tP_UDL := '0'B, tP_DCS := '0'B, tP_PID := '0'B };

After Change

template (value) TP_ParameterIndicator_Type crs_TP_ParameterIndicator := // @sic R5s210112 sic@ { /* @status APPROVED (IMS, LTE_A_R12) */ extBit1 := '0'B, spare4 := '0000'B, tP_UDL := '0'B, tP_DCS := '?'B, tP_PID := '?'B };

MCC160 Implementation

The original template was completely specified with values (crs_TP_ParameterIndicator). The modified template contains ‘?’. Therefore different templates have to be used in send and receive direction.

Modified/new templates + Resulting changes of other templates

template (present) TP_ParameterIndicator_Type cr_TP_ParameterIndicator := // @sic R5s210112 sic@ { /* @status APPROVED (IMS, LTE_A_R12) */ extBit1 := '0'B, spare4 := '0000'B, tP_UDL := '0'B, tP_DCS := '?'B, tP_PID := '?'B };

template (value) TP_ParameterIndicator_Type cs_TP_ParameterIndicator := // @sic R5s210112 sic@ { /* @status APPROVED (IMS, LTE_A_R12) */ extBit1 := '0'B, spare4 := '0000'B, tP_UDL := '0'B, tP_DCS := '0'B, tP_PID := '0'B };

template (value) SMS_STATUS_REPORT_Type cs_SMS_STATUS_REPORT(TP_MessageReference_Type p_MessageRef, TP_Address_Type p_RA, template (omit) TP_ServiceCentreTimeStamp_Type p_SCTP):= { // TS 34.229-1 clause A.7.5 … tP_Status := { reserved := '0'B, status := '0000000'B }, tP_ParameterIndicator := cs_TP_ParameterIndicator, // @sic R5s210112, R5s211245 sic@ tP_ProtocolIdentifier := cs_TP_ProtocolIdentifier, … };

template (present) SMS_DELIVER_REPORT_Type cr_SMS_DELIVER_REPORT := { /* @status APPROVED (IMS) */ … tP_FailureCause := omit, tP_ParameterIndicator := cr_TP_ParameterIndicator, // @sic R5s210112, R5s211245 sic@ tP_ProtocolIdentifier := *, // @sic R5-205345 sic@ .. };

template (value) SMS_SUBMIT_REPORT_Type cs_SMS_SUBMIT_REPORT := { /* @status APPROVED (IMS, LTE_A_R12) */ … tP_FailureCause := omit, tP_ParameterIndicator := cs_TP_ParameterIndicator, // @sic R5s210112, R5s211245 sic@ tP_ServiceCentreTimeStamp := f_BCD_TimestampWithTimezone(), … };