3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211248

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2015 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NRDC functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When creating the SCG cell, the v_NR_RachProcedure is populated with contentionbased with CCCH message to send RRCSetup, this is incorrect as the RACH procedure should be similar to EN-DC where once message3 is on SCG cell the CBRA procedure is completed with an uplink grant. In function f_ReconfigureSS_DRBs_NRDC the CRNTI is updated and SS is reconfigured with updated C-RNTI, this additional SS configuration can be avoided and the C-RNTI updated during creation of cell In order to configure the RACH procedure list correctly the parameter needs to be set to true to indicate the secondary cell configuration. Since the C-RNTI change is removed from f_ReconfigureSS_DRBs_NRDC, the f_NR_CellInfo_SetRNTI is called to update the C-RNTI value during cell creation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a parameter to indicate secondary Cell and used this to determine the RACH procedure list Removed set C-RNTI and f_NR_SS_C_RNTI_Config. Updated f_NR_CellConfig_Def to indicate as Secondary cell for nr_cell10 Called f_NR_CellInfo_SetRNTI to initialise c-rnti value Note: the parameter introduced here is also referenced in R5s210891 Chg 2 ALL NR-DC test cases needs to be updated with similar changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SS configuration for RACH procedure will remain incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | ALL NR-DC test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required for NRDC functions	3

1.1	Correction to f_NR_InitialiseRachProcedure	3

1.2	Correction to f_NR_CellConfig_Def	5

1.3	Correction to f_ReconfigureSS_DRBs_NRDC	7

1.4	Correction to f_TC_8_2_2_4_2_NR5GC	8

# Corrections required for NRDC functions

## Correction to f_NR_InitialiseRachProcedure

| Function name | f_NR_InitialiseRachProcedure |
| --- | --- |
| Reason for change | When creating the SCG cell, the v_NR_RachProcedure is populated with contentionbased with CCCH message to send RRCSetup, this is incorrect as the RACH procedure should be similar to EN-DC where once message3 is on SCG cell the CBRA procedure is completed with an uplink grant. |
| Summary of change | Added a parameter to indicate secondary Cell and used this to determine the RACH procedure list |
| TTCN module | NR_ConfigurationSteps |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_InitialiseRachProcedure(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return template(value) NR_RachProcedureConfig_Type { var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); var template(value) DL_CCCH_Message v_RrcSetup; var template(value) NR_RachProcedureMsg4RrcMsg_Type v_RachProcedureMsg4RrcMsg; var octetstring v_EncodedRrcSetup; if (f_GetTestcaseAttrib_ENDC(testcasename())) { //ENDC test cases -> C-RNTI based CBRA procedure // As per 38.523-3 Table 7.1.2.3.3-1: Default grants @sic R5-198994 sic@ //WA#WI=858265 7_1_1_4_1_1 requires shortDCI and the RACHprocedure configuration should apply the DCI format correctly according to the UE search space configuration if( not (f_GetTestcaseAttrib_ShortDCI(testcasename()))) { v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, -,-, tsc_NR_ENDC_Msg3Grant_Def); /* @sic R5-187672, R5-187748: CBRA instead of CFRA sic@ */ } else { v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased_DCI_x_0(v_NR_PhysicalParameters,-,-,tsc_NR_ENDC_Msg3Grant_Def); } } else { //NRSA test cases -> Contention Based with RRCSetupRequest in Msg3 if (not f_NR_EarlyContentionResolution()) { v_RrcSetup := f_NR_38508_RRCSetup(p_NR_CellId); v_EncodedRrcSetup := f_NR_RRC_DL_CCCH_Encvalue(v_RrcSetup); v_RachProcedureMsg4RrcMsg := cs_NR_RachProcedureMsg4WithCcchMsg(v_EncodedRrcSetup); } else { v_RachProcedureMsg4RrcMsg := cs_NR_RachProcedureMsg4WithNoRrcMsg; } v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, v_RachProcedureMsg4RrcMsg); } return v_NR_RachProcedure; }

After Change:

function f_NR_InitialiseRachProcedure(NR_CellId_Type p_NR_CellId, boolean p_NRDC_secondary_Cell :=false) runs on NR_BASE_PTC return template(value) NR_RachProcedureConfig_Type { var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); var template(value) DL_CCCH_Message v_RrcSetup; var template(value) NR_RachProcedureMsg4RrcMsg_Type v_RachProcedureMsg4RrcMsg; var octetstring v_EncodedRrcSetup; if (f_GetTestcaseAttrib_ENDC(testcasename()) or p_NRDC_secondary_Cell) { //ENDC test cases -> C-RNTI based CBRA procedure //WA#WI=880752 // As per 38.523-3 Table 7.1.2.3.3-1: Default grants @sic R5-198994 sic@ //WA#WI=858265 7_1_1_4_1_1 requires shortDCI and the RACHprocedure configuration should apply the DCI format correctly according to the UE search space configuration if( not (f_GetTestcaseAttrib_ShortDCI(testcasename()))) { v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, -,-, tsc_NR_ENDC_Msg3Grant_Def); /* @sic R5-187672, R5-187748: CBRA instead of CFRA sic@ */ } else { v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased_DCI_x_0(v_NR_PhysicalParameters,-,-,tsc_NR_ENDC_Msg3Grant_Def); } } else { //NRSA test cases -> Contention Based with RRCSetupRequest in Msg3 if (not f_NR_EarlyContentionResolution()) { v_RrcSetup := f_NR_38508_RRCSetup(p_NR_CellId); v_EncodedRrcSetup := f_NR_RRC_DL_CCCH_Encvalue(v_RrcSetup); v_RachProcedureMsg4RrcMsg := cs_NR_RachProcedureMsg4WithCcchMsg(v_EncodedRrcSetup); } else { v_RachProcedureMsg4RrcMsg := cs_NR_RachProcedureMsg4WithNoRrcMsg; } v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, v_RachProcedureMsg4RrcMsg); } return v_NR_RachProcedure; }

## Correction to f_NR_CellConfig_Def

| Function name | f_NR_CellConfig_Def |
| --- | --- |
| Reason for change | When creating the SCG cell, the v_NR_RachProcedure is populated with contentionbased with CCCH message to send RRCSetup, this is incorrect as the RACH procedure should be similar to EN-DC where once message3 is on SCG cell the CBRA procedure is completed with an uplink grant. |
| Summary of change | Added a parameter to indicate secondary Cell and used this to determine the RACH procedure list. Note : the parameter introduced here is also referenced in R5s210891 Chg 2 |
| TTCN module | NR_ConfigurationSteps |
| MCC160 Comment | Accepted, implemented using the parameter p_NRDC_SCG_Cell added with R5s210891 |

Before Change:

function f_NR_CellConfig_Def(NR_CellId_Type p_NR_CellId, template (omit) NR_CellInitialCAConfig_Type p_CA_CellInitialConfig := None ) runs on NR_BASE_PTC { // @sic R5s200038: set default value of p_CA_CellInitialConfig to None sic@ var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var template(omit) NR_CellConfigPhysicalLayerDownlink_Type v_NR_CellConfigPhysicalLayerDownlink := cs_NR_CellConfigPhysicalLayerDownlink(v_NR_CellInfo); var template(omit) NR_CellConfigPhysicalLayerUplink_Type v_NR_CellConfigPhysicalLayerUplink := omit; var template(omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(p_NR_CellId); var template(omit) NR_PcchConfig_Type v_NR_PcchConfig := f_NR_InitialisePcchConfig(p_NR_CellId); var template(omit) NR_RachProcedureConfig_Type v_NR_RachProcedure; var FreqBandIndicatorNR v_FreqBandIndicatorNR := f_NR_CellInfo_GetBandIndicator(p_NR_CellId); if (f_NR_IsBandSDL(v_FreqBandIndicatorNR)) { //SDL v_NR_CellConfigPhysicalLayerUplink := omit; v_NR_RachProcedure := omit } else { v_NR_CellConfigPhysicalLayerUplink := cs_NR_CellConfigPhysicalLayerUplink(v_NR_CellInfo); v_NR_RachProcedure := f_NR_InitialiseRachProcedure(p_NR_CellId); }

After Change:

function f_NR_CellConfig_Def(NR_CellId_Type p_NR_CellId, template (omit) NR_CellInitialCAConfig_Type p_CA_CellInitialConfig := None, boolean p_NRDC_secondary_Cell :=false // ) runs on NR_BASE_PTC { // @sic R5s200038: set default value of p_CA_CellInitialConfig to None sic@ var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var template(omit) NR_CellConfigPhysicalLayerDownlink_Type v_NR_CellConfigPhysicalLayerDownlink := cs_NR_CellConfigPhysicalLayerDownlink(v_NR_CellInfo); var template(omit) NR_CellConfigPhysicalLayerUplink_Type v_NR_CellConfigPhysicalLayerUplink := omit; var template(omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(p_NR_CellId); var template(omit) NR_PcchConfig_Type v_NR_PcchConfig := f_NR_InitialisePcchConfig(p_NR_CellId); var template(omit) NR_RachProcedureConfig_Type v_NR_RachProcedure; var FreqBandIndicatorNR v_FreqBandIndicatorNR := f_NR_CellInfo_GetBandIndicator(p_NR_CellId); if (f_NR_IsBandSDL(v_FreqBandIndicatorNR)) { //SDL v_NR_CellConfigPhysicalLayerUplink := omit; v_NR_RachProcedure := omit } else { v_NR_CellConfigPhysicalLayerUplink := cs_NR_CellConfigPhysicalLayerUplink(v_NR_CellInfo); v_NR_RachProcedure := f_NR_InitialiseRachProcedure(p_NR_CellId, p_NRDC_secondary_Cell ); }

## Correction to f_ReconfigureSS_DRBs_NRDC

| Function name | f_ReconfigureSS_DRBs_NRDC |
| --- | --- |
| Reason for change | In this function the CRNTI is updated and SS is reconfigured with updated C-RNTI, this additional SS configuration can be avoided and the C-RNTI updated during creation of cell |
| Summary of change | Removed set C-RNTI and f_NR_SS_C_RNTI_Config. |
| TTCN module | NR5GC_RRCSteps_NRDC |
| MCC160 Comment | Not needed, the C-RNTI value is paremeterised, it can be set to any value in the RRCReconfiguration message depending on the test case prose. Therefore the parameter shall be kept and the C-RNTI configured in this function, i.e. when the SCG cell group is added. |

Before Change:

function f_ReconfigureSS_DRBs_NRDC( NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_RadioBearerList_Type p_SS_ConfigMCG, template (omit) NR_RadioBearerList_Type p_SS_ConfigSCG, NRDC_SecurityActivation_Type p_NRDC_SecurityActivation, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4 ) runs on NR_BASE_PTC { var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release if (isvalue(p_SS_ConfigMCG)) { f_NR_SS_CommonRadioBearerConfig(p_MCG_CellId, p_SS_ConfigMCG, p_TimingInfo); } //Reconfigure SS SCG if (isvalue(p_SS_ConfigSCG)) { f_NR_SS_CommonRadioBearerConfig(p_SCG_CellId, p_SS_ConfigSCG, p_TimingInfo); f_NR_CellInfo_SetRNTI(p_SCG_CellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_SCG_CellId, p_RNTI_Value); f_NR_ULGrantConfiguration_Start(p_SCG_CellId); }

After Change:

function f_ReconfigureSS_DRBs_NRDC( NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_RadioBearerList_Type p_SS_ConfigMCG, template (omit) NR_RadioBearerList_Type p_SS_ConfigSCG, NRDC_SecurityActivation_Type p_NRDC_SecurityActivation, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4 ) runs on NR_BASE_PTC { var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release if (isvalue(p_SS_ConfigMCG)) { f_NR_SS_CommonRadioBearerConfig(p_MCG_CellId, p_SS_ConfigMCG, p_TimingInfo); } //Reconfigure SS SCG if (isvalue(p_SS_ConfigSCG)) { f_NR_SS_CommonRadioBearerConfig(p_SCG_CellId, p_SS_ConfigSCG, p_TimingInfo); //f_NR_CellInfo_SetRNTI(p_SCG_CellId, p_RNTI_Value); // removed //f_NR_SS_C_RNTI_Config(p_SCG_CellId, p_RNTI_Value); // removed f_NR_ULGrantConfiguration_Start(p_SCG_CellId); }

## Correction to f_TC_8_2_2_4_2_NR5GC

| Function name | f_TC_8_2_2_4_2_NR5GC ( and all other NR-DC test cases |
| --- | --- |
| Reason for change | In order to configure the RACH procedure list correctly the parameter needs to be set to true to indicate the secondary cell configuration. Since the C-RNTI change is removed from f_ReconfigureSS_DRBs_NRDC, the f_NR_CellInfo_SetRNTI is called to update the C-RNTI value during cell creation |
| Summary of change | Updated f_NR_CellConfig_Def to indicate as Secondary cell for nr_cell10 Called f_NR_CellInfo_SetRNTI to initialise c-rnti value Note: the parameter introduced here is also referenced in R5s210891 Chg 2 ALL NR-DC test cases needs to be updated with similar changes |
| TTCN module | RRC_Reconfiguration_NRDC_NR5GC |
| MCC160 Comment | - Accepted, implemented as per R5s210891 - Not needed see change 1.3. The C-RNTI value is configured in the SCG cell when it is added to the cell group |

Before Change:

function f_TC_8_2_2_4_2_NR5GC() runs on NR5GC_PTC { //PSCell addition, modification and release / SCG DRB / NR-DC // @sic R5-203650 sic@ f_NR5GC_Init_NRDC(NR_1, {nr_Cell1, nr_Cell10}); //WA#WI=857215 R5s210716 //Create and configure NR cell1 and NR cell 10. f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10);

After Change:

*/ function f_TC_8_2_2_4_2_NR5GC() runs on NR5GC_PTC { //PSCell addition, modification and release / SCG DRB / NR-DC // @sic R5-203650 sic@ f_NR5GC_Init_NRDC(NR_1, {nr_Cell1, nr_Cell10}); //WA#WI=857215 R5s210716 //Create and configure NR cell1 and NR cell 10. f_NR_CellConfig_Def(nr_Cell1); f_NR_CellInfo_SetRNTI(nr_Cell10, tsc_C_RNTI_Value4); //WA#WI=880752 f_NR_CellConfig_Def(nr_Cell10,-,true); // Change already covered in