3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211330

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2083 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NRDC test case 8.2.2.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Function f_NR_GetDefaultDRB_SCG() is called to initialise v_DRBId, but this function will not return a valid result before any SCG is established. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved the call to f_NR_GetDefaultDRB_SCG() to initialise v_DRBId after the SCG Bearer is established. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.5.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

## 2.1 Change 1

| Function name | f_TC_8_2_2_5_2_TestBody() |
| --- | --- |
| Reason for change | The Function f_NR_GetDefaultDRB_SCG() is called to initialise v_DRBId, but this function will not return a valid result before any SCG is established. |
| Summary of change | Moved call to f_NR_GetDefaultDRB_SCG() to initialise v_DRBId after the SCG Bearer is established. |
| TTCN module | RRC_Reconfiguration_NRDC_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change

function f_TC_8_2_2_5_2_TestBody() runs on NR5GC_PTC { var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) RRCReconfiguration_v1560_IEs v_V1560Ext; var template (omit) RadioBearerConfig v_RadioBearerConfig2; var template GSM_MobilityInfo_Type v_PDUSessionInfo; var template(value) CellGroupConfig v_CellGroupConfigMCG; var template (value) DRB_ToAddModList v_DRBToAddModList; var DRB_Identity v_DRBId :=f_NR_GetDefaultDRB_SCG(); …. //@siclog "Step 3" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB over the SCG path?// @sic R5-206342 sic@ v_PDUSessionInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); f_NR5GC_CheckDataPath(nr_Cell10, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo)), dedicatedBearer, v_DRBId, "Step 3", -, -, nr_Cell10); //@siclog "Step 4" siclog@ …. }//End of fl_TC_8_2_2_5_2_TestBody

After Change

function f_TC_8_2_2_5_2_TestBody() runs on NR5GC_PTC { var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) RRCReconfiguration_v1560_IEs v_V1560Ext; var template (omit) RadioBearerConfig v_RadioBearerConfig2; var template GSM_MobilityInfo_Type v_PDUSessionInfo; var template(value) CellGroupConfig v_CellGroupConfigMCG; var template (value) DRB_ToAddModList v_DRBToAddModList; var DRB_Identity v_DRBId ; // Removed := f_NR_GetDefaultDRB_SCG(); …… //@siclog "Step 3" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB over the SCG path?// @sic R5-206342 sic@ v_PDUSessionInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); v_DRBId :=f_NR_GetDefaultDRB_SCG(); f_NR5GC_CheckDataPath(nr_Cell10, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo)), dedicatedBearer, v_DRBId, "Step 3", -, -, nr_Cell10); //@siclog "Step 4" siclog@ …… }//End of fl_TC_8_2_2_5_2_TestBody