3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211343

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2093 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC multilayer test case 11.4.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following implementation of MCC comments to R5s210810, IMS1 PTC is using the function f_IMS_IpcanReleaseWithOptionalDeregistration(), which calls f_IMS_OptionalDeregistrationOrC24Reinvite() with parameter p_WaitForIpcanRelease defaulting to TRUE. This requires an extra trigger from NR5GC PTC to indicate IPCAN release has completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add trigger to IMS1 PTC following UE switch off, to unblock function f_IMS_OptionalDeregistrationOrC24Reinvite () |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_11_4_8_NR5GC() |
| --- | --- |
| Reason for change | Following implementation of MCC comments to R5s210810, IMS1 PTC is using the function f_IMS_IpcanReleaseWithOptionalDeregistration(), which calls f_IMS_OptionalDeregistrationOrC24Reinvite() with parameter p_WaitForIpcanRelease defaulting to TRUE. This requires an extgra trigger from NR5GC PTC to indicate IPCAN release has completed. |
| Summary of change | Add trigger to IMS1 PTC following UE switch off, to unblock function f_IMS_OptionalDeregistrationOrC24Reinvite () |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_11_4_8_NR5GC() runs on NR5GC_PTC { //Handling of Local and Extended emergency numbers / Switch-off and maximum local numbers storage var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var TrackingAreaCode v_TAC; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_MobileIdentity v_GUTIToSend; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) EmergNumList v_EmergNumList; // emergency numbers in format as per 24.008 var template (value) ExtdEmergNumList v_XtdEmergNumList; // emergency numbers in format as per 24.301 f_NR5GC_Init(NR_1); // Emergency support set in default message contents //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell12); // The UE is equipped with a USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-20. f_UT_USIM_Insert(UT, "USIM configuration 20 specified in table 6.4.1-20 TS 38.508-1"); v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13(nr_Cell1); // Registration Accept v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); v_EmergNumList := f_Build_EmergNumList (1, {'120'H}); v_XtdEmergNumList := f_Build_ExtdEmergNumList (1, {'10'H}, '0'B); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_EmergNumList, v_XtdEmergNumList); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20 (nr_Cell1, TESTMode_OFF); f_NR_UE_DeRegisterOnSwitchOff (nr_Cell1, STATE_IDLE_1A); // cell powers will already be: All off expect Cell 1 = Serving Cell f_NR_TestBody_Set(true); fl_TC_11_4_8_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell12, STATE_IDLE_1A); }

After Change

function f_TC_11_4_8_NR5GC() runs on NR5GC_PTC { //Handling of Local and Extended emergency numbers / Switch-off and maximum local numbers storage var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var TrackingAreaCode v_TAC; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_MobileIdentity v_GUTIToSend; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) EmergNumList v_EmergNumList; // emergency numbers in format as per 24.008 var template (value) ExtdEmergNumList v_XtdEmergNumList; // emergency numbers in format as per 24.301 f_NR5GC_Init(NR_1); // Emergency support set in default message contents //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell12); // The UE is equipped with a USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-20. f_UT_USIM_Insert(UT, "USIM configuration 20 specified in table 6.4.1-20 TS 38.508-1"); v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13(nr_Cell1); // Registration Accept v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); v_EmergNumList := f_Build_EmergNumList (1, {'120'H}); v_XtdEmergNumList := f_Build_ExtdEmergNumList (1, {'10'H}, '0'B); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_EmergNumList, v_XtdEmergNumList); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20 (nr_Cell1, TESTMode_OFF); f_NR_UE_DeRegisterOnSwitchOff (nr_Cell1, STATE_IDLE_1A); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // cell powers will already be: All off expect Cell 1 = Serving Cell f_NR_TestBody_Set(true); fl_TC_11_4_8_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell12, STATE_IDLE_1A); }