3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211399

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2107 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EN-DC MAC test case 7.1.3.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The DRB type to be configured is passed into this function in parameter p_DRB_Type, but this is then not passed on to the function f_EUTRA38_508_ENDC_Reconfiguration().Currently the value SCG_RBConfig is always passed. As a result the SS may not be configured correctly, e.g. for bearer type MCG_SPLIT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass bearer type parameter on to function f_EUTRA38_508_ENDC_Reconfiguration() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 7.1.3.5.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_EUTRA38_ENDC_ReConfigAM_UM() |
| --- | --- |
| Reason for change | The DRB type to be configured is passed into this function in parameter p_DRB_Type, but this is then not passed on to the function f_EUTRA38_508_ENDC_Reconfiguration(). Currently the value SCG_RBConfig is always passed. As a result the SS may not be configured correctly, e.g. for bearer type MCG_SPLIT. |
| Summary of change | Pass bearer type parameter on to function f_EUTRA38_508_ENDC_Reconfiguration() |
| TTCN module | EUTRA38_ENDC_CommonProcedures.ttcn |
| MCC160 Comment | Alternative solution implemented: specified the SS DRB config info in the call to this function |

Before Change

function f_EUTRA38_ENDC_ReConfigAM_UM(EUTRA_CellId_Type p_CellId, integer p_NoOfNR_AMBearersN, integer p_NoOfNR_UMBearersM, EUTRA_RRC_STATE_Type p_RRC_State := RRC_IDLE, template (omit) integer p_SK_Counter := omit, template (omit) NAS_MSG_Request_Type p_NASMsg := omit, template (omit) RadioResourceConfigDedicated p_RadioResourceConfigDedicated := omit, template (omit) RadioBearerList_Type p_DrbConfigAtSS := omit, ENDC_Bearer_Type p_DRB_Type:= MCG_SCG ) runs on EUTRA_5GS_PTC { //@sic R5s200338 : added p_DRB_Type sic@ var template (omit) NAS_MSG_RequestList_Type v_ActivateBearer := omit; var template (omit) RadioResourceConfigDedicated v_RadioResourceConfigDedicated; var integer i; if (isvalue(p_NASMsg)) { v_ActivateBearer := {p_NASMsg}; } else if (p_RRC_State == RRC_IDLE) { //@sic R5s190288 additional change sic@ // Dedicated bearer as per default messages for (i:=0; i< (p_NoOfNR_AMBearersN+p_NoOfNR_UMBearersM); i := i+1) { //@sic R5s190163 change 4.1 sic@ v_ActivateBearer[i] := f_EPS_GetActivateDedicatedBearer (int2hex(6+i, 1), cds_508_EPS_QoS_Dedicated_6_tlv, f_EUTRA_Get_508_TrafficFlowTemplate(6, int2hex(6+i, 1), -, PDN_2)); // @sic R5s190188, R5s190607 sic@ } } if (isvalue(p_RadioResourceConfigDedicated)) { v_RadioResourceConfigDedicated := p_RadioResourceConfigDedicated; } else { v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(p_CellId, p_DRB_Type, p_RRC_State); } f_EUTRA38_508_ENDC_Reconfiguration (p_CellId, SCG_RBConfig, p_SK_Counter, p_RRC_State, v_ActivateBearer, v_RadioResourceConfigDedicated, p_DrbConfigAtSS); }

After Change

function f_EUTRA38_ENDC_ReConfigAM_UM(EUTRA_CellId_Type p_CellId, integer p_NoOfNR_AMBearersN, integer p_NoOfNR_UMBearersM, EUTRA_RRC_STATE_Type p_RRC_State := RRC_IDLE, template (omit) integer p_SK_Counter := omit, template (omit) NAS_MSG_Request_Type p_NASMsg := omit, template (omit) RadioResourceConfigDedicated p_RadioResourceConfigDedicated := omit, template (omit) RadioBearerList_Type p_DrbConfigAtSS := omit, ENDC_Bearer_Type p_DRB_Type:= MCG_SCG ) runs on EUTRA_5GS_PTC { //@sic R5s200338 : added p_DRB_Type sic@ var template (omit) NAS_MSG_RequestList_Type v_ActivateBearer := omit; var template (omit) RadioResourceConfigDedicated v_RadioResourceConfigDedicated; var integer i; if (isvalue(p_NASMsg)) { v_ActivateBearer := {p_NASMsg}; } else if (p_RRC_State == RRC_IDLE) { //@sic R5s190288 additional change sic@ // Dedicated bearer as per default messages for (i:=0; i< (p_NoOfNR_AMBearersN+p_NoOfNR_UMBearersM); i := i+1) { //@sic R5s190163 change 4.1 sic@ v_ActivateBearer[i] := f_EPS_GetActivateDedicatedBearer (int2hex(6+i, 1), cds_508_EPS_QoS_Dedicated_6_tlv, f_EUTRA_Get_508_TrafficFlowTemplate(6, int2hex(6+i, 1), -, PDN_2)); // @sic R5s190188, R5s190607 sic@ } } if (isvalue(p_RadioResourceConfigDedicated)) { v_RadioResourceConfigDedicated := p_RadioResourceConfigDedicated; } else { v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(p_CellId, p_DRB_Type, p_RRC_State); } f_EUTRA38_508_ENDC_Reconfiguration (p_CellId, p_DRB_Type, p_SK_Counter, p_RRC_State, v_ActivateBearer, v_RadioResourceConfigDedicated, p_DrbConfigAtSS); }

## MCC160 Implementation:

function f_TC_7_1_3_5_2_ENDC_EUTRA() runs on EUTRA_5GS_PTC

{//PDCP Uplink Routing / Split DRB

var template (omit) RadioBearerList_Type v_DrbConfigAtSS := omit;

f_EUTRA38_Init(EN_DC, c1);

//Create and configure cell

f_EUTRA_CellConfig_Def(eutra_Cell1);

//Preamble to enter UE in state 2

f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTModeA_ON, MCG_Split, RRC_IDLE);

//Set null cipher Algorithm

f_EUTRA_AS_CipheringAlgorithm_Set(eea0);

//Procedure to enter UE in state 2A

v_DrbConfigAtSS := {

cs_RadioBearer_Reconfig_SplitDRB2 (nr_Cell1)

};

f_EUTRA38_ENDC_ConfigAM_UM (eutra_Cell1, 1, 0, TEST_LOOPModeA_ON,-, -, -, -, v_DrbConfigAtSS, MCG_Split); //@sic R5-200338 sic@

f_EUTRA_TestBody_Set(true);

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

fl_TC_7_1_3_5_2_EUTRA_TestBody ();

f_EUTRA_TestBody_Set(false);

f_EUTRA_OpenUE_TestLoopMode_Deactivate_TestMode (eutra_Cell1);

f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED); // @sic R5-204475 sic@

// If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell!

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble"));

}