3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211487

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2122 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for SDAP test case 7.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Implementation for QoS rule number 1 is missing (required for test case 7.1.4.1) - There can only be one default QoS rule defined. Since QoS Rule#4 is set as the Default QoS Rule, QoS Rule #1 cannot also be the default QoS Rule. - Following PDU session establishment, ICMPv6 address allocation signalling will occur on QFI 1. - At steps 1 and 2 IP datagram should be created using filter#3 to use QFI 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add building for QoS rule number 1 - Changed the DQR bit for the QoSRule#1 to ‘0’ in QoS rules for PDU SESSION ESTABLISHMENT ACCEPT. Note : An associated prose CR on 38.523-1 will be raised at RAN5#93. - Start IP handling on QFI 1 - Create datagram for QFI 2, for steps 1 and 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 R5-216817 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1 Change 1	4

2.2 Change 2	6

2.3 Change 3	10

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

### 2.1 Change 1

| Function name | f_BuildQoSRuleRemoteAccess() |
| --- | --- |
| Reason for change | QoS rule number 1 is missing (required for test case 7.1.4.1) |
| Summary of change | Add building for QoS rule number 1 |
| TTCN module | NG_NAS_Common.ttcn |
| MCC160 Comment | No need to modify f_BuildQoSRuleRemoteAccess.The existing function f_BuildDefaultQoSRule is used instead. See proposed implementation below. |

Before Change:

function f_BuildQoSRuleRemoteAccess (charstring p_RuleNum) return template (value) QoS_Rule

{

var O1_Type v_Id;

var B1_Type v_DQR;

var O1_Type v_Precedence;

var B6_Type v_FlowId;

var octetstring v_Component;

var O2_Type v_Length;

var B4_Type v_FilterId;

select (p_RuleNum) {

case ("3") {

v_Id := int2oct(str2int(p_RuleNum),1);

v_DQR := '0'B;

v_Precedence := int2oct(str2int(p_RuleNum),1);

v_FlowId := '000001'B;

v_FilterId := '0010'B;

v_Component := fl_BuildRemoteAddressPacketFilter("2");

}

case ("4") {

v_Id := int2oct(str2int(p_RuleNum),1);

v_DQR := '1'B;

v_Precedence := int2oct(str2int(p_RuleNum),1);

v_FlowId := '000010'B;

v_FilterId := '0011'B;

v_Component := fl_BuildRemoteAddressPacketFilter("3");

}

case ("4a") {

v_Id := '0F'O;

v_DQR := '0'B;

v_Precedence := '0F'O;

v_FlowId := '000100'B;

v_FilterId := '1111'B;

v_Component := fl_BuildRemoteAddressPacketFilter("3a");

}

case ("5") {

v_Id := int2oct(str2int(p_RuleNum),1);

v_DQR := '0'B;

v_Precedence := int2oct(str2int(p_RuleNum),1);

v_FlowId := '000101'B;

v_FilterId := '0100'B;

v_Component := fl_BuildRemoteAddressPacketFilter("4");

}

case ("6") {

v_Id := int2oct(str2int(p_RuleNum),1);

v_DQR := '0'B;

v_Precedence := int2oct(str2int(p_RuleNum),1);

v_FlowId := '000110'B;

v_FilterId := '0101'B;

v_Component := fl_BuildRemoteAddressPacketFilter("5");

}

}

v_Length := int2oct(lengthof(v_Component) + 5, 2);   // @sic R5s190944 sic@

return cs_QoS_RuleRemoteAccess(v_Id, v_DQR, v_Precedence, v_FlowId, v_Component, v_Length, v_FilterId);

}

After Change:

function f_BuildQoSRuleRemoteAccess (charstring p_RuleNum) return template (value) QoS_Rule

{

var O1_Type v_Id;

var B1_Type v_DQR;

var O1_Type v_Precedence;

var B6_Type v_FlowId;

var octetstring v_Component;

var O2_Type v_Length;

var B4_Type v_FilterId;

select (p_RuleNum) {

case ("1") {

v_Id := int2oct(str2int(p_RuleNum),1);

v_DQR := '1'B;

v_Precedence  := bit2oct('11111111'B);

v_FlowId := '000001'B;

v_FilterId := '0000'B;

v_Component := bit2oct('00000001'B);

}

case ("3") {

v_Id := int2oct(str2int(p_RuleNum),1);

v_DQR := '0'B;

v_Precedence := int2oct(str2int(p_RuleNum),1);

v_FlowId := '000001'B;

v_FilterId := '0010'B;

v_Component := fl_BuildRemoteAddressPacketFilter("2");

}

case ("4") {

v_Id := int2oct(str2int(p_RuleNum),1);

v_DQR := '1'B;

v_Precedence := int2oct(str2int(p_RuleNum),1);

v_FlowId := '000010'B;

v_FilterId := '0011'B;

v_Component := fl_BuildRemoteAddressPacketFilter("3");

}

case ("4a") {

v_Id := '0F'O;

v_DQR := '0'B;

v_Precedence := '0F'O;

v_FlowId := '000100'B;

v_FilterId := '1111'B;

v_Component := fl_BuildRemoteAddressPacketFilter("3a");

}

case ("5") {

v_Id := int2oct(str2int(p_RuleNum),1);

v_DQR := '0'B;

v_Precedence := int2oct(str2int(p_RuleNum),1);

v_FlowId := '000101'B;

v_FilterId := '0100'B;

v_Component := fl_BuildRemoteAddressPacketFilter("4");

}

case ("6") {

v_Id := int2oct(str2int(p_RuleNum),1);

v_DQR := '0'B;

v_Precedence := int2oct(str2int(p_RuleNum),1);

v_FlowId := '000110'B;

v_FilterId := '0101'B;

v_Component := fl_BuildRemoteAddressPacketFilter("5");

}

}

v_Length := int2oct(lengthof(v_Component) + 5, 2);   // @sic R5s190944 sic@

return cs_QoS_RuleRemoteAccess(v_Id, v_DQR, v_Precedence, v_FlowId, v_Component, v_Length, v_FilterId);

}

MCC160 proposed implementation

function f_TC_7_1_4_1_NR5GC() runs on NR5GC_PTC

{//SDAP Data Transfer and PDU Header Handling UL/DL

var template (value) DRB_ToAddModList v_DRB_ToAddModList;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type;

var IntegerList_Type v_DRBList;

var integer v_FirstDRB;

var integer v_SecondDRB;

var template (value) QoS_Rules v_QoS_Rules;

var template (value) QoSFlowDescr v_QoSFlowDescr;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList;

var PDN_Index_Type v_PDN_Index;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var integer v_PduSessionId; // @sic R5s191119 sic@

f_NR5GC_Init_NAS(NR_1);

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

//Preamble

// Perform Registration and get PDU Establishment Request message

v_GSM_MobilityInfo_Type := f_MultipleQoSPreamble_Part1 (nr_Cell1);

v_PduSessionId := oct2int(v_GSM_MobilityInfo_Type.SessionId); // @sic R5s191119, R5s200564 sic@

//Get DRB identities

v_DRBList:= f_NR_GetNextDRBIds(v_GSM_MobilityInfo_Type, true, 2); // First is Default DRB

v_FirstDRB := v_DRBList[0];

v_SecondDRB := v_DRBList[1];

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_SecondDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_Configuration(v_PduSessionId, Present, Present, {1,2})), //Configure SDAP in normal mode for the moment; TTCN CR 190946

f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_FirstDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_Configuration(v_PduSessionId, Present, Present, {5,6}))}; //Configure SDAP in normal mode for the moment; TTCN CR 190946

v_DRB_ToAddModList := {cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, false, {1,2}), // @sic R5s191119 sic@

v_SecondDRB,

cs_38508_PDCP_Config(ms300)),

cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, true, {5,6}), // @sic R5s191119 sic@

v_FirstDRB,

cs_38508_PDCP_Config(ms300))};

v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule ("1"), f_BuildQoSRuleRemoteAccess("4"), f_BuildQoSRuleRemoteAccess("5"), f_BuildQoSRuleRemoteAccess("6")}); // @sic R5-215714 sic@

v_QoSFlowDescr := cs_QoSFlowDescr ({cs_QoS_Flow1, cs_QoS_Flow2, cs_QoS_Flow3, cs_QoS_Flow4});

### 2.2 Change 2

| Function name | f_TC_7_1_4_1_NR5GC() |
| --- | --- |
| Reason for change | - There can only be one default QoS rule defined. Since QoS Rule#4 is set as the Default QoS Rule, QoS Rule #1 cannot also be the default QoS Rule. - Following PDU session establishment, ICMPv6 address allocation signalling will occur on QFI 1. |
| Summary of change | - Changed the DQR bit for the QoSRule#1 to ‘0’ in QoS rules for PDU SESSION ESTABLISHMENT ACCEPT. Note : An associated prose CR on 38.523-1 will be raised at RAN5#93. - Start IP handling on QFI 1. |
| TTCN module | SDAP_NR5GC.ttcn |
| MCC160 Comment | Accepted, conditional to approval of prose CR. See one correction on Keysight’s proposed code. |

Before Change:

function f_TC_7_1_4_1_NR5GC() runs on NR5GC_PTC

{//SDAP Data Transfer and PDU Header Handling UL/DL

var template (value) DRB_ToAddModList v_DRB_ToAddModList;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type;

var IntegerList_Type v_DRBList;

var integer v_FirstDRB;

var integer v_SecondDRB;

var template (value) QoS_Rules v_QoS_Rules;

var template (value) QoSFlowDescr v_QoSFlowDescr;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList;

var PDN_Index_Type v_PDN_Index;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var integer v_PduSessionId; // @sic R5s191119 sic@

f_NR5GC_Init_NAS(NR_1);

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

//Preamble

// Perform Registration and get PDU Establishment Request message

v_GSM_MobilityInfo_Type := f_MultipleQoSPreamble_Part1 (nr_Cell1);

v_PduSessionId := oct2int(v_GSM_MobilityInfo_Type.SessionId); // @sic R5s191119, R5s200564 sic@

//Get DRB identities

v_DRBList:= f_NR_GetNextDRBIds(v_GSM_MobilityInfo_Type, true, 2); // First is Default DRB

v_FirstDRB := v_DRBList[0];

v_SecondDRB := v_DRBList[1];

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_SecondDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_Configuration(v_PduSessionId, Present, Present, {1,2})), //Configure SDAP in normal mode for the moment; TTCN CR 190946

f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_FirstDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_Configuration(v_PduSessionId, Present, Present, {5,6}))}; //Configure SDAP in normal mode for the moment; TTCN CR 190946

v_DRB_ToAddModList := {cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, false, {1,2}), // @sic R5s191119 sic@

v_SecondDRB,

cs_38508_PDCP_Config(ms300)),

cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, true, {5,6}), // @sic R5s191119 sic@

v_FirstDRB,

cs_38508_PDCP_Config(ms300))};

v_QoS_Rules := cs_QoS_Rules ({f_BuildQoSRuleRemoteAccess("1"), f_BuildQoSRuleRemoteAccess("4"), f_BuildQoSRuleRemoteAccess("5"), f_BuildQoSRuleRemoteAccess("6")}); // @sic R5-215714 sic@

v_QoSFlowDescr := cs_QoSFlowDescr ({cs_QoS_Flow1, cs_QoS_Flow2, cs_QoS_Flow3, cs_QoS_Flow4});

// Start IP handling in order for the TTCN to handle IPv6 address allocation,

// then stop it before the test body, so that TTCN can route the IP data

v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo_Type);

v_QosFlowId := cs_IP_QosFlowId(oct2int(v_GSM_MobilityInfo_Type.SessionId), 2); //IP handling will go through QFI 2, because this QFI is associated with the default QoS rule @sic R5s200564, R5s201505 sic@

f_IP_Handling_StartPDN(IP, v_PDN_Index, cs_DrbInfo_NR(nr_Cell1, v_QosFlowId));

f_MultipleQoSPreamble_Part2 (nr_Cell1,

v_GSM_MobilityInfo_Type,

v_FirstDRB,

v_SecondDRB,

v_DRB_ToAddModList,

v_SS_Drb_ConfigList,

v_QoS_Rules,

v_QoSFlowDescr,

v_PDN_Index); //@sic R5s201361 sic@

f_NR_AS_CipheringAlgorithm_Set(nea0);

// Stop IP handling before reconfiguring SDAP to transparent mode

f_IP_Handling_StopPDN(IP);

// Reconfigure SDAP to transparent mode when moving to connected state

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_SecondDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_ConfigTransparentMode(Present)), //Configure SDAP in transparent mode; TTCN CR 190946

f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_FirstDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_ConfigTransparentMode(Present))}; //Configure SDAP in transparent mode; TTCN CR 190946

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeB_ON,

-,

'01'O, //IP PDU delay  = 1 second

-,-,

v_DRB_ToAddModList,

v_SS_Drb_ConfigList);

f_NR_TestBody_Set(true);

fl_TC_7_1_4_1_NR_TestBody(v_FirstDRB, v_SecondDRB, v_GSM_MobilityInfo_Type);

f_NR_TestBody_Set(false);

f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); // @sic R5s191119 sic@

//Postamble

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

f_NR_ReleaseAllCells();

}

After Change:

function f_TC_7_1_4_1_NR5GC() runs on NR5GC_PTC

{//SDAP Data Transfer and PDU Header Handling UL/DL

var template (value) DRB_ToAddModList v_DRB_ToAddModList;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type;

var IntegerList_Type v_DRBList;

var integer v_FirstDRB;

var integer v_SecondDRB;

var template (value) QoS_Rules v_QoS_Rules;

var template (value) QoSFlowDescr v_QoSFlowDescr;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList;

var PDN_Index_Type v_PDN_Index;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var integer v_PduSessionId; // @sic R5s191119 sic@

f_NR5GC_Init_NAS(NR_1);

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

//Preamble

// Perform Registration and get PDU Establishment Request message

v_GSM_MobilityInfo_Type := f_MultipleQoSPreamble_Part1 (nr_Cell1);

v_PduSessionId := oct2int(v_GSM_MobilityInfo_Type.SessionId); // @sic R5s191119, R5s200564 sic@

//Get DRB identities

v_DRBList:= f_NR_GetNextDRBIds(v_GSM_MobilityInfo_Type, true, 2); // First is Default DRB

v_FirstDRB := v_DRBList[0];

v_SecondDRB := v_DRBList[1];

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_SecondDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_Configuration(v_PduSessionId, Present, Present, {1,2})), //Configure SDAP in normal mode for the moment; TTCN CR 190946

f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_FirstDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_Configuration(v_PduSessionId, Present, Present, {5,6}))}; //Configure SDAP in normal mode for the moment; TTCN CR 190946

v_DRB_ToAddModList := {cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, false, {1,2}), // @sic R5s191119 sic@

v_SecondDRB,

cs_38508_PDCP_Config(ms300)),

cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, true, {5,6}), // @sic R5s191119 sic@

v_FirstDRB,

cs_38508_PDCP_Config(ms300))};

v_QoS_Rules := cs_QoS_Rules ({f_BuildQoSRuleRemoteAccess("1"), f_BuildQoSRuleRemoteAccess("4"), f_BuildQoSRuleRemoteAccess("5"), f_BuildQoSRuleRemoteAccess("6")}); // @sic R5-215714 sic@

v_QoS_Rules.listofQoSRules[0].dqrBit := '0'B;

v_QoSFlowDescr := cs_QoSFlowDescr ({cs_QoS_Flow1, cs_QoS_Flow2, cs_QoS_Flow3, cs_QoS_Flow4});

// Start IP handling in order for the TTCN to handle IPv6 address allocation,

// then stop it before the test body, so that TTCN can route the IP data

v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo_Type);

v_QosFlowId := cs_IP_QosFlowId(oct2int(v_GSM_MobilityInfo_Type.SessionId), 1); //IP handling will go through QFI 2, because this QFI is associated with the default QoS rule @sic R5s200564, R5s201505 sic@

f_IP_Handling_StartPDN(IP, v_PDN_Index, cs_DrbInfo_NR(nr_Cell1, v_QosFlowId));

f_MultipleQoSPreamble_Part2 (nr_Cell1,

v_GSM_MobilityInfo_Type,

v_FirstDRB,

v_SecondDRB,

v_DRB_ToAddModList,

v_SS_Drb_ConfigList,

v_QoS_Rules,

v_QoSFlowDescr,

v_PDN_Index); //@sic R5s201361 sic@

f_NR_AS_CipheringAlgorithm_Set(nea0);

// Stop IP handling before reconfiguring SDAP to transparent mode

f_IP_Handling_StopPDN(IP);

// Reconfigure SDAP to transparent mode when moving to connected state

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_SecondDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_ConfigTransparentMode(Present)), //Configure SDAP in transparent mode; TTCN CR 190946

f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_FirstDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_ConfigTransparentMode(Present))}; //Configure SDAP in transparent mode; TTCN CR 190946

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeB_ON,

-,

'01'O, //IP PDU delay  = 1 second

-,-,

v_DRB_ToAddModList,

v_SS_Drb_ConfigList);

f_NR_TestBody_Set(true);

fl_TC_7_1_4_1_NR_TestBody(v_FirstDRB, v_SecondDRB, v_GSM_MobilityInfo_Type);

f_NR_TestBody_Set(false);

f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); // @sic R5s191119 sic@

//Postamble

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

f_NR_ReleaseAllCells();

}

### 2.3 Change 3

| Function name | fl_TC_7_1_4_1_NR_TestBody() |
| --- | --- |
| Reason for change | At steps 1 and 2 IP datagram should be created using filter#3 to use QFI 2 |
| Summary of change | Create datagram for QFI 2, for steps 1 and 2. |
| TTCN module | SDAP_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function fl_TC_7_1_4_1_NR_TestBody(integer p_FirstDRB,

integer p_SecondDRB,

GSM_MobilityInfo_Type p_GSM_MobilityInfo_Type) runs on NR_BASE_PTC

{

var template (present) SDAP_PDU_Type v_ExpectedPdu;

var octetstring v_IPDataQFI1 := f_SdapTests_CreateDatagram("2"); // Packet with remote address matching filter #2

var octetstring v_IPDataQFI5 := f_SdapTests_CreateDatagram("4"); // Packet with remote address matching filter #4

var octetstring v_IPDataQFI4 := f_SdapTests_CreateDatagram("3a"); // Packet with remote address matching filter #3a

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var template (value) DRB_ToAddModList v_DRB_ToAddModList;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var integer v_PduSessionId := oct2int(p_GSM_MobilityInfo_Type.SessionId);  // @sic R5s191119, R5s200564 sic@

var boolean v_PDU := false; // @sic R5s191119 sic@; boolean to indicate if the SDAP PDU has been received

var boolean v_EndMarker := false; // @sic R5s191119 sic@; boolean to indicate if the End-Marker has been received

var boolean v_RrcMsg := false; // @sic R5s191119 sic@; boolean to indicate if the RRC message has been received

var boolean v_NasMsg := false; // @sic R5s191119 sic@; boolean to indicate if the NAS message has been received

var NR_SRB_COMMON_IND v_ReceivedAsp; // @sic R5s191119 sic@

// @siclog "Step 1" siclog@

// The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=0. RQI=0, QFI=2.

f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI0, tsc_RQI0, tsc_QFI2, v_IPDataQFI1); //@sic R5-215714 sic@

// @siclog "Step 2" siclog@

// Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping flow with QFI=1?

v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI2),v_IPDataQFI1); //@sic R5-215714 sic@

DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu}));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2");

// @siclog "Step 3" siclog@

// The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=1, RQI=0, QFI=5.

f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI1, tsc_RQI0, tsc_QFI5, v_IPDataQFI5);

// EXCEPTION: In parallel to the event described in step 4 the events specified in Table 7.1.4.1.3.2-2 shall take place.

// @siclog "Table 7.1.4.1.3.2-2, Step 1" siclog@

// @siclog "Step 4" siclog@

v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI5),v_IPDataQFI5);

// @sic R5s191119 sic@

do {

alt {

[not v_PDU] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu}))

{ // Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping Flow with QFI=5?

v_PDU := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");

}

[not v_EndMarker] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_FirstDRB, -, {cr_SDAP_PDU_UL_EndMarker(tsc_QFI5)}))

{ // Check: Does the UE transmits End-Marker Control PDU on DRB1 for QFI=5?

v_EndMarker := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "End-Marker received");

}

}

}

while (not(v_PDU and v_EndMarker));

// @siclog "Step 5" siclog@

// The SS transmits an RRCReconfiguration message including a PDU SESSION MODIFICATION COMMAND

// Note: As SDAP is in transparent mode, there is no need to reconfigure the SS

v_DRB_ToAddModList := {cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, false, {4}), // @sic R5s191119 sic@

p_SecondDRB,

cs_38508_PDCP_Config(ms300))};

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit,

v_DRB_ToAddModList,

omit,

cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master));

v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand (p_GSM_MobilityInfo_Type.SessionId,

-,-,-,-,-,

cs_QoS_Rules ({f_BuildQoSRuleRemoteAccess("4a")}, '7A'O), // @sic R5s191119 sic@

cs_QoSFlowDescr ({cs_QoS_Flow2a}));

SRB.send(cas_NR_RrcNasPduList_REQ(nr_Cell1,

tsc_NR_RbId_SRB1,

cs_TimingInfo_Now,

cs_NR_RRC_MSG_RequestDCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig)),

{cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend)}));

// EXCEPTION: In parallel to the event described in step 6 the events specified in Tables 7.1.4.1.3.2-3 and 7.1.4.1.3.2-4 shall take place.

// @siclog "Table 7.1.4.1.3.3-3, Step 1" siclog@

// @siclog "Table 7.1.4.1.3.3-4, Step 1" siclog@

// @siclog "Step 6" siclog@

// @sic R5s191119 sic@

v_EndMarker := false; // Reset variable for reusing

do {

alt {

[not v_RrcMsg] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete))

{ // The UE transmits an RRCReconfigurationComplete message.

v_RrcMsg := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6");

}

[not v_EndMarker] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_FirstDRB, -, {cr_SDAP_PDU_UL_EndMarker(tsc_QFI4)}))

{ // Check: Does the UE transmit End-Marker Control PDU on DRB 1 for QFI=4?

v_EndMarker := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "End-Marker received");

}

[not v_NasMsg] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{ // The UE Transmits PDU SESSION MODIFICATION COMPLETE

if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg , cr_NG_PDU_SESSION_MODIFICATION_COMPLETE(?,?))) {

v_NasMsg := true;

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected NAS message received");

}

}

}

}

while (not(v_RrcMsg and v_EndMarker and v_NasMsg));

// @siclog "Step 7" siclog@

// The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=0. RQI=0, QFI=4.

f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI0, tsc_RQI0, tsc_QFI4, v_IPDataQFI4);

// @siclog "Step 8" siclog@

// Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping Flow with QFI=4?

v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI4),v_IPDataQFI4);

DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu}));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");

}

After Change:

function fl_TC_7_1_4_1_NR_TestBody(integer p_FirstDRB,

integer p_SecondDRB,

GSM_MobilityInfo_Type p_GSM_MobilityInfo_Type) runs on NR_BASE_PTC

{

var template (present) SDAP_PDU_Type v_ExpectedPdu;

var octetstring v_IPDataQFI1 := f_SdapTests_CreateDatagram("2"); // Packet with remote address matching filter #2

var octetstring v_IPDataQFI2 := f_SdapTests_CreateDatagram("3"); // Packet with remote address matching filter #3

var octetstring v_IPDataQFI5 := f_SdapTests_CreateDatagram("4"); // Packet with remote address matching filter #4

var octetstring v_IPDataQFI4 := f_SdapTests_CreateDatagram("3a"); // Packet with remote address matching filter #3a

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var template (value) DRB_ToAddModList v_DRB_ToAddModList;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var integer v_PduSessionId := oct2int(p_GSM_MobilityInfo_Type.SessionId);  // @sic R5s191119, R5s200564 sic@

var boolean v_PDU := false; // @sic R5s191119 sic@; boolean to indicate if the SDAP PDU has been received

var boolean v_EndMarker := false; // @sic R5s191119 sic@; boolean to indicate if the End-Marker has been received

var boolean v_RrcMsg := false; // @sic R5s191119 sic@; boolean to indicate if the RRC message has been received

var boolean v_NasMsg := false; // @sic R5s191119 sic@; boolean to indicate if the NAS message has been received

var NR_SRB_COMMON_IND v_ReceivedAsp; // @sic R5s191119 sic@

// @siclog "Step 1" siclog@

// The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=0. RQI=0, QFI=2.

f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI0, tsc_RQI0, tsc_QFI2, v_IPDataQFI2); //@sic R5-215714 sic@

// @siclog "Step 2" siclog@

// Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping flow with QFI=1?

v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI2),v_IPDataQFI2); //@sic R5-215714 sic@

DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu}));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2");

// @siclog "Step 3" siclog@

// The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=1, RQI=0, QFI=5.

f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI1, tsc_RQI0, tsc_QFI5, v_IPDataQFI5);

// EXCEPTION: In parallel to the event described in step 4 the events specified in Table 7.1.4.1.3.2-2 shall take place.

// @siclog "Table 7.1.4.1.3.2-2, Step 1" siclog@

// @siclog "Step 4" siclog@

v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI5),v_IPDataQFI5);

// @sic R5s191119 sic@

do {

alt {

[not v_PDU] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu}))

{ // Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping Flow with QFI=5?

v_PDU := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");

}

[not v_EndMarker] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_FirstDRB, -, {cr_SDAP_PDU_UL_EndMarker(tsc_QFI5)}))

{ // Check: Does the UE transmits End-Marker Control PDU on DRB1 for QFI=5?

v_EndMarker := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "End-Marker received");

}

}

}

while (not(v_PDU and v_EndMarker));

// @siclog "Step 5" siclog@

// The SS transmits an RRCReconfiguration message including a PDU SESSION MODIFICATION COMMAND

// Note: As SDAP is in transparent mode, there is no need to reconfigure the SS

v_DRB_ToAddModList := {cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, false, {4}), // @sic R5s191119 sic@

p_SecondDRB,

cs_38508_PDCP_Config(ms300))};

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit,

v_DRB_ToAddModList,

omit,

cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master));

v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand (p_GSM_MobilityInfo_Type.SessionId,

-,-,-,-,-,

cs_QoS_Rules ({f_BuildQoSRuleRemoteAccess("4a")}, '7A'O), // @sic R5s191119 sic@

cs_QoSFlowDescr ({cs_QoS_Flow2a}));

SRB.send(cas_NR_RrcNasPduList_REQ(nr_Cell1,

tsc_NR_RbId_SRB1,

cs_TimingInfo_Now,

cs_NR_RRC_MSG_RequestDCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig)),

{cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend)}));

// EXCEPTION: In parallel to the event described in step 6 the events specified in Tables 7.1.4.1.3.2-3 and 7.1.4.1.3.2-4 shall take place.

// @siclog "Table 7.1.4.1.3.3-3, Step 1" siclog@

// @siclog "Table 7.1.4.1.3.3-4, Step 1" siclog@

// @siclog "Step 6" siclog@

// @sic R5s191119 sic@

v_EndMarker := false; // Reset variable for reusing

do {

alt {

[not v_RrcMsg] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete))

{ // The UE transmits an RRCReconfigurationComplete message.

v_RrcMsg := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6");

}

[not v_EndMarker] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_FirstDRB, -, {cr_SDAP_PDU_UL_EndMarker(tsc_QFI4)}))

{ // Check: Does the UE transmit End-Marker Control PDU on DRB 1 for QFI=4?

v_EndMarker := true;

f_NR_PreliminaryPass(__FILE__, __LINE__, "End-Marker received");

}

[not v_NasMsg] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{ // The UE Transmits PDU SESSION MODIFICATION COMPLETE

if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg , cr_NG_PDU_SESSION_MODIFICATION_COMPLETE(?,?))) {

v_NasMsg := true;

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected NAS message received");

}

}

}

}

while (not(v_RrcMsg and v_EndMarker and v_NasMsg));

// @siclog "Step 7" siclog@

// The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=0. RQI=0, QFI=4.

f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI0, tsc_RQI0, tsc_QFI4, v_IPDataQFI4);

// @siclog "Step 8" siclog@

// Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping Flow with QFI=4?

v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI4),v_IPDataQFI4);

DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu}));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");

}