3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211579

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2170 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR2 : Addition of RRC test case 8.1.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add RRC test case 8.1.1.4.1 in FR2 to the IWD_21wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to RRC test case 8.1.1.4.1 required for agreement. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 R5-217275 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	7

4	Execution Log Files	7

4.1	HUAWEI Balong 5000	7

5	References	7

- Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.1.4.1 in FR2 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D21wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Shaun Harry

shaun.harry@keysight.com

  - Verification Test Summary

Test Case: 	8.1.1.4.1.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HUAWEI Balong 5000

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | fl_TC_8_1_1_4_1_TestBody() |
| --- | --- |
| Reason for change | For FR2, at step 22A, the PRACH slot defined by prach-ConfigurationIndex = 157 conflicts with the tdd-ul-dl-configuration According to 38.211 Table 6.3.3.2-4, PRACH slots for prach-ConfigurationIndex = 157 is (3, 5, 7) in 60kHz, translated to 120kHz is (7, 11, 15), But according to 38.508 Table 5.4.2.0-3: TDD-UL-DL-Config, in case of FR2, slot pattern (with periodicity 5) is DL DL DL X UL. This gives DL slots at positions 7, 11 and 15. Slot num; 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15. pattern; D; D; D; X; U; D; D; D; X; U; D; D; D; X; U; D. |
| Summary of change | Change prach-ConfigurationIndex to 156 for FR2 Note : An associated prose CR on 38.523-1 will be raised at RAN5#93. |
| TTCN module | RRC_Resume_NR5GC |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_8_1_1_4_1_TestBody() runs on NR5GC_PTC { //@sic R5-197053, R5-198912, R5s191051 sic@ var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfo; var integer v_DRBId; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var template (value) CellGroupConfig v_CellGroupConfig; var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2; // @sic R5-204436 sic@ var PDSCH_Config v_PDSCH_Config; // @sic R5-204436 sic@ var template (omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; // @sic R5-204436 sic@ var integer v_PrachConfigurationIndex; // @sic R5-213465 sic@ var NR_PhysicalParameters_Type v_PhysicalParameters; // @sic R5-213465 sic@ var template (omit) NR_SRB_COMMON_IND v_SrbIndByRef := omit; // not used (needed when other information of the RRC message needs to be access) // @sic R5-213465 sic@ var template (omit) NR_SYSTEM_IND v_SysIndByRef := omit; // to store SYSTEM_IND if it takes over the RRC message // @sic R5-213465 sic@ var template (present) TimingInfo_Type v_TimingInfo; // @sic R5-213465 sic@ timer t_WaitTime := 60.0; ……………….... //@siclog "Step 22A" siclog@ //@sic R5-213465, R5-214592 sic@ //The SS changes the prach-ConfigurationIndex in the system information. if (f_NR_CellInfo_GetIsFR1(nr_Cell2)){ v_PrachConfigurationIndex := 157; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //FR1 TDD - As per 38.211 Table 6.3.3.2-3 // nSFN mod 1 = 0=> any SFN Subframe 4 v_TimingInfo := cr_TimingInfo(?, (4)); } else { // FR2 v_PrachConfigurationIndex := 157; // slots (3,5,7) - As per 38.211 Table 6.3.3.2-4 v_TimingInfo := ( cr_TimingInfo(?, (3 / 8), -, cs_SlotTimingInfo_Numerology3(3 mod 8)), //Slot 3 cr_TimingInfo(?, (5 / 8), -, cs_SlotTimingInfo_Numerology3(5 mod 8)), //Slot 5 cr_TimingInfo(?, (7 / 8), -, cs_SlotTimingInfo_Numerology3(7 mod 8)) //Slot 7 ); } //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell2, v_PrachConfigurationIndex); //Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell2, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable)); //@siclog "Step 22B" siclog@ ……… }

After Change

function fl_TC_8_1_1_4_1_TestBody() runs on NR5GC_PTC { //@sic R5-197053, R5-198912, R5s191051 sic@ var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfo; var integer v_DRBId; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var template (value) CellGroupConfig v_CellGroupConfig; var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2; // @sic R5-204436 sic@ var PDSCH_Config v_PDSCH_Config; // @sic R5-204436 sic@ var template (omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; // @sic R5-204436 sic@ var integer v_PrachConfigurationIndex; // @sic R5-213465 sic@ var NR_PhysicalParameters_Type v_PhysicalParameters; // @sic R5-213465 sic@ var template (omit) NR_SRB_COMMON_IND v_SrbIndByRef := omit; // not used (needed when other information of the RRC message needs to be access) // @sic R5-213465 sic@ var template (omit) NR_SYSTEM_IND v_SysIndByRef := omit; // to store SYSTEM_IND if it takes over the RRC message // @sic R5-213465 sic@ var template (present) TimingInfo_Type v_TimingInfo; // @sic R5-213465 sic@ timer t_WaitTime := 60.0; ……………. //@siclog "Step 22A" siclog@ //@sic R5-213465, R5-214592 sic@ //The SS changes the prach-ConfigurationIndex in the system information. if (f_NR_CellInfo_GetIsFR1(nr_Cell2)){ v_PrachConfigurationIndex := 157; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //FR1 TDD - As per 38.211 Table 6.3.3.2-3 // nSFN mod 1 = 0=> any SFN Subframe 4 v_TimingInfo := cr_TimingInfo(?, (4)); } else { // FR2 v_PrachConfigurationIndex := 156; // slots (19,39)in 60kHz - As per 38.211 Table 6.3.3.2-4 v_TimingInfo := ( cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(7)), //Slot 39 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(7)) //Slot 79 ); } //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell2, v_PrachConfigurationIndex); //Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell2, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable)); //@siclog "Step 22B" siclog@ ………… }

- Branches executed

## This test case was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band n258 using the Ciphering Algorithm nea2 and Integrity Algorithm nia2.

- Execution Log Files

  - HUAWEI Balong 5000

The HUAWEI Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset platform. The documentation below is enclosed as evidence of the successful test case run [1]:

•	Test case execution log file:

TC_8_1_1_4_1_LOG.html

•	PICS/PIXIT parameter file:

TC_8_1_1_4_1_PIXIT.html

- References

| [1] | R5s211580: NR5GC FR2: Supporting information for addition of RRC test case 8.1.1.4.1. |
| --- | --- |