3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211586

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2173 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC Power Saving test case 8.1.5.10.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | ANRITSU LTD, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC Power Saving test case 8.1.5.10.1 UE Assistance Information/ Release Preference |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC Power Saving test case 8.1.5.10.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.10.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications 38.523-1 |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches Executed	5

4	Execution Log Files	5

5	References	5

- Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 8.1.5.10.1 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

Shaun Harry
	shaun.harry@keysight.com

  - Verification Test Summary

Test Case: 	8.1.5.10.1

ATS Version:	iWD-TTCN3-B2020-09_D21wk37

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR, Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SM 8450 + SDX65

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | f_TC_8_1_5_10_1_TestBody |
| --- | --- |
| Reason for change | - As per the prose, the Preferred RRC State is to be set as ‘inactive’ (=01) at step 7-8 - As per the prose, the Preferred RRC State is to be set as ‘connected’ (=10) at step 11-12 |
| Summary of change | - Changed the preferred RRC state to inactive at step 7-8 - Changed the preferred RRC state to connected at step 11-12 |
| TTCN module | RRC_Others_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_8_1_5_10_1_TestBody() runs on NR_BASE_PTC { //UE Assistance Information/ Release Preference … … … //@siclog "Steps 7-8" siclog@ //The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = 'inactive'. //The UE transmits an SET UAI RESPONSE message. f_NR5GC_SET_UAI(nr_Cell1, '10'B); //@siclog "Step 9" siclog@ //Check: Does the UE transmit an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = 'inactive' within the ReleasePreference-r16 IE?. alt { [] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_UEAssistanceInformation(crs_ReleasePreference_r16(inactive)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9"); } []t_Wait_T346f.timeout{}//SS waits for timer T346f expiry. } t_Wait_NoPreferredRRC_State_r16_connected.start(5.0); //@siclog "Steps 11-12" siclog@ //The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = connected. //The UE transmits an SET UAI RESPONSE message. f_NR5GC_SET_UAI(nr_Cell1, '01'B); … … … }

After Change

function f_TC_8_1_5_10_1_TestBody() runs on NR_BASE_PTC { //UE Assistance Information/ Release Preference … … … //@siclog "Steps 7-8" siclog@ //The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = 'inactive'. //The UE transmits an SET UAI RESPONSE message. f_NR5GC_SET_UAI(nr_Cell1, '01'B); //@siclog "Step 9" siclog@ //Check: Does the UE transmit an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = 'inactive' within the ReleasePreference-r16 IE?. alt { [] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_UEAssistanceInformation(crs_ReleasePreference_r16(inactive)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9"); } []t_Wait_T346f.timeout{}//SS waits for timer T346f expiry. } t_Wait_NoPreferredRRC_State_r16_connected.start(5.0); //@siclog "Steps 11-12" siclog@ //The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = connected. //The UE transmits an SET UAI RESPONSE message. f_NR5GC_SET_UAI(nr_Cell1, '10'B); … … … }

- Branches Executed

On Anritsu, the test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2.

On Keysight, the test case was executed on NR band n41 using NR ciphering nea1 and integrity algorithm nia1.

- Execution Log Files

The Qualcomm SM 8450 + SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR and Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_8_1_5_10_1_LOG.xml

Keysight\TC_8_1_5_10_1_LOG.html

- PICS/PIXIT parameter file:

Anritsu\ TC_8_1_5_10_1_PIXIT.txt

Keysight\ TC_8_1_5_10_1_PIXIT.xml

- References

| [1] | R5s211587 - Supporting information for addition of NR5GC Power Saving test case 8.1.5.10.1 in FR1 |
| --- | --- |