3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211609

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2187 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.1.5.1.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to Note 1: in 38.523-1 Table 9.1.5.1.8.3.2-1: Main behaviour, For Rel-16 or later Releases UE, the REGISTRATION REQUEST message shall be sent without integrity protection. For Rel-16 UE, the current implementation is sending the AUTHENTICATION REQUEST with integrity protection, because the registration type in use is not being passed through to the function f_NR5GC_RRC_Idle_Steps5_20() |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass the registration type in use into to the function f_NR5GC_RRC_Idle_Steps5_20() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A coformant Rel-16 UE can fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

  - Change 1

| Function name | fl_TC_9_1_5_1_8_TestBody() |
| --- | --- |
| Reason for change | According to Note 1: in 38.523-1 Table 9.1.5.1.8.3.2-1: Main behaviour, For Rel-16 or later Releases UE, the REGISTRATION REQUEST message shall be sent without integrity protection. For Rel-16 UE, the current implementation is sending the AUTHENTICATION REQUEST with integrity protection, because the registration type in use is not being passed through to the function f_NR5GC_RRC_Idle_Steps5_20() |
| Summary of change | Pass the registration type in use into to the function f_NR5GC_RRC_Idle_Steps5_20() Note : This change is based on IWD 21wk37 + TTCN CR R5s211379 |
| TTCN module | Initial_Registration_NR5GC |
| MCC160 Comment | Accepted. |

Before Change

function fl_TC_9_1_5_1_8_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellE := f_NR5GC_MapNASCell(ngc_CellE, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellI := f_NR5GC_MapNASCell(ngc_CellI, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellE); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var AccessStratumRelease v_AccessStratumRelease;//@sic R5s210768 sic@ var SecurityHeaderType v_SecurityHeader;//@sic R5s210768 sic@ var TypeOfRegistration_Type v_RegType;//@sic R5s210768 sic@ timer t_Wait := 30.0; …… //@siclog "Step 14" siclog@ //Check: Does the UE perform Registration procedure on NGC Cell A as specified in TS 38.508-1 [4] subclause 4.5.2? v_AccessStratumRelease:= f_NR_MobileInfo_NRCap_Get_AccessStratumRelease();//@sic R5s210768 sic@ if (v_AccessStratumRelease == rel15){//@sic R5s210768 sic@ //R5s211379 v_SecurityHeader:= tsc_SHT_IntegrityProtected;//@sic R5s210768 sic@ v_RegType:= Initial_Secure;//@sic R5s210768 sic@ } else { v_SecurityHeader:= tsc_SHT_NoSecurityProtection;//@sic R5s210768 sic@ v_RegType:= Initial_NoSecurity;//@sic R5s210768 sic@ } v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, v_SecurityHeader);//@sic R5 200154 sic@//@sic R5s210768 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, v_RegType)) {//@sic R5s210768 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 14"); } //Steps 5-20 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF); //@siclog "Step 14A" siclog@//@sic R5-203413, R5s201113 sic@ …………………. }

After Change

function fl_TC_9_1_5_1_8_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellE := f_NR5GC_MapNASCell(ngc_CellE, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellI := f_NR5GC_MapNASCell(ngc_CellI, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellE); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var AccessStratumRelease v_AccessStratumRelease;//@sic R5s210768 sic@ var SecurityHeaderType v_SecurityHeader;//@sic R5s210768 sic@ var TypeOfRegistration_Type v_RegType;//@sic R5s210768 sic@ timer t_Wait := 30.0; ……………………………. //@siclog "Step 14" siclog@ //Check: Does the UE perform Registration procedure on NGC Cell A as specified in TS 38.508-1 [4] subclause 4.5.2? v_AccessStratumRelease:= f_NR_MobileInfo_NRCap_Get_AccessStratumRelease();//@sic R5s210768 sic@ if (v_AccessStratumRelease == rel15){//@sic R5s210768 sic@ //KS R5s21xxx v_SecurityHeader:= tsc_SHT_IntegrityProtected;//@sic R5s210768 sic@ v_RegType:= Initial_Secure;//@sic R5s210768 sic@ } else { v_SecurityHeader:= tsc_SHT_NoSecurityProtection;//@sic R5s210768 sic@ v_RegType:= Initial_NoSecurity;//@sic R5s210768 sic@ } v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, v_SecurityHeader);//@sic R5 200154 sic@//@sic R5s210768 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, v_RegType)) {//@sic R5s210768 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 14"); } //Steps 5-20 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF, -, v_RegType); //@siclog "Step 14A" siclog@//@sic R5-203413, R5s201113 sic@ …………………… }