3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211700

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2220 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 11.3.5 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester Technology |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-17 NR5GC test case 11.3.5 to the NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 11.3.5 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

2.4	Change 4	11

2.5	Change 5	12

3	Branches executed	12

4	Execution Log Files	12

4.1	HiSilicon Balong 5000	12

5	References	13

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.3.5 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Pengchong Shen
	shenpengchong@datangmobile.cn

## Verification Test Summary

Test Case: 	11.3.5

ATS Version:	iwd-TTCN3-B2020-09_D21wk37

System Simulator used:	Datang LinkTester 5G Protocol Conformance Test System ECT9610

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | TC_11_3_5_NR5GC |
| --- | --- |
| Reason for change | - It missed IMS1for 11.3.5. - Guard timer is too short for selecte PLMN. |
| Summary of change | - Add IMS1 procedure. - Modify 300s to 600s. |
| TTCN module | NR5GC_Testsuite.ttcn |
| MCC160 Comment | - Accepted - Postoponed until RAN5 decision |

Before Change

testcase TC_11_3_5_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // UAC / Access Identity 1 / New cell not in the country of its HPLMN/EHPLMN 0% access probability/MPS indicator / HPLMN/0%/100% accessibility AC5/MMTEL-Video call var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_3_5_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change

testcase TC_11_3_5_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // UAC / Access Identity 1 / New cell not in the country of its HPLMN/EHPLMN 0% access probability/MPS indicator / HPLMN/0%/100% accessibility AC5/MMTEL-Video call var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(600); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_3_5_NR5GC()); v_IMS1.start(f_TC_11_3_5_NR5GC_IMS1()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

## Change 2

| Function name | f_TC_11_3_5_NR5GC |
| --- | --- |
| Reason for change | - In Table 11.3.5.3.3-2 for SIB1 of NR Cell 12，IE uac-BarringFactor for Access Category 3 is set to p95. The leftmost bit, bit 0 corresponding to Access Identity 1 is set to 1 in IE uac-BarringForAccessIdentity. According to TS 38.331 cl 5.3.14.5, assume the following two situations: At step 2, UE considers Access Identity 1 as valid in NR Cell 12 which is not in the country of its HPLMN or in an EHPLMN. When the random number ‘rand’ drawed by UE is lower than the value indicated by uac-BarringFactor (p95), UE considers the access attempt with Access Category 3 is allowed and continues with the Registration procedure on NR Cell 12. At step 2, UE does not consider Access Identity 1 as valid in NR Cell 12 which is not in the country of its HPLMN or in an EHPLMN. UE will use Access Identity 0. When the random number ‘rand’ drawed by UE is lower than the value indicated by uac-BarringFactor (p95), UE also considers the access attempt with Access Category 3 is allowed and continues with the Registration procedure on NR Cell 12. Thus, the case can not verify that UE does not consider Access Identity 1 as valid in NR Cell 12. Referring to test case 11.3.6, we will correct the description in TP1 and propose a reasonable test method. Note : it will have a prose CR to support. 2. It is wrong to deregistration on cell11. - The postamble should be STATE_CONNECTED_3A not STATE_CONNECTED_3A_T3540. |
| Summary of change | - Modify p95 to p00 for cell1, and modfiy p95 to p00 and modify '1111111'B to '0111111'B for cell12. - Modify nr_Cell11 to nr_Cell1. - Modify STATE_CONNECTED_3A_T3540 to STATE_CONNECTED_3A. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | - Postponed until RAN5 decision - Accepted - Accepted |

Before Change

function f_TC_11_3_5_NR5GC() runs on NR5GC_PTC { // UAC / Access Identity 1 / New cell not in the country of its HPLMN/EHPLMN 0% access probability/MPS indicator / HPLMN/0%/100% accessibility AC5/MMTEL-Video call // System information combination NR-2 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR Cells. f_NR5GC_Init(NR_1); //Set SIB1 for NR cell 1 according to Table 11.3.5.3.3-1 for Preamble f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(5,1), -, cs_UAC_BarringInfoSetList_1Entry(p95, s16, '1111111'B))); //Set SIB1 for NR cell 12 according to Table 11.3.5.3.3-3, this is used for Step 1. Configuring this at this point as the NR Cell 12 only gets active after preamble, thus no impact on behavior during preamble. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell12, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(3,1), -, cs_UAC_BarringInfoSetList_1Entry(p95, s16, '1111111'B))); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -80, -80); // Max Power level for FR2 is FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell12, -80, -80); // Max Power level for FR2 is FFS //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell12); //The UE is equipped with a USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-18 f_UT_USIM_Insert (UT, "USIM Configuration 18 specified in Table 6.4.1-18 TS 38.508-1"); // The UE is in state 1N-A on NR cell 1 according to TS 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); //Set SS/PBCH EPRE levels acc. to row "T0" f_NR_TestBody_Set(true); fl_TC_11_3_5_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell11, STATE_CONNECTED_3A_T3540); }

After Change

function f_TC_11_3_5_NR5GC() runs on NR5GC_PTC { // UAC / Access Identity 1 / New cell not in the country of its HPLMN/EHPLMN 0% access probability/MPS indicator / HPLMN/0%/100% accessibility AC5/MMTEL-Video call // System information combination NR-2 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR Cells. f_NR5GC_Init(NR_1); //Set SIB1 for NR cell 1 according to Table 11.3.5.3.3-1 for Preamble f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(5,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '1111111'B))); //Set SIB1 for NR cell 12 according to Table 11.3.5.3.3-3, this is used for Step 1. Configuring this at this point as the NR Cell 12 only gets active after preamble, thus no impact on behavior during preamble. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell12, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(3,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0111111'B))); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -80, -80); // Max Power level for FR2 is FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell12, -80, -80); // Max Power level for FR2 is FFS //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell12); //The UE is equipped with a USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-18 f_UT_USIM_Insert (UT, "USIM Configuration 18 specified in Table 6.4.1-18 TS 38.508-1"); // The UE is in state 1N-A on NR cell 1 according to TS 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); //Set SS/PBCH EPRE levels acc. to row "T0" f_NR_TestBody_Set(true); fl_TC_11_3_5_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

## Change 3

| Function name | fl_TC_11_3_5_TestBody |
| --- | --- |
| Reason for change | - Modify for change 2.1 ponit1. - At step 34, if UE starts T390=11.2s ('rand' is equal to 0). During the waiting time at step36, when receiving an indication from the lower layers that the barring is alleviated for the access category 5 after step 36, UE may initial MO vedio call procedure immediately again once T390 expires according to TS 24.501 cl 4.5.4.1. Then, step 36 will fail. Besides, after UE receives the AT command, UE will transmit RRCSetupRequest immediately which will stop the procude f_UT_RequestIMSVideoCall. Note : it will have a proses CR to support. - The trigger in function f_NR5GC_CallRelease is not needed. |
| Summary of change | - Add step 1A and 1B. - Add ignore procedure. And add branch for the UE send rrc setup request deose not depend on AT command. - Modify f_NR5GC_CallRelease to f_NR5GC_ModifyPDUSession_RemoveIMSVideo, and add wait trigger to conduct trigger from IMS1. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | Postponed until RAN5 decision |

Before Change

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC { var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell12); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_GutiParameters_Type v_GutiParams_NRCell1 := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI_NR_Cell1 := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams_NRCell1); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (nr_Cell12); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var TypeOfRegistration_Type v_TypeOfRegistration; var float v_TimerValue1 := 10.0; timer t_WaitTime; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, -80, -80) // Power level for FR2 is FFS }; v_CellPowerList_T2 := { cs_NR_CellPower (nr_Cell1, -80, -80), // Power level for FR2 is FFS cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //@siclog "Step 1" siclog@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.5.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message on NR cell 12 including the establishmentCause which is any value except mps-PriorityAccess. SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp; if(match(v_ReceivedAsp.Signalling.Rrc.Ccch.message_.c1.rrcSetupRequest.rrcSetupRequest.establishmentCause, mps_PriorityAccess)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } …… //@siclog "Step 33" siclog@ f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); //@siclog "Steps 34" siclog@ // Check: Does the UE transmit in the next 10 sec an RRCSetupRequest message? f_NR_CheckNoRRCSetupReq(nr_Cell1, 6.0); //@siclog "Step 35 siclog@ //SS changes SIB1 of NR cell 1 according to Table 11.3.5.3.3-1 and notifies the UE of change of System Information on NR Cell 1 by send Short Message on PDCCH using P-RNTI f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); //@siclog "Step 36 siclog@ //Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 37" siclog@ f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); //@siclog "Step 38" siclog@ t_WaitTime.start(v_TimerValue1); alt { //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 38"); } [] t_WaitTime.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 38"); } } //@siclog "Step 39 - 49b3 siclog@ //Steps 3-13b3 of generic test procedure for IMS MO Video call establishment in 5GC as specified in 38.508-1 [4] Table 4.9.xx24.2.2-1 are performed. f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_NR_Cell1, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); // Wait until need to add video QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_AddVideo(nr_Cell1, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_NR_PreliminaryPass(__FILE__, __LINE__, "Video Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Step 50" siclog@ //The generic test procedure for IMS MT call release in 5GC as specified in TS 38.508-1 [4] Table 4.9.18.2.2-1 takes place f_NR5GC_CallRelease(nr_Cell1, IPCAN_VideoCall); }

After Change

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC { var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell12); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_GutiParameters_Type v_GutiParams_NRCell1 := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI_NR_Cell1 := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams_NRCell1); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (nr_Cell12); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var TypeOfRegistration_Type v_TypeOfRegistration; var float v_TimerValue1 := 10.0; timer t_WaitTime; var float v_TimerValue2 := 26.0; var boolean v_step38 := false; var default v_IgnoreNRMsg3:= null; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, -80, -80) // Power level for FR2 is FFS }; v_CellPowerList_T2 := { cs_NR_CellPower (nr_Cell1, -80, -80), // Power level for FR2 is FFS cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //@siclog "Step 1" siclog@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.5.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 1A" siclog@ f_NR_CheckNoRRCSetupReq(nr_Cell12, 120.0); //@siclog "Step 1B" siclog@ f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell12, omit); f_NR_ModifySysinfo(nr_Cell12); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message on NR cell 12 including the establishmentCause which is any value except mps-PriorityAccess. SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp; if(match(v_ReceivedAsp.Signalling.Rrc.Ccch.message_.c1.rrcSetupRequest.rrcSetupRequest.establishmentCause, mps_PriorityAccess)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } …… //@siclog "Step 33" siclog@ f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); //@siclog "Steps 34" siclog@ // Check: Does the UE transmit in the next 6 sec an RRCSetupRequest message? f_NR_CheckNoRRCSetupReq(nr_Cell1, 6.0); //@siclog "Step 35 siclog@ //SS changes SIB1 of NR cell 1 according to Table 11.3.5.3.3-1 and notifies the UE of change of System Information on NR Cell 1 by send Short Message on PDCCH using P-RNTI f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); //@siclog "Step 36 siclog@ //Wait for 2.1* modification period to allow the new system information to take effect. t_WaitTime.start(v_TimerValue2); alt{ [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { t_WaitTime.stop; v_step38 := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 38"); } [] t_WaitTime.timeout { v_IgnoreNRMsg3 := activate (a_NR5GC_Msg3_Handler (nr_Cell1)); f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); deactivate (v_IgnoreNRMsg3); } } //REMOVE f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 37" siclog@ //MOVE f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); //@siclog "Step 38" siclog@ if(not v_step38){ t_WaitTime.start(v_TimerValue1); alt { //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 38"); } [] t_WaitTime.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 38"); } } } //@siclog "Step 39 - 49b3 siclog@ //Steps 3-13b3 of generic test procedure for IMS MO Video call establishment in 5GC as specified in 38.508-1 [4] Table 4.9.xx24.2.2-1 are performed. f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_NR_Cell1, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); // Wait until need to add video QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_AddVideo(nr_Cell1, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_NR_PreliminaryPass(__FILE__, __LINE__, "Video Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Step 50" siclog@ //The generic test procedure for IMS MT call release in 5GC as specified in TS 38.508-1 [4] Table 4.9.18.2.2-1 takes place //wait IMS that call is released f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_RemoveIMSVideo(nr_Cell1); }

## Change 4

| Function name | f_TC_11_3_5_NR5GC_IMS1 |
| --- | --- |
| Reason for change | - It is not need for the trigger has been included in fuction f_IMS_NR5GC_MOVideoCall_Step2ToEnd. - Step 50 should be IMS MT call release, but the current TTCN is MO call release. |
| Summary of change | - Delete the trigger. - Modify f_IMS_CallReleaseMO to f_IMS_CallReleaseMT. |
| TTCN module | UAC_NR5GC_IMS.ttcn |
| MCC160 Comment | Accepted but different implementation |

Before Change

function f_TC_11_3_5_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //Wait for the INVITE v_InviteRequestWithSdp := f_IMS_NR5GC_MOVideoCall_Step1(); //Tell NR to add video f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //Wait for NR to tell that Video is added f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Steps 39 - 49b3" siclog@ // Step3 to step13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.24.2.2-1 take place, f_IMS_NR5GC_MOVideoCall_Step2ToEnd(v_InviteRequestWithSdp); //Tell NR that call is setup f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //Wait for NR to tell that call is released f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Steps 29-30" siclog@ //Steps 1-2 from test procedure for test procedure for IMS MO speech call release as described in TS 38.508-1 [4] Table 4.9.18.2.2-1 take place, f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); //Tell NR that call is released f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change

function f_TC_11_3_5_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //Wait for the INVITE v_InviteRequestWithSdp := f_IMS_NR5GC_MOVideoCall_Step1(); //Tell NR to add video //REMOVE f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //Wait for NR to tell that Video is added // REMOVE f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Steps 39 - 49b3" siclog@ // Step3 to step13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.24.2.2-1 take place, f_IMS_NR5GC_MOVideoCall_Step2ToEnd(v_InviteRequestWithSdp); //Tell NR that call is setup //REMOVE f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //Wait for NR to tell that call is released //REMOVE f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Steps 29-30" siclog@ //Steps 1-2 from test procedure for test procedure for IMS MO speech call release as described in TS 38.508-1 [4] Table 4.9.18.2.2-1 take place, f_IMS_CallReleaseMT(f_SIP_BuildSipUri_TX(px_IMS_CalleeContactUri)); //Tell NR that call is released f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

MCC160 Implementation function f_TC_11_3_5_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // MO Video Call f_IMS_NR5GC_MOVideoCall(); // Call Release MT f_IMS_CallReleaseMT(f_SIP_BuildSipUri_TX(px_IMS_CalleeContactUri)); //Tell NR that call is released f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

## Change 5

| Function name | a_NR5GC_Msg3_Handler |
| --- | --- |
| Reason for change | - Add for change3 point2. |
| Summary of change | - Add for change3 point2. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | Postponed until RAN5 decision |

New alstep

altstep a_NR5GC_Msg3_Handler(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))){repeat;}; }

# Branches executed

NR Band 41, nea2, Nia2.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on Datang LinkTester 5G Protocol Conformance Test System ECT9610. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:
\ TC_11_3_5_NR5GC_N41.log

- PIXCIT settings used:
\ Pics(Hisilicon-IRAT).xml

\ Pixit(TC_11_3_5_NR5GC).xml

# References

| [1] | R5s211701: Supporting information for agreement of NR5GC test case 11.3.5 in FR1 |
| --- | --- |