3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220016

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2250 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC MDT template cr_NR_RLF_Report_r16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The template cr_NR_MeasResult2NR_r16 have defined like first parameter ARFCN_ValueNR and second PhysCellId. These 2 parameters are swapped in definition of template cr_NR_RLF_Report_r16 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Swap ARFCN_ValueNR and second PhysCellId inside cr_NR_MeasResult2NR_r16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.1.NR5GC, 8.1.6.1.3.2.NR5GC, 8.1.6.1.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

| Function name | cr_NR_RLF_Report_r16 |
| --- | --- |
| Reason for change | The template cr_NR_MeasResult2NR_r16 have defined like first parameter ARFCN_ValueNR and second PhysCellId. These 2 parameters are swapped in definition of template cr_NR_RLF_Report_r16 |
| Summary of change | Swap ARFCN_ValueNR and second PhysCellId inside cr_NR_MeasResult2NR_r16 |
| TTCN module | NR5GC_MDT_Specific_Templates.ttcn |
| MCC160 Comment | Accepted |

Before change:

template RLF_Report_r16 cr_NR_RLF_Report_r16 (PhysCellId p_PhysCellId_New,

ARFCN_ValueNR p_ARFCN_ValueNR_New,

CellIdentity p_CellIdentity,

RNTI_Value p_RNTI_Val_OldServCell,

template (present) SSB_Index p_SSB_Index := 1

) :=

{

nr_RLF_Report_r16 :=

{

measResultLastServCell_r16 := cr_MeasResultRLFNR_r16_1SSB(p_SSB_Index),

measResultNeighCells_r16 := cr_NR_MeasResultNeighCells_r16_NRList ({cr_NR_MeasResult2NR_r16 (p_PhysCellId_New, p_ARFCN_ValueNR_New)}),

c_RNTI_r16 := p_RNTI_Val_OldServCell,

previousPCellId_r16 := omit,

failedPCellId_r16 := {

nrFailedPCellId_r16 := ?  //@sic R5s211638 sic@

},

reconnectCellId_r16 := omit,

timeUntilReconnection_r16 := omit,      /* @sic R5s211526 BASELINE MOVING 2021 sic@ */

reestablishmentCellId_r16 := cr_NR_CGI_Info_Logging_r16_Def(p_CellIdentity),

timeConnFailure_r16 := omit,

timeSinceFailure_r16 := ?,

connectionFailureType_r16 := rlf,

rlf_Cause_r16 := t310_Expiry,

locationInfo_r16 := *,

noSuitableCellFound_r16 := omit,

ra_InformationCommon_r16 := omit,

csi_rsRLMConfigBitmap_v1650 := omit /* @sic R5s211526 BASELINE MOVING 2021 FFS prose CR sic@ */

}

};

After change:

template RLF_Report_r16 cr_NR_RLF_Report_r16 (PhysCellId p_PhysCellId_New,

ARFCN_ValueNR p_ARFCN_ValueNR_New,

CellIdentity p_CellIdentity,

RNTI_Value p_RNTI_Val_OldServCell,

template (present) SSB_Index p_SSB_Index := 1

) :=

{

nr_RLF_Report_r16 :=

{

measResultLastServCell_r16 := cr_MeasResultRLFNR_r16_1SSB(p_SSB_Index),

measResultNeighCells_r16 := cr_NR_MeasResultNeighCells_r16_NRList ({cr_NR_MeasResult2NR_r16 (p_ARFCN_ValueNR_New, p_PhysCellId_New)}),

c_RNTI_r16 := p_RNTI_Val_OldServCell,

previousPCellId_r16 := omit,

failedPCellId_r16 := {

nrFailedPCellId_r16 := ?  //@sic R5s211638 sic@

},

reconnectCellId_r16 := omit,

timeUntilReconnection_r16 := omit,      /* @sic R5s211526 BASELINE MOVING 2021 sic@ */

reestablishmentCellId_r16 := cr_NR_CGI_Info_Logging_r16_Def(p_CellIdentity),

timeConnFailure_r16 := omit,

timeSinceFailure_r16 := ?,

connectionFailureType_r16 := rlf,

rlf_Cause_r16 := t310_Expiry,

locationInfo_r16 := *,

noSuitableCellFound_r16 := omit,

ra_InformationCommon_r16 := omit,

csi_rsRLMConfigBitmap_v1650 := omit /* @sic R5s211526 BASELINE MOVING 2021 FFS prose CR sic@ */

}

};